| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C172 WG11 Number: m18201 |
| --- | --- |

| Title: |  |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ali Tabatabai | Tel: +1-408-352-4715 Email:ali.tabatabai@am.sony.com |  |
| Source: | TE06 Coordinator |  |  |

_____________________________

# Abstract

This contribution is a summary of Tool Experiment 6, Intra Prediction Improvements.  Nine Organizations participated in cross checking of 5 out of 6 toolsets. For cross checking we used the recommended test conditions of intra-only prediction mode for both high compression efficiency and low complexity as defined in the document B306_r3 [1].

Introduction

T06 is organized in 3 tool subsets:

## TE06.a: Bidirectional Intra Prediction

Proposed tools in this tool subset are:

- JCTVC-B042 (Toshiba):  Weighted Bidirectional Prediction (WBP) and Adaptive Sub-block Coding Order (ASCO)

- JCTVC-B040 (Huawei and Hisilicon):  Short Distance Intra Prediction Using Correlation between Lines & Pixels

- JCTVC-A109 (MediaTek):  Overlapped Block Intra Prediction (OBIP), Multiple Model Karhunen Loeve Transform (MMKLT), Context Dependent Mode Representation (CDMR)

- JCTVC-A118 (Microsoft):  Line/Pixel Based Prediction Modes

## TE06.b: Edge Based Intra Prediction

- JCTVC-B109 (Sony):  Differential Coding of Intra Prediction Mode

## TE06.c: Parallel Intra Coding

- JCTVC-B112 (Sharp Labs. of America)

# Cross Verification

A summary Table of proponents, cross-checking organizations and related documents is here:

| Tool | Proponent | Cross-checker(s) |
| --- | --- | --- |
| TE6.a.1 | Toshiba JCTVC-C07, m18102 | Renesas (JCTVC-C187) Sharp (JCTVC-C179) |
| TE6.a.2 | Huawei, Hisilicon JCTVC-C101, m18124 | DOCOMO (JCTVC-C045, m18066) LGE (JCTVC-C028, m18039) Microsoft (JCTVC-C27, m18314) Ericsson (JCTVC-C157, m18183) |
| TE6.a.3 | MediaTek (OBIP) JCTVC-C193 | Microsoft (JCTVC-C272, m18315) |
| TE6.a.4 | Microsoft | Huawei (JCTVC-C139, m18163) DOCOMO (JCTVC-C045, m18066) |
| TE6.b.1 | Sony JCTVC-C169, m18198 | Sharp (JCTVC-C175, m18204) Panasonic (JCTVC-C217, m18257) |

# Experimental Results

Table 1: TE6.a.1 BD Rate Summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.7 | -2.1 | -2.1 | -2.5 | -1.8 | -1.6 |
| Class B | -2.1 | -1.4 | -1.4 | -2.1 | -1.3 | -1.2 |
| Class C | -1.9 | -1.5 | -1.4 | -2.4 | -1.5 | -1.4 |
| Class D | -2.0 | -1.4 | -1.4 | -2.4 | -1.4 | -1.4 |
| Class E | -2.2 | -2.0 | -1.9 | -2.5 | -2.2 | -2.2 |
| All | -2.1 | -1.6 | -1.6 | -2.3 | -1.6 | -1.5 |
| Enc Time[%] | 94% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |

Table 2: TE6.a.2  BD Rate Summary

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.1 | -0.5 |
| Class B | -0.6 | 0.2 | -0.2 |
| Class C | -1.3 | 0.1 | -0.3 |
| Class D | -1.3 | -0.3 | -0.5 |
| Class E | -0.9 | -1.0 | -0.9 |
| All | -0.9 | -0.2 | -0.4 |
| Enc Time[%] |  |  |  |
| Dec Time[%] | 103% |  |  |

Table 3: TE6.a.3 BD Rate Summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.2 | -1.6 | -1.8 | -1.2 | -1.1 |
| Class B | -0.8 | -0.6 | -0.7 | -1.4 | -0.8 | -0.8 |
| Class C | -1.1 | -0.7 | -0.7 | -1.8 | -1.0 | -0.8 |
| Class D | -1.2 | -0.5 | -0.6 | -1.7 | -0.9 | -0.8 |
| Class E | -0.7 | -1.2 | -0.6 | -1.7 | -0.3 | -0.8 |
| All | -1.0 | -0.8 | -0.7 | -1.7 | -0.8 | -0.8 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table 4: TE06.a.4 BD Rate Summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -0.6 | -0.7 | -1.2 | -0.4 | -0.6 |
| Class B | -2.3 | -1.0 | -0.9 | -2.4 | -0.6 | -0.8 |
| Class C | -2.7 | -1.1 | -1.3 | -3.3 | -1.1 | -1.2 |
| Class D | -2.1 | -0.9 | -0.9 | -2.7 | -0.8 | -0.8 |
| Class E | -1.9 | -0.7 | -0.6 | -1.9 | -1.2 | -1.1 |
| All | -2.1 | -0.9 | -0.9 | -2.4 | -0.8 | -0.9 |
| Enc Time[%] | 172% |  |  | 229% |  |  |
| Dec Time[%] | 208% |  |  | 301% |  |  |

Table 5:  TE6.b.1 BD Rate Summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | 1.2 | 1.7 | 2.3 | 0.9 | 2.8 |
| Class B | -1.9 | 0.2 | 0.5 | 3.9 | 2.9 | 3.9 |
| Class C | -2.5 | -0.5 | -0.6 | -0.1 | -2.5 | -2.2 |
| Class D | -1.4 | 0.4 | 0.0 | 0.2 | -2.4 | -2.7 |
| Class E | -2.3 | 1.9 | 1.2 | 1.8 | 8.6 | 6.2 |
| All | -1.9 | 0.5 | 0.4 | 1.7 | 1.2 | 1.4 |
| Enc Time[%] | 214% |  |  | 191% |  |  |
| Dec Time[%] | 62% |  |  | 48% |  |  |

Note that proponent of TE6.b.1 have informed that the results on intra low complexity configuration is affected by a VLC bug when MDDT is turned off (Ticket #79).  The bug fix has recently been incorporated into the implementation and tests are being redone. The results for intra low complexity configuration will thus be updated once the test results become available.

# Visual Evaluation

No independent visual information was reported

# Conclusion

Performance of 5 toolsets have been independently cross verified. These new intra prediction tools are showing  potential for further investigation and incorporation into Test Model.

# References

- Ali Tabatabai, “Tool Experiment 6: Intra Prediction Improvement”, Doc. JCTVC-B306_r3, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.