| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C174 WG11 Number: m18203 |
| --- | --- |

| Title: | TE12.3: Report on combined intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto Andrew Segall 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides the cross-check result of Combined Intra Prediction (CIP) in the context of TE12. The experimental results show CIP introduces average gain (Y BD-rate for each condition) ranging from 0.0% to 0.3%. CIP introduces about 20% encoding-time increase (Intra Only case), but it does not affect decoding time. It is confirmed that the results provided by NEC match the results provided in this contribution.

# Experimental condition

The evaluation of CIP defined in TE12 was carried out. (Description of CIP can be found in TMuC [1].) We use TMuC 0.7 software, and apply configuration described in JCTVC-C312 [2] and JCTVC-C300 [3]. The computing platform is summarized in Table 1. In the experiments, encoding was done using the grid system composed of variety of PCs, but it was configured so that the same-performance PC is used to encode both anchor and the proposed method for each test point. Decoding was done using the same machine for both anchor and the proposed method.

Table 1: Computing platforms

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Windows XP (32bit) | Linux (64bit) |
| CPU | Intel Core 2 Duo E6750 2.66GHz | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz Intel Core2 Quad 2.66GHz |
| Compiler | Visual Studio 2005 | gcc 4.3.3 |

# Performance experiments

The experimental results are summarized in Table 2. Detailed results are included in the attachment (JCTVC-C174.xls).

Table 2: Experimental results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 1.6 | 1.6 | 0.5 | 1.9 | 1.8 |
| Class B | 0.2 | 0.9 | 0.9 | 0.2 | 1.2 | 1.2 |
| Class C | 0.3 | 0.7 | 0.7 | 0.2 | 0.6 | 0.6 |
| Class D | 0.3 | 0.6 | 0.6 | 0.3 | 0.7 | 0.7 |
| Class E | 0.3 | 1.2 | 1.3 | 0.2 | 1.4 | 1.3 |
| All | 0.3 | 0.9 | 0.9 | 0.2 | 1.1 | 1.1 |
| Enc Time[%] | 81% |  |  | 80% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.7 | 0.6 | 0.1 | 0.6 | 0.6 |
| Class B | 0.2 | 0.2 | 0.2 | 0.0 | 0.4 | 0.4 |
| Class C | 0.3 | 0.2 | 0.5 | 0.0 | 0.3 | 0.3 |
| Class D | 0.2 | 0.5 | 0.4 | 0.1 | 0.2 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.4 | 0.4 | 0.1 | 0.4 | 0.4 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.3 | 0.3 | 0.0 | 0.1 | 0.1 |
| Class C | 0.2 | 0.2 | 0.4 | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | 0.2 | 0.1 | 0.0 | -0.1 |
| Class E | -0.2 | 0.0 | 0.0 | 0.0 | 0.3 | 0.4 |
| All | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Cross-check

The experimental results are compared with the results provided by NEC [4]. Although there were small differences exist that is assumed to be come from the platform difference, BD-rate and BD-snr result from both parties practically match. Also, encoding and decoding time ratio are consistent.

# Conclusion

The cross-check result of Combined Intra Prediction (CIP) in the context of TE12 was reported. The experimental results show that CIP introduces average gain (Y BD-rate for each condition) ranging from 0.0% to 0.3%. With the current implementation, CIP introduces about 20% encoding-time increase (Intra Only case), but it does not affect decoding time. The results were confirmed to be consistent with the results provided by the partner. We encourage further study of CIP but do not recommend inclusion of the tool in any TM or TMuC default configuration at this time due to the reported performance.

# Reference

- JCT-VC “Draft Test Model under Consideration”, JCTVC-B205, 2nd JCT-VC Meeting, Geneva, July 2010.

- JCTVC “Common test conditions and software reference configurations”, JTCVC-B300, 2nd JCT-VC Meeting, Geneva, July 2010.

- JCTVC “Tool Experiments 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd JCT-VC Meeting, Geneva, July 2010.

- Keiichi Chono, Kenta Senzaki, Hirofumi Aoki, Junji Tajima, and Yuzo Senda “TE12: Report on Combined Intra Prediction evaluation”, JCTVC-C092, 3rd JCT-VC Meeting, Guangzhou, October 2010.