| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C175 WG11 Number: m18204 |
| --- | --- |

| Title: | TE6.b: Cross verification of differential coding of intra modes (DCIM) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto Yukinobu Yasugi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides the cross-check result of the method called Differential Coding of Intra Modes (DCIM) by Sony in the context of TE6 (TE6.b: Edge based intra prediction). The experiments defined in TE6 were carried out using the software provided by Sony. The experimental results show the proposed method provides 1.9% gain in High Coding Efficiency / Intra Only condition and 1.7% loss in Low Delay / Intra Only condition with the BD-rate criteria. It is confirmed that these results match the results provided by the proponent.

# Performance experiments

The coding efficiency was evaluated in IntraOnly High Coding Efficiency and Low Complexity conditions defined in TE6 [1]. The experimental results are summarized in Table 1 and Table 2. Detailed results are included in the attachment (JCTVC-C175.xls).

Compared to the anchor, the proposed technique (DCIM) provides 1.9 % gain (IntraOnly, High Coding Efficiency) and 1.7 % loss (IntraOnly, Low Complexity) measured using BD-rate criteria. We confirm that our results match those provided by the proponent [2].

Table 1: Experimental results in IntraOnly, High Coding Efficiency

|  | BD-rate Y | BD-rate U | BD-rate V |
| --- | --- | --- | --- |
| Class A | -0.8 | 1.2 | 1.7 |
| Class B | -1.9 | 0.2 | 0.5 |
| Class C | -2.5 | -0.5 | -0.6 |
| Class D | -1.4 | 0.3 | 0.0 |
| Class E | -2.3 | 1.9 | 1.3 |
| Average | -1.9 | 0.5 | 0.4 |

Table 2: Experimental results in IntraOnly, Low Complexity

|  | BD-rate Y | BD-rate U | BD-rate V |
| --- | --- | --- | --- |
| Class A | 2.3 | 0.9 | 2.8 |
| Class B | 3.9 | 2.9 | 3.9 |
| Class C | -0.1 | -2.5 | -2.2 |
| Class D | 0.2 | -2.4 | -2.7 |
| Class E | 1.8 | 8.6 | 6.2 |
| Average | 1.7 | 1.2 | 1.4 |

# Complexity

Complexity was measured by decoding time and encoding time. The environment used for the evaluation is shown in Table 3 and the results are shown in Table 4. Encoding was done using the grid system composed of variety of PCs, but it was configured so that the same-performance PC is used to encode both anchor and the proposed method for each test point. Decoding was done using the same machine for both anchor and the proposed method.

Note that MDDT and Edge-based intra prediction are turned on in the anchor, while they are not used in the DCIM software.

Table 3: Experimental conditions

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Windows Vista (32bit) | Linux (64bit) |
| CPU | Intel Core 2 Duo Q9650 3.00GHz | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz Intel Core2 Quad 2.66GHz |
| Compiler | Visual Studio 2005 | gcc 4.3.3 |

Table 4: Experimental results of time ratio

|  | Encoding Time ratio | Decoding Time ratio |
| --- | --- | --- |
| IntraOnly, High Coding Efficiency | 207% | 54% |
| IntraOnly, Low Complexity | 190% | 42% |

# Additional Experiment

Additional experiment was done by disabling AIS, CIP, Edge-based intra prediction, and MDDT in both reference and target (DCIM).  Note that DEFAULT_AIS has not been changed in the experiment.  The result is shown in Table 5.  Compared to the result in section 1, gain for High Coding Efficiency condition decreases, while loss for Low Complexity condition turns to be gain.

Table 5: Result of additional experiment

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -0.8 | -0.8 | -1.1 | -0.6 | -0.7 |
| Class B | -1.0 | -0.8 | -0.8 | -1.0 | -0.6 | -0.6 |
| Class C | -1.2 | -0.7 | -0.7 | -1.0 | -0.6 | -0.6 |
| Class D | -0.5 | -0.3 | -0.3 | -0.4 | -0.2 | -0.2 |
| Class E | -1.9 | -1.4 | -1.3 | -2.0 | -1.4 | -1.4 |
| All | -1.1 | -0.8 | -0.7 | -1.0 | -0.6 | -0.7 |
| Enc Time[%] | 161% |  |  | 220% |  |  |
| Dec Time[%] | 61% |  |  | 49% |  |  |

- Reference

- Ali Tabatabai, Teruhiko Suzuki, Akiyuki Tanizawa, Andrew Segall, Haoping Yu, Shawmin Lei, Changcai Lai “Tool Experiment 6: Intra Prediction Improvement”, JCTVC-B306, 2nd JCT-VC Meeting, Geneva, July 2010.

- E. Maani, L. Dong, W. Liu, “	TE6.b: Experiment results of DCIM”, JCTVC-C169, 3rd JCT-VC Meeting, Guangzhou, October 2010.