| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C179 WG11 Number: m18208 |
| --- | --- |

| Title: | TE6: Cross-check of bi-directional intra-prediction results from Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution cross-checks the bi-directional intra-prediction results provided by Toshiba [3]. This cross-check was performed within the context of TE6 for “Intra high-performance” and “Intra low-complexity” configuration points. The experimental results closely match the rate-distortion results provided by Toshiba. The maximum Y BD Rate differs from the provided results by at most 0.1%.

# Experimental condition

The evaluation of bi-directional intra-prediction defined in TE6 was carried out. We used the TMuC 0.7 software in experiments and applied the configuration described in JCTVC-C312 [1] and JCTVC-C300 [2]. The computing platform is summarized in Table 1. In the experiments, coding was done using a cloud system. The cloud system was used to encode the proposed method for each test point. Decoding was done using the same machine for the proposed method. The usage of a shared resource such as cloud in encoding and decoding make the timing information highly variable and unreliable for making complexity predictions.

Table 1: Computing platforms

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| CPU | Quad-Core AMD Opteron™ Processor 2374 HE | Quad-Core AMD Opteron™ Processor 2374 HE |
| Compiler | gcc 4.3.2 | gcc 4.3.2 |

# Performance experiments

The experimental results of the comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-C179.xls).

Table 2: Experimental results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# Cross-check

The experimental results listed in table 2 show the difference with respect to the results provided by Toshiba [3]. Although there were small differences it can be attributed to platform differences. BD-Rate and BD-SNR result from both parties practically match.

# Conclusion

The cross-check of bi-directional intra-prediction results provided by Toshiba [3] was reported. The experimental results show maximum difference of 0.1% (Y BD-rate for each condition). The results were confirmed to be consistent with the results provided by the partner.

# Reference

- JCTVC “Common test conditions and software reference configurations”, JTCVC-B300, 2nd JCT-VC Meeting, Geneva, July 2010.

- JCTVC “Tool Experiments 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd JCT-VC Meeting, Geneva, July 2010.

- Taichiro Shiodera, Akiyuki Tanizawa, Takeshi Chujoh, Tomoo Yamakage, “TE6 subset a: Bidirectional intra prediction”, JCTVC-C079, 3rd JCT-VC Meeting, Guangzhou, October 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.