| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C181 WG11 Number: m18210 |
| --- | --- |

| Title: | TE6: Cross-check of rotational transform (ROT) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution cross-checks the performance of rotational transform in the context of TE12. The experimental results shows ROT introduces an average loss (Y BD-rate for each condition) ranging from 0.0% to 0.6%.  This closely matches the rate-distortion results provided by Qualcomm [4].

# Experimental condition

The evaluation of rotational transform (ROT) defined in TE12 was carried out. Description of ROT can be found in TMuC [3]. We used the in test model under consideration TMuC 0.7 software in experiments and applied the six configurations described in JCTVC-C312 [1] and JCTVC-C300 [2]. The inverse rotational transform implementation in TMuC 0.7 was verified to be identical to that specified in [3]. The computing platform is summarized in Table 1. In the experiments, coding was done using a cloud system. The cloud system was used to encode the proposed method for each test point. Decoding was done using the same machine for the proposed method. The usage of a shared resource such as cloud in encoding and decoding make the timing information highly variable and unreliable for making complexity predictions.

Table 1: Computing platforms

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| CPU | Quad-Core AMD Opteron™ Processor 2374 HE | Quad-Core AMD Opteron™ Processor 2374 HE |
| Compiler | gcc 4.3.2 | gcc 4.3.2 |

# Performance experiments

Testing was carried out by setting the encoder configuration parameter as follows:

- With Rotational Transform : “ROT : 1”

- Without Rotational Transform : “ROT: 0”

The experimental results of the comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-C181.xls). A comparison of the summary result Table 2 with that provided by Qualcomm[4] is included as an attachment (JCTVC-C181-Summary-Comparison.xls). The comparison shows close agreement between the results of both parties.

Table 2: Performance of ROT against reference TMuC 0.7 anchor configuration

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | -0.4 | -0.8 | 0.8 | 0.5 | 0.6 |
| Class B | 0.6 | 0.1 | -0.2 | 0.9 | 1.0 | 1.0 |
| Class C | 0.2 | -0.1 | -0.1 | 0.2 | 0.2 | 0.3 |
| Class D | 0.2 | -0.2 | -0.2 | 0.2 | 0.1 | 0.1 |
| Class E | 0.6 | -1.5 | -1.0 | 0.9 | 1.2 | 1.1 |
| All | 0.4 | -0.3 | -0.4 | 0.6 | 0.6 | 0.6 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.1 | -0.1 | 0.2 | 0.3 | 0.2 |
| Class B | 0.2 | 0.2 | 0.0 | 0.3 | 0.5 | 0.5 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.2 |
| Class D | 0.1 | 0.2 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.0 | 0.2 | 0.3 | 0.3 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class C | -0.1 | -0.2 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | -0.1 | 0.2 | 0.0 | -0.1 | 0.0 |
| Class E | 0.2 | -0.7 | 0.0 | 0.1 | -0.3 | 1.0 |
| All | 0.0 | -0.3 | 0.0 | 0.0 | -0.1 | 0.2 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# Cross-check

The experimental results listed in table 2 show the performance of ROT when compared with anchors generated using TMuC 0.7. The Y BD-rate for shows an average increase varying between 0.0% and 0.6%. The chroma fares slightly better varying between an average decrease of -0.4% and an average increase of 0.6% BD bitrate. The results provided by Qualcomm [4] are in close agreement with those listed in Table 2. The small differences when compared to results in [4] can be attributed to platform differences. BD-Rate and BD-SNR results from both parties practically match.

# Additional Discussion

In this cross check, we generated the results presented above.  In addition, we studied the implementation of ROT in the TMuC software and compared it to the description in the TMuC document (JCTVC-B205).  Our observation is that the implementation agrees with the tool description, syntax and semantics in the TMuC document.  Also, note that in our tests MDDT is enabled – so the ROT tool is disabled only on blocks with size greater than 8x8.  As such, we did not explicitly test the implementation of the 4x4 ROT that is described in the document and represented in the TComTrQuant::InvRotTransform4I function.  Additionally, we note that most of the Class A encoding did not complete on our system due to hardware problems. In these cases, we have used the values reported by Qualcomm. These points are marked in bold in the XLS.

We observe that the gains of the ROT tool are modest.  We also observe that the gains in the All Intra, High Coding Efficiency condition come at the expense of chroma performance.

# Conclusion

The evaluation and cross-check of ROT results was reported. The experimental results show an average increase varying between 0.0% and 0.6% for Y BD-rate. The results were confirmed to be consistent with the results provided by the Qualcomm [4].

# Reference

- JCTVC “Common test conditions and software reference configurations”, JTCVC-B300, 2nd JCT-VC Meeting, Geneva, July 2010.

- JCTVC “Tool Experiments 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd JCT-VC Meeting, Geneva, July 2010.

- JCT-VC “Draft Test Model under Consideration”, JCTVC-B205, 2nd JCT-VC Meeting, Geneva, July 2010.

- R. Joshi, M. Coban, M. Karczewicz, "TE12: Report on evaluation of MDDT and ROT", JCTVC-C268, 3rd JCT-VC Meeting, Guangzhou, October 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.