| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C182 WG11 Number: m18211 |
| --- | --- |

| Title: | TE12.2: Cross-check of Adaptive Motion Vector Resolution (AMVRES) Technology in the TMuC Software |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Andrew Segall and Jie Zhou 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution provides the cross-check of Adaptive Motion Vector Resolution (AMVRES) tool by Qualcomm [1]. This cross-check was performed within the context of TE12. It contains two cases.

- Cross-check of AMVRES in TMuC0.7

- Cross-check of improved AMVRES called AMVRES with unified design.

Qualcomm provided coded bitstreams for both cases, and modified TMuC software containing AMVRES with unified design tool. For both AMVRES in TMuC0.7 and AMVRES with unified design, we verified that the bitstreams are able to be correctly decoded and generated same RD result as in the excel sheet provided by Qualcomm.

# Experimental condition

The evaluation of AMVRES defined in TE12 was carried out. For AMVRES in TMuC 0.7, we used the TMuC0.7 decoder; for AMVRES with unified design, we used the modified TMuC0.7 software provided by Qualcomm. We compiled the software, decoded the bitstreams and calculated the RD data. The computing platform is summarized in Table 1. Decoding is done on a cluster, whose speed varies among nodes.

Table 1: Computing platforms

|  | Decoding |
| --- | --- |
| OS | Linux (64bit) |
| CPU | Intel(R) Core(TM)2 CPU 6300 @ 1.86GHz cache size : 2048 KB |
| Compiler | gcc 4.3.2 |

We verified the following cases.

- TMuC0.7 AMVRES

  - Low Delay High Efficiency

  - Random Access High Efficiency

- TMuC0.7 AMVRES + Unified design

  - Low Delay High Efficiency

  - Random Access High Efficiency

# Performance experiments

The rate and PSNR of the decoded YUV files are same as the data listed in the excel sheet provided by Qualcomm. This verifies that the RD performance of the AMVRES tool is as shown in their excel sheet (included in the attachment JCTVC-C182-xxx.xls).The result is also summarized in the tables below.

Table 1. RD result of AMVRES in TMuC0.7 On/Off (High Efficiency)

| Random access. Y BD-rate; U BD-rate; V BD-rate. Class A; -0.1; -0.6; -0.8. Class B; 0.6; -0.4; 0.0. Class C; 0.5; -0.5; -0.4. Class D; 0.8; -0.5; -0.3. Class E. All; 0.5; -0.5; -0.3. | Low delay. Y BD-rate; U BD-rate; V BD-rate. Class A. Class B; 0.1; -2.6; -3.0. Class C; 0.2; -1.8; -2.1. Class D; 1.3; -4.3; -3.2. Class E; 0.2; 2.3; 3.7. All; 0.4; -1.9; -1.6. |
| --- | --- |

Table 2. Results of “AMVRES + Unified design” On/Off

| Random access. Y BD-rate; U BD-rate; V BD-rate. Class A; 0.0; -0.7; -0.7. Class B; 0.7; -0.4; -0.1. Class C; 0.6; -0.4; -0.3. Class D; 0.9; -0.2; -0.2. Class E. All; 0.6; -0.4; -0.3. | Low delay. Y BD-rate; U BD-rate; V BD-rate. Class A. Class B; 0.8; -1.8; -2.4. Class C; 1.1; -1.0; -1.6. Class D; 2.3; -3.5; -2.2. Class E; 1.5; 4.0; 4.9. All; 1.4; -1.0; -0.8. |
| --- | --- |

# Cross-check

The experimental results are compared with the results provided by Qualcomm. The RD results of the decoded files are same as the results provided by Qualcomm.

# Additional Comments

This verification report considers two versions of the AMVRES tool.  The first version is the method included in TMuC 0.7.  The second version is a “unified method”.  This second method addresses compatibility issues related to AMVRES, GPB and MRG tools in the TMuC.  Our comments on the two methods follows:

For the first version (TMuC 0.7), we verified the performance of the software as reported above.  Here, we had access to bit-streams generated by Qualcomm.  We decoded these bit-streams and verified R-D performance.  Moreover, we encoded a sub-set of the test sequences to provide additional verification of the bit-stream data.  Note that this was not a full sweep of the sequences and configurations, it was simply a sub-test.  We note in the result that there is an interesting trade-off between the luma and chroma gains of the tool – specifically, the luma gains appears to come at the expense of chroma.

For the first version, we also inspected the software source code and compared it to the description, syntax and semantics as presented in the TMuC document. (JCTVC-B205). Our assessment is that the software matches the description of the tool.  Additionally, in discussion with Qualcomm, we decided to only test the high efficiency configuration of the TE12 activity.  This is because the low complexity configuration uses the diagonal filter, and the diagonal filter does not support AMVRES.  (Thus, AMVRES is effectively disabled in low complexity mode.)

For the second version, we used software sent to us directly by Qualcomm.  This software contains a “unified design”, which addresses issues related to the AMVRES, GPB and MRG tools.  This was initially reported in an e-mail on August 17, 2010, (3:16PM) to the reflector.  Our verification of this software was slightly less detailed than the verification of the TMuC 0.7 scenario.  Our understanding of the “unified design” is that it’s significant modification is related to the entropy coding/decoding of the adaptive motion vector resolution flags (*_mvres_*) and handling of the sub-partitions motion vector resolution.  We also note an increase in the number of context models from 4 to 10. We did not have modified syntax, semantics or descriptions available and so did not compare to that information.

# Conclusion

The cross-check results of AMVRES in TMuC0.7 and additional improvement of AMVRES proposed by Qualcomm are reported. The experimental results show that the encoded bitstream provided by Qualcomm can be corrected decoded and yield same RD results as stated in the proponent’s result sheet.

# Reference

- JCT-VC “Draft Test Model under Consideration”, JCTVC-B205, 2nd JCT-VC Meeting, Geneva, July 2010.

- JCTVC “Tool Experiments 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd JCT-VC Meeting, Geneva, July 2010.