| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C183 WG11 Number: m18212 |
| --- | --- |

| Title: | Enhanced Switching of Interpolation Filter for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonobu Yoshino Sei Naito Shigeyuki Sakazawa 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7587 to-yoshino@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories) |  |  |

_____________________________

# Abstract

Adaptive Interpolation Filter (AIF) technology [1,2] has been proposed to improve the interpolation filter performance for fractional-pel motion compensation. However, this scheme requires huge computational cost for calculating the filter coefficients on the encoder side. This contribution introduces an Enhanced Switch Interpolation Filter (ESIF) approach that pre-defines several interpolation filter coefficients in order to reduce the computational cost for calculating the coefficients. This scheme allows updating the number of filter coefficient sets and the filter coefficients slice by slice. The experimental results show that the BD-bitrate against H.264 reached 1.9% under CS2 condition.

# Introduction

H.264 standard supports 1/4-pel accuracy for motion compensated prediction technology. This standard adopts a 6-tap filter, and the fixed coefficients that are generated based on Wiener Filter method are defined for each fractional-pel position. On the other hand, AIF technology [1, 2] has been proposed to improve the interpolation filter performance. This scheme calculates filter coefficients that minimize the motion compensated error for every slice. However, this technology requires huge computational cost for calculating the filter coefficients. This contribution introduces an Enhanced Switch Interpolation Filter technology that pre-defines interpolation filter coefficient sets in order to reduce the processing time for calculating the filter coefficients. This scheme updates the number of filter coefficient sets and refines the filter coefficients slice by slice. Furthermore, the interpolation filter is separable for the horizontal and vertical direction. Thus, the filter coefficient set is independently determined for each direction.

# Selectable Adaptive Interpolation Filter

This scheme pre-defines several filter coefficient sets, and a filter coefficient set is determined for every slice. The interpolation filter is assumed as a 6-tap filter for 1/4-pel motion compensated prediction. Figure 1 shows the fractional-pel position for the 1/4-pel motion compensated prediction. This approach defines the filter coefficients for fractional-pel position b and h, and filter coefficients for the other fractional-pel positions are generated by linear interpolation process from the coefficients of neighboring position. Actually, coefficients in 1/4-pel precision are calculated based on bi-linear interpolation from the coefficients of 1/2-pel and integer-pel position.

Fractional-pixel

Integer-pixel

Fig.1 Fractional-pixel position

This scheme defines the N types of filter coefficients for fractional-pel position b and h, and N is allowed to be updated slice by slice.

The pre-defined filter coefficients for the 1/2-pel position are descrbed in the bitstream. Actually, the coefficients are descrbed as Picture Parameter Set (PPS) syntax element. The additional syntax elements for PPS is shown in the following.

- Flag to identify whether ESIF is ON or OFF.

- The number of pre-defined filter coefficient sets

- Filter coefficients

The additional syntax element for the inter coding slice header is as follows.

- Flag to identify whether the coefficients pre-defined in PPS are available for the current slice.

- Index value to identify the selected coefficient set, if ESIF is available in the current slice.

In the case when ESIF is disabled, the coefficients defined within H.264 standard are applied to all fractional-pel position.

# Experimental Results

We conducted a coding experiment to evaluate the performance of the new coding mode. The proposed scheme introduced in Section 2 was implemented on JM17.0. For simplification, the following fixed filter coefficients were applied for all sequences and all frames in this experiment. The number of coefficient sets was four (4).

- Filter set 1 : [13 -40 155 155 -40 13]/256

- Filter set 2 : [-5 -28 161 161 -28 -5]/256

- Filter set 3 : [-30 30 128 128 30 -30]/256

- Filter set 4 : [-22 38 112 112 38 -22]/256

The coding condition was compliant with N11113. Table 1 shows the coding performance against JM17.0 under Beta anchor condition. The coding performance was evaluated based on BD-bitrate and BD-PSNR.

Table 1 : Experimental results for CS2

| Class | Sequence | BD-bitrate[%] | BD-PSNR[dB] |
| --- | --- | --- | --- |
| B1 | Kimono | -3.21 | 0.14 |
|  | ParkScene | -2.49 | 0.09 |
| B2 | Cactus | -0.49 | 0.02 |
|  | BasketBallDrive | -2.37 | 0.08 |
|  | BQTerrace | -3.84 | 0.10 |
| C | BasketBallDrill | -0.96 | 0.04 |
|  | BQMall | -1.30 | 0.06 |
|  | PartyScene | -0.28 | 0.01 |
|  | RaceHorses | -1.24 | 0.05 |
| D | BasketBallPass | -2.50 | 0.12 |
|  | BQSquare | -0.14 | 0.01 |
|  | BlowingBubbles | -0.98 | 0.04 |
|  | RaceHorses | -0.78 | 0.04 |
| E | Vidyo1 | -3.00 | 0.13 |
|  | Vidyo3 | -4.16 | 0.20 |
|  | Vidyo4 | -2.69 | 0.11 |
| Average for all sequences |  | -1.90 | 0.08 |

In this experiment, the coefficient set was determined based on R-D optimization criteria, thus large computational cost was required. However, several low-complexity interpolation filter approaches [3, 4], which are expected to improve the computational efficiency for deciding the best interpolation filter, have been proposed. By utilizing such implementation technologies, it is expected that the decision process of the coefficient set can be conducted at the complexity corresponding to the practical implementation.

# Conclusion

This contribution introduces an ESIF scheme that pre-defines interpolation filter coefficient sets. The number of filter coefficient sets and filter coefficients are adaptively updated slice by slice, thus ESIF has high flexibility compared to H.264 standard with the single set of static filter coefficients. The experimental results show that BD-bitrate against H.264 reaches 1.9% under CS2 condition, even under the simple enhancement case where adaptive selection is conducted from four predefined filter sets.

# Patent rights declaration(s)

KDDI Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Dmytro Rusanovskyy et al., " Adaptive Interpolation with Directional Filters," ITU-T Q.6/SG16 VCEG, VCEG-AG21, Oct., 2007

[2] Yan Ye et al., " Enhanced Adaptive Interpolation Filter," ITU-T Q.6/SG16 VCEG, C464, Apr., 2008

[3] Tomonobu Yoshino et al., "Low-complexity Scheme for Adaptive Interpolation Filter Based on Amplitude Characteristic Analysis," ICIP2010, Sep. 2010.

[4] Kai Zhang, et al., "A Single-pass based Adaptive Interpolation Filtering Algorithm for Video Coding," ICIP2010, Sep. 2010.