| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C184 WG11 Number: m18215 |
| --- | --- |

| Title: | TE12.6 PIPE tested against LCEC by Cisco |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Arild Fuldseth | Tel: Email: | +47 92414082 arilfuld@cisco.com |
| Source: | Cisco Systems Norway |  |  |

_____________________________

# Abstract

This document presents the TE12 results on PIPE vs. LCEC comparisons. Depending on the configuration (low delay/random access/intra and low complexity/high efficiency), average BD-Rate figures between 12.3% and 18.0% are observed.

- Introduction

Document JCTVC-B300 [1] specifies two entropy coders; PIPE and a low complexity entropy coder (LCEC) to be used in high efficiency and low complexity default configurations respectively. As part of Tool Experiment 12 (TE12) the difference in compression efficiency between PIPE and LCEC has been assessed by comparing the default configuration of [1] with a modified configuration as follows:

Modified low complexity configuration:	Use PIPE instead of LCEC

Modified high efficiency configuration:	Use LCEC instead of PIPE

The difference in compression performance has been measured using the BD-Rate measure [2]. Low complexity and high efficiency results are provide for intra, random access and low delay configurations. Furthermore, as specified in [1], sequences of five different resolutions (class A-E) have been tested.

# Results

The average BD-Rate numbers for each class and for all classes together are listed in Table 1. Detailed results can be found in the attached spreadsheet.

In order to get more reliable comparisons of encoder- and decoder-time all sequences/QP values for a particular comparison were run sequentially on the same CPU. To reduce the total simulation time, only 9 frames of each sequence were used. The results were then multiplied by N/9, where N is the number of frames for the full length sequence, to normalize the results.

Table 1  BD-Rate numbers for PIPE vs. LCEC

|  | Intra |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | 14,4 |  | 8,1 | 8,5 | -12,6 |  | -7,2 | -7,3 |  |
| Class B | 15,0 |  | 7,9 | 8,6 | -13,1 |  | -7,9 | -8,2 |  |
| Class C | 12,5 |  | 8,1 | 8,6 | -11,2 |  | -7,7 | -8,2 |  |
| Class D | 11,8 |  | 7,6 | 8,4 | -10,6 |  | -7,1 | -7,7 |  |
| Class E | 16,4 |  | 11,1 | 9,9 | -14,5 |  | -9,6 | -8,5 |  |
| All | 13,9 |  | 8,4 | 8,8 | -12,3 |  | -7,9 | -8,0 |  |
| Enc Time[%] | 57 % |  |  |  | 178 % |  |  |  |  |
| Dec Time[%] | 95 % |  |  |  | 105 % |  |  |  |  |
|  | Random access |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | 18,7 |  | 11,0 | 9,7 | -15,1 |  | -9,1 | -8,2 |  |
| Class B | 21,8 |  | 11,7 | 11,0 | -17,8 |  | -9,3 | -7,8 |  |
| Class C | 15,8 |  | 8,3 | 9,0 | -13,5 |  | -7,5 | -7,8 |  |
| Class D | 15,0 |  | 6,7 | 7,7 | -13,0 |  | -4,6 | -5,4 |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All | 18,0 |  | 9,4 | 9,4 | -15,0 |  | -7,6 | -7,2 |  |
| Enc Time[%] | 84 % |  |  |  | 141 % |  |  |  |  |
| Dec Time[%] | 98 % |  |  |  | 102 % |  |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 17,6 |  | 0,7 | -2,0 | -15,7 |  | -2,6 | 0,1 |  |
| Class C | 11,7 |  | 1,3 | 0,6 | -11,9 |  | -2,7 | -2,7 |  |
| Class D | 12,6 |  | -4,2 | -3,4 | -11,0 |  | 3,3 | 1,9 |  |
| Class E | 32,0 |  | 3,4 | 11,2 | -24,8 |  | -7,4 | -12,5 |  |
| All | 17,6 |  | 0,1 | 0,8 | -15,3 |  | -2,1 | -2,5 |  |
| Enc Time[%] | 81 % |  |  |  | 132 % |  |  |  |  |
| Dec Time[%] | 98 % |  |  |  | 102 % |  |  |  |  |

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

# Patent rights declaration(s)

Cisco Systems Norway may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).