| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C185r1 WG11 Number: m18216 |
| --- | --- |

| Title: | Recent improvements of the low complexity entropy coder in TMuC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Arild Fuldseth, Gisle Bjøntegaard (Cisco) Kemal Ugur, Antti Hallapuro, Jani Lainema (Nokia) | Tel: Email: | +47 92414082 arilfuld@cisco.com +358 50 486087 kemal.ugur@nokia.com |
| Source: | Cisco, Nokia |  |  |

_____________________________

# Abstract

This contribution reports on recent improvements of the low complexity entropy coder (LCEC) in the Test Model under Consideration (TMuC). The document describes LCEC phase 2 which was included in TMuC v0.7.4 and LCEC phase 3 which has not bee adopted in the TMuC software yet. Also, results on the transform skip flag (provided by HHI) are included. Compared with the anchor conditions defined in JCTVC_B300 the combination of phase2 & phase3 provides BD-Rate gains between 2.9% (low delay) and 0.3% (intra). However, with more favorable settings of the residual quadtree macros, HHI_RQT and HHI_RQT_ROOT, total gains between 6.9% (low delay) and 0.2% (intra) are observed.  With all LCEC improvements enabled, the gain of PIPE over LCEC is 9.1%, 10.4%, and 11.8% for low delay, random access, and intra respectively.

- Introduction

The low complexity entropy coder (LCEC) which is based on the JCTVC-A119 CfP proposal [1] has been adopted into TMuC as described in JCTVC-B205 [2]. This has proved to be a challenging task, especially since the coding structures of the two algorithms are quite different. In particular, the side information parameters of JCTVC-A119 could not be directly mapped to the side information parameters of TMuC and the strategy of JCTVC-A119 had to be modified as appropriate.

LCEC has been integrated into the TMuC software in four phases:

Phase 0:	TMuC v0.1, preliminary version integrated by Samsung&BBC.

Phase 1:	TMuC v0.7, used as anchor in TE12 low complexity configurations, 
(including the PIPE vs. LCEC experiment).

Phase 2:	TMuC v0.7.4, disabled by default but evaluated as an independent tool as part of TE12.

Phase 3:	Recent improvements that are not adopted for integration in the official TMuC software yet.

In this document, BD-Rate [3] results for LCEC phase 2 and phase 3 relative to phase 1 are provided. Results with and without residual quadtree (HHI_RQT) and the transform skip flag (HHI_RQT_ROOT) are reported. The reason for studying the LCEC improvements together with the two RQT macros is that significant side information parameters such as the coded block flag (CBF) are treated differently dependent on the values of these RQT macros. Finally, a comparison between PIPE and the improved LCEC is given.

- Description of LCEC software integration phases

Phase 0 (TMuC v0.1)

The initial version of LCEC was very much work in progress, and included the following:

- Coefficient encoding as specified in JCTVC-A119 and JCTVC-B205.

- Interleaving of transform coefficients (similar to CALVC/8x8 transform in AVC/H.264) for transforms larger than 8x8 to reuse the 8x8 LCEC coefficient coding kernel.

- A somewhat rudimentary method for encoding of side information.

Phase 1 (TMuC v0.7 – enabled by default)

- Basic RDOQ supported.

- Replacing interleaving with forcing high frequency coefficients of large transforms (>8x8) to zero.

- The adaptive sorting (ranking) tables that are used for coefficients coding are updated during the first encoding pass for better RDO-based mode selection.

Phase 2 (TMuC v0.7.4 – disabled by default)

- Joint coding of coded block flags (CBFs) for Y, U, and V at CU level using VLC with adaptive sorting tables. A VLC code with one bit for the most probable combination of the CBF values is used. Note that this feature is only implemented with RQT disabled. The transform skip flag implements a similar feature when RQT is enabled.

- Joint and adaptive VLC coding of reference frame index, inter prediction direction, and adaptive motion vector resolution flag.

- Better bit estimates for RDOQ by passing the adaptive sorting tables used for coefficient coding to the RDOQ function.

Phase 3 (not yet integrated in TMuC software)

- Improved default values of adaptive sorting tables.

- Adaptive VLC coding of intra prediction direction.

- Minor tuning of RDOQ to align intra slice PSNR with “RDOQ=off” and PIPE.

- Elimination of the CU-level terminating bit.

- Joint coding of merge flag and transform skip flag using a fixed VLC table (0,10,110,111).

- Results

This section provides average results for various low complexity configurations (low delay, random access, and intra).

3.1 LCEC phase2

In Tables 1-3, BD-Rate results for LCEC phase 2 are given for low complexity configurations as defined in JCTVC-B300 [4]. Results for various combinations of RQT on/off and phase2 on/off are provided. As can be seen from the table, phase 2 provides most gain for RQT=off. This is because phase 2 includes joint coding of CBFs in that case. That method is not directly extendable to the case where RQT=on. However, as discussed below, the transform skip flag (HHI_RQT_ROOT) provides a similar solution for the RQT=on case.

Table 1 BD-Rate results (%) for low delay.
(Anchor: Phase2=off, RQT=on)

|  |  | RQT=on | RQT=off | RQT=off |
| --- | --- | --- | --- | --- |
|  | Sequence | Phase2=on | Phase2=off | Phase2=on |
| Class A |  |  |  |  |
| Class B | Kimono | -1,4 | -0,8 | -5,6 |
|  | ParkScene | -1,7 | 1,5 | -5,1 |
|  | Cactus | -1,6 | 0,6 | -5,3 |
|  | BasketballDrive | -1,7 | 0,3 | -5,1 |
|  | BQTerrace | -1,2 | 2,8 | -4,1 |
| Class C | BasketballDrill | -1,8 | 0,7 | -4,0 |
|  | BQMall | -2,1 | 1,0 | -5,7 |
|  | PartyScene | -1,8 | 2,8 | -2,2 |
|  | RaceHorses | -1,8 | 1,5 | -3,1 |
| Class D | BasketballPass | -2,0 | 0,5 | -4,5 |
|  | BQSquare | -1,4 | 3,5 | -2,1 |
|  | BlowingBubbles | -1,8 | 2,1 | -3,0 |
|  | RaceHorses | -2,4 | 0,9 | -4,4 |
| Class E | Vidyo1 | -2,2 | -2,3 | -12,6 |
|  | Vidyo3 | -1,8 | -1,1 | -11,5 |
|  | Vidyo4 | -1,7 | -1,7 | -11,1 |
|  | Class A |  |  |  |
|  | Class B | -1,5 | 0,9 | -5,0 |
|  | Class C | -1,9 | 1,5 | -3,8 |
|  | Class D | -1,9 | 1,7 | -3,5 |
|  | Class E | -1,9 | -1,7 | -11,7 |
|  | All | -1,8 | 0,8 | -5,6 |

Table 2 BD-Rate results (%) for random access.
(Anchor: Phase2=off, RQT=on)

|  |  | RQT=on | RQT=off | RQT=off |
| --- | --- | --- | --- | --- |
|  | Sequence | Phase2=on | Phase2=off | Phase2=on |
| Class A | Traffic | -0,4 | -0,5 | -4,8 |
|  | PeopleOnStreet | -0,4 | -1,5 | -6,2 |
| Class B | Kimono | -0,2 | -1,6 | -5,8 |
|  | ParkScene | -0,4 | -0,1 | -4,8 |
|  | Cactus | -0,3 | -1,0 | -5,6 |
|  | BasketballDrive | -0,3 | -0,7 | -5,0 |
|  | BQTerrace | 0,3 | 0,6 | -4,4 |
| Class C | BasketballDrill | -0,2 | 0,3 | -3,1 |
|  | BQMall | -0,4 | 0,0 | -5,0 |
|  | PartyScene | -0,8 | 0,8 | -3,0 |
|  | RaceHorses | -0,4 | 0,3 | -3,1 |
| Class D | BasketballPass | -0,1 | -0,2 | -3,9 |
|  | BQSquare | -0,9 | 1,1 | -3,8 |
|  | BlowingBubbles | -0,6 | 0,4 | -3,7 |
|  | RaceHorses | -0,3 | -0,3 | -4,1 |
| Class E |  |  |  |  |
|  | Class A | -0,4 | -1,0 | -5,5 |
|  | Class B | -0,2 | -0,6 | -5,1 |
|  | Class C | -0,4 | 0,4 | -3,6 |
|  | Class D | -0,5 | 0,2 | -3,9 |
|  | Class E |  |  |  |
|  | All | -0,4 | -0,2 | -4,4 |

Table 3 BD-Rate results (%) for intra.
(Anchor: Phase2=off, RQT=on)

|  |  | RQT=on | RQT=off | RQT=off |
| --- | --- | --- | --- | --- |
|  | Sequence | Phase2=on | Phase2=off | Phase2=on |
| Class A | Traffic | -0,2 | -0,3 | -1,3 |
|  | PeopleOnStreet | -0,2 | -0,1 | -1,4 |
| Class B | Kimono | 0,0 | -1,2 | -1,9 |
|  | ParkScene | -0,2 | 0,2 | -0,9 |
|  | Cactus | -0,3 | 0,8 | -0,5 |
|  | BasketballDrive | -0,3 | 0,7 | -0,7 |
|  | BQTerrace | -0,5 | 1,2 | -0,3 |
| Class C | BasketballDrill | -0,2 | 2,4 | 1,1 |
|  | BQMall | -0,1 | 1,1 | -0,2 |
|  | PartyScene | -0,7 | 1,0 | -0,8 |
|  | RaceHorses | -0,4 | 0,9 | -0,3 |
| Class D | BasketballPass | -0,2 | 1,0 | -0,1 |
|  | BQSquare | -0,8 | 1,2 | -0,7 |
|  | BlowingBubbles | -0,5 | 1,2 | -0,2 |
|  | RaceHorses | -0,1 | 0,9 | -0,1 |
| Class E | Vidyo1 | -0,1 | 0,0 | -1,4 |
|  | Vidyo3 | -0,1 | 0,3 | -1,2 |
|  | Vidyo4 | -0,1 | -0,3 | -1,6 |
|  | Class A | -0,2 | -0,2 | -1,3 |
|  | Class B | -0,3 | 0,4 | -0,8 |
|  | Class C | -0,3 | 1,3 | 0,0 |
|  | Class D | -0,4 | 1,1 | -0,3 |
|  | Class E | -0,1 | 0,0 | -1,4 |
|  | All | -0,3 | 0,6 | -0,7 |

2.2 LCEC – Phase3

In Table 4, BD-Rate results for phase 2 and phase 2 & phase 3 combined are provided for low complexity configurations as defined in JCTVC-B300 [4].

Table 4 BD-Rate results (%) for various combinations of Phase2 & Phase 3.
(Anchor: Phase2=off, Phase2=off, RQT=on)

|  |  | Low delay |  | Random access |  | Intra |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Phase2=on | Phase2=on | Phase2=on | Phase2=on | Phase2=on | Phase2=on |
|  | Sequence | Phase3=off | Phase3=on | Phase3=off | Phase3=on | Phase3=off | Phase3=on |
| Class A |  |  |  | -0,4 | -1,7 | -0,2 | -0,2 |
|  |  |  |  | -0,4 | -0,7 | -0,2 | -0,8 |
| Class B | Kimono | -1,4 | -2,1 | -0,2 | -0,8 | 0,0 | 0,5 |
|  | ParkScene | -1,7 | -2,5 | -0,4 | -1,4 | -0,2 | -0,3 |
|  | Cactus | -1,6 | -2,9 | -0,3 | -1,3 | -0,3 | 0,0 |
|  | BasketballDrive | -1,7 | -2,6 | -0,3 | -1,0 | -0,3 | -0,1 |
|  | BQTerrace | -1,2 | -2,0 | 0,3 | -0,2 | -0,5 | -0,2 |
| Class C | BasketballDrill | -1,8 | -2,6 | -0,2 | -1,5 | -0,2 | 0,4 |
|  | BQMall | -2,1 | -3,0 | -0,4 | -1,2 | -0,1 | -0,4 |
|  | PartyScene | -1,8 | -2,4 | -0,8 | -1,5 | -0,7 | -0,5 |
|  | RaceHorses | -1,8 | -2,2 | -0,4 | -0,4 | -0,4 | 0,0 |
| Class D | BasketballPass | -2,0 | -2,9 | -0,1 | -0,8 | -0,2 | -0,2 |
|  | BQSquare | -1,4 | -2,0 | -0,9 | -1,5 | -0,8 | -0,8 |
|  | BlowingBubbles | -1,8 | -2,3 | -0,6 | -1,2 | -0,5 | -0,3 |
|  | RaceHorses | -2,4 | -2,6 | -0,3 | -0,4 | -0,1 | 0,1 |
| Class E | Vidyo1 | -2,2 | -5,2 |  |  | -0,1 | -0,6 |
|  | Vidyo3 | -1,8 | -4,3 |  |  | -0,1 | -0,6 |
|  | Vidyo4 | -1,7 | -4,7 |  |  | -0,1 | -0,8 |
|  | Class A |  |  | -0,4 | -1,2 | -0,2 | -0,5 |
|  | Class B | -1,5 | -2,4 | -0,2 | -0,9 | -0,3 | 0,0 |
|  | Class C | -1,9 | -2,5 | -0,4 | -1,2 | -0,3 | -0,1 |
|  | Class D | -1,9 | -2,4 | -0,5 | -1,0 | -0,4 | -0,3 |
|  | Class E | -1,9 | -4,7 |  |  | -0,1 | -0,7 |
|  | All | -1,8 | -2,9 | -0,4 | -1,0 | -0,3 | -0,3 |

3.3 Combination of LCEC phase2, LCEC phase3, and the transform skip flag

In Tables 5 and 6, accumulated results for the transform skip flag (HHI_RQT_ROOT), LCEC phase 2, and LCEC phase 3 are provided for the low delay, low complexity configuration. The configurations defined in JCTVC-B300 [4] are used except that the sequence length is limited to one second. Table 5 shows results for RQT=off, while Table 6 shows results for RQT=on. The reason for including the transform skip flag in these experiments is that this flag does approximately the same with RQT=on as LCEC phase 2 does with RQT=off (allows for use of one bit to signal whether there are coefficients in a CU or not). As can be seen from the table, RQT=on & RQT=off are comparable provided that both LCEC phase2 and the transform skip flag are enabled. The additional gain of phase 3 is larger for RQT=on than for RQT=off due to joint coding of the transform skip flag (only present with RQT=on) and the merge flag.

Table 5 Average BD-Rate results (%) for low delay, RQT=off
(Anchor:  Phase2=off, Phase3=off, Transform skip flag =off, RQT=on)

|  |  | Phase2 = off | Phase2 = on | Phase2 = on |
| --- | --- | --- | --- | --- |
|  | Sequence | Phase3 = off | Phase3 = off | Phase3 = on |
| Class A |  |  |  |  |
| Class B | Kimono | -0,7 | -4,0 | -4,4 |
|  | ParkScene | 0,5 | -5,3 | -7,0 |
|  | Cactus | 0,2 | -4,8 | -6,4 |
|  | BasketballDrive | -0,4 | -5,5 | -6,9 |
|  | BQTerrace | 3,1 | -2,1 | -3,1 |
| Class C | BasketBallDrill | 1,2 | -3,3 | -5,5 |
|  | BQMall | 0,8 | -5,1 | -6,7 |
|  | PartyScene | 2,3 | -2,2 | -3,4 |
|  | RaceHorses | 1,7 | -2,8 | -3,1 |
| Class D | BasketballPass | -0,5 | -6,1 | -9,2 |
|  | BQSquare | 3,7 | -2,4 | -3,4 |
|  | BlowingBubbles | 1,3 | -3,0 | -3,9 |
|  | RaceHorses | 0,6 | -4,1 | -4,6 |
| Class E | Vidyo1 | -2,9 | -11,3 | -14,8 |
|  | Vidyo3 | -0,6 | -9,3 | -11,7 |
|  | Vidyo4 | -2,1 | -9,7 | -12,9 |
|  | Class A |  |  |  |
|  | Class B | 0,5 | -4,3 | -5,6 |
|  | Class C | 1,5 | -3,4 | -4,7 |
|  | Class D | 1,3 | -3,9 | -5,3 |
|  | Class E | -1,8 | -10,1 | -13,1 |
|  | All | 0,5 | -5,1 | -6,7 |

Table 6 Average BD-Rate results (%) for low delay, RQT=on, Transform skip flag = on
(Anchor:  Phase2=off, Phase3=off, Transform skip flag =off, RQT=on)

|  |  | Phase2 = off | Phase2= on | Phase3= on |
| --- | --- | --- | --- | --- |
|  | Sequence | Phase3 = off | Phase3 = off | Phase3 = on |
| Class A |  |  |  |  |
| Class B | Kimono | -1,2 | -2,4 | -2,7 |
|  | ParkScene | -3,5 | -5,2 | -7,6 |
|  | Cactus | -3,0 | -4,5 | -6,3 |
|  | BasketballDrive | -2,1 | -3,9 | -5,2 |
|  | BQTerrace | -2,5 | -3,3 | -4,9 |
| Class C | BasketBallDrill | -1,7 | -3,5 | -5,5 |
|  | BQMall | -3,1 | -5,1 | -6,7 |
|  | PartyScene | -0,8 | -2,6 | -3,4 |
|  | RaceHorses | -1,2 | -3,0 | -3,1 |
| Class D | BasketballPass | -4,5 | -6,8 | -10,1 |
|  | BQSquare | -1,9 | -3,8 | -4,8 |
|  | BlowingBubbles | -1,3 | -3,0 | -4,2 |
|  | RaceHorses | -1,7 | -4,1 | -4,4 |
| Class E | Vidyo1 | -9,0 | -10,7 | -15,5 |
|  | Vidyo3 | -7,2 | -9,2 | -12,6 |
|  | Vidyo4 | -6,8 | -8,8 | -13,5 |
|  | Class A |  |  |  |
|  | Class B | -2,5 | -3,9 | -5,3 |
|  | Class C | -1,7 | -3,6 | -4,7 |
|  | Class D | -2,4 | -4,4 | -5,9 |
|  | Class E | -7,7 | -9,5 | -13,9 |
|  | All | -3,2 | -5,0 | -6,9 |

In Table 7, BD-Rate results with phase 2, phase 3, and transform skip flag enabled are shown for low delay, random access, and intra. The configurations defined in JCTVC-B300 [4] are used except that sequences of length one second are used.

Table 7 BD-Rate results (%) with RQT=on, Phase2=on, Phase3=on, transform skip flag=on.
(Anchor: Phase2=off, Phase3=off, Transform skip flag =off, RQT=on)

|  | Sequence | Low delay | Random access | Intra |
| --- | --- | --- | --- | --- |
| Class A | Traffic |  | -6,2 | -0,3 |
|  | PeopleOnStreet |  | -5,0 | -0,8 |
| Class B | Kimono | -2,7 | -3,6 | 1,3 |
|  | ParkScene | -7,6 | -6,0 | -0,4 |
|  | Cactus | -6,3 | -5,7 | -0,1 |
|  | BasketballDrive | -5,2 | -6,2 | -0,2 |
|  | BQTerrace | -4,9 | -5,6 | -0,5 |
| Class C | BasketBallDrill | -5,5 | -4,9 | 0,4 |
|  | BQMall | -6,7 | -5,7 | -0,3 |
|  | PartyScene | -3,4 | -3,0 | -0,4 |
|  | RaceHorses | -3,1 | -3,1 | -0,1 |
| Class D | BasketballPass | -10,1 | -8,1 | -0,1 |
|  | BQSquare | -4,8 | -5,0 | -0,8 |
|  | BlowingBubbles | -4,2 | -3,7 | -0,1 |
|  | RaceHorses | -4,4 | -3,6 | 0,2 |
| Class E | Vidyo1 | -15,5 |  | -0,6 |
|  | Vidyo3 | -12,6 |  | -0,7 |
|  | Vidyo4 | -13,5 |  | -0,8 |
|  | Class A |  | -5,6 | -0,5 |
|  | Class B | -5,3 | -5,4 | 0,0 |
|  | Class C | -4,7 | -4,2 | -0,1 |
|  | Class D | -5,9 | -5,1 | -0,2 |
|  | Class E | -13,9 |  | -0,7 |
|  | All | -6,9 | -5,0 | -0,2 |

In Table 8, BD-Rate results for PIPE vs. LCEC are shown. For LCEC, phase2, phase3 and the transform skip flag are enabled. The configurations defined in JCTVC-B300 [4] are used except that sequences of length one second are used.

Table 8 BD-Rate results (%) for PIPE vs. LCEC 
(Anchor: LCEC with Phase2=on, Phase3=on, transform skip flag =on, RQT=on)

|  | Sequence | Low delay | Random access | Intra |
| --- | --- | --- | --- | --- |
| Class A | Traffic |  | -9,6 | -12,2 |
|  | PeopleOnStreet |  | -10,2 | -11,5 |
| Class B | Kimono | -13,7 | -16,1 | -17,2 |
|  | ParkScene | -9,1 | -11,0 | -12,7 |
|  | Cactus | -9,2 | -12,7 | -12,0 |
|  | BasketballDrive | -15,2 | -16,0 | -14,1 |
|  | BQTerrace | -9,6 | -10,2 | -10,8 |
| Class C | BasketBallDrill | -7,5 | -10,0 | -11,2 |
|  | BQMall | -7,6 | -10,0 | -11,4 |
|  | PartyScene | -8,1 | -8,4 | -9,6 |
|  | RaceHorses | -7,0 | -9,1 | -10,5 |
| Class D | BasketballPass | -5,1 | -8,0 | -11,2 |
|  | BQSquare | -8,1 | -8,5 | -9,0 |
|  | BlowingBubbles | -5,8 | -8,6 | -9,6 |
|  | RaceHorses | -6,3 | -8,4 | -10,3 |
| Class E | Vidyo1 | -11,0 |  | -12,3 |
|  | Vidyo3 | -12,0 |  | -14,1 |
|  | Vidyo4 | -9,7 |  | -13,5 |
|  | Class A |  | -9,9 | -11,9 |
|  | Class B | -11,4 | -13,2 | -13,4 |
|  | Class C | -7,5 | -9,4 | -10,7 |
|  | Class D | -6,3 | -8,4 | -10,0 |
|  | Class E | -10,9 |  | -13,3 |
|  | All | -9,1 | -10,4 | -11,8 |

# Conclusions

Phase2 and phase3 of the LCEC integration into TMuC have been described. Together with use of the transform skip flag, significant BD-Rate gains have been achieved both for RQT=on and RQT=off. We propose to enable LCEC phase 2, LCEC phase 3 and the transform skip flag as default in the TMuC software.

# References

- Kemal Ugur, Kenneth R. Andersson, Arild Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson”, JCTVC-A119, JCT-VC Meeting, April 2010.

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, JCT-VC Meeting, Geneva, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

Cisco Systems Norway may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).