| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C191 WG11 Number: m18226 |
| --- | --- |

| Title: | TE11: Cross-check result of merge/skip (3.2c) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seyoon Jeong, Jinho Lee, Hui Yong Kim, and Sung-Chang Lim ETRI, 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea Kyungyong Kim, Hansoo Lee, and Gwanghoon Park Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 449-701 | Tel: Email: | +82-42-860-5724 {jsy, jinosoul, hykim5, sclim} @etri.re.kr +82-31-201-3680 {kimky, way2bliss, ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

The purpose of this document is to cross-check the efficiency of the ‘Merge’ method and the ‘Skip’ method (3.2c) proposed respectively in [1] and [2]. Simulation results were generated for sequences and coding conditions specified in the document JCTVC-B300 [3]. TMuC version 0.7.3 was used for evaluation.

- Verification of source code change

Samsung has provided the patched TMuC 0.7.3 source code. By comparing the original TMuC 0.7.3 source code with the patched version, we found that some of inter_merge_flag check routines in the prediction_unit() was eliminated in the new version in order to enable the skip and direct modes when the merging mode is on. The prediction_unit() syntaxes correspond to the original source code and the new one are listed below.

The syntax corresponding to the original TMuC 0.7.3:

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if(!inter_merge_flag && slice_type != I) |  |  |
| skip_flag |  |  |
| if( skip_flag ) { |  |  |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else { |  |  |
| if(inter_merge_flag && NumMergeCandidates > 0 ) { |  |  |
| merge_flag | 2 | ue(1) ¦ ae(v) |
| if( merge_flag && NumMergeCandidates > 1 ) |  |  |
| merge_left_flag | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| if(!merge_flag && slice_type != I) { |  |  |
| if(!inter_merge_flag) |  |  |
| pred_mode_1 | 2 | u(1) ¦ ae(v) |
| if(!pred_mode_1) // PredMode != MODE_DIRECT |  |  |
| pred_mode_2 | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( PredMode == MODE_INTRA ) { |  |  |
| … |  |  |
| } |  |  |
| else if( PredMode == MODE_INTER ) { |  |  |
| if(!merge_flag) { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| inter_partitioning_idc | 2 | ue(v) ¦ ae(v) |
| for( i= 0; i < NumPuParts( inter_partitioning_idc ); i++ ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc[ i ] | 2 | ue(v) ¦ ae(v) |
| … |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_DIRECT ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc | 2 | ue(v) ¦ ae(v) |
| … |  |  |
| } |  |  |
| } |  |  |

The changed syntax corresponding to the patched TMuC 0.7.3:

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if(slice_type != I) |  |  |
| skip_flag |  |  |
| if( skip_flag ) { |  |  |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else { |  |  |
| if(inter_merge_flag && NumMergeCandidates > 0 ) { |  |  |
| merge_flag | 2 | ue(1) ¦ ae(v) |
| if( merge_flag && NumMergeCandidates > 1 ) |  |  |
| merge_left_flag | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| if(!merge_flag && slice_type != I) { |  |  |
| if(!inter_merge_flag) |  |  |
| pred_mode_1 | 2 | u(1) ¦ ae(v) |
| if(!pred_mode_1) // PredMode != MODE_DIRECT |  |  |
| pred_mode_2 | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( PredMode == MODE_INTRA ) { |  |  |
| … |  |  |
| } |  |  |
| else if( PredMode == MODE_INTER ) { |  |  |
| if(!merge_flag) { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| inter_partitioning_idc | 2 | ue(v) ¦ ae(v) |
| for( i= 0; i < NumPuParts( inter_partitioning_idc ); i++ ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc[ i ] | 2 | ue(v) ¦ ae(v) |
| … |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_DIRECT ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc | 2 | ue(v) ¦ ae(v) |
| … |  |  |
| } |  |  |
| } |  |  |

- Verification result

The provided source code was compiled with Microsoft Visual Studio 2008 on Windows XP 32-bit operating system. The encoding and decoding platforms used for the experiments are described in the last Sheet of the attached Excel file: “JCTVC-C191_cross-check_result_(ETRI and KHU).xlsx”.

We found no problem in compiling, building, encoding and decoding.

Bit rates and PSNR results were compared: The results are listed in “JCTVC-C191_cross-check_result_(ETRI and KHU)_r1.xlsx”. Our experiment results were almost exactly matched with those provided by Samsung. More specifically, about 0.05% bitrate differences were observed in a few cases. and also were observed in a few cases for anchor comparison. These small differences are most likely due to OS and compiler difference.

The decoding time was measured five times on an Intel Xeon E5420, 8 Core, 2.50GHz XP 32 PC. The mean and standard deviation values of the measured decoding times were summarized in the attached Excel file: “JCTVC-C191_decoding_time_r1.xlsx”.

Due to lack of time, we used several PCs for encoding with different spec. Therefore, we only compared the encoding time ratio when used the same PC for encoding both anchor and ‘3.c’. It was summarized in attached Excel file: “JCTVC-C191_Time_comparison.xls”.

# Reference

| [1] | JCTVC-A116 contribution, Description of video coding technology proposal by Fraunhofer HHI. |
| --- | --- |
| [2] | JCTVC-A124 contribution, Samsung’s Response to the Call for Proposals on Video Compression Technology. |
| [3] | JCTVC-B300 contribution, Common test conditions and software reference configurations, F. Bossen. |