| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C199 WG11 Number: m18237 |
| --- | --- |

| Title: | TE11: Report on experiment 3.2.c: Check skip and merge together |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool experiment report |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee Woo-Jin Han | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this document, the brief description and coding efficiency of a block merging process (MRG) and Skip/Direct mode of TMuC software is presented. Performance of Skip/Direct mode is -0.7% (random access), -2.6% (random access low complexity), 0.1% (low delay) and -2.9% (low delay low complexity) compared to MRG which is turned on by default in TMuC. This contribution also presents combination of MRG and Skip/Direct where Skip/Direct is enabled on top of MRG. It results in -1.2% (random access condition), -2.4% (random access low complexity condition), -0.5% (low delay condition) and -3.3% (low delay low complexity condition).

- Introduction

In TMuC software, Skip/Direct and MRG can be tested by switching encoder option MRG. MRG is turned by default currently [1]. Both Skip/Direct and MRG infers motion information from neighboring blocks. But there is some difference how to derive it. In this contribution, the difference between two methods is described in the next section. And performance of each tool is evaluated and combined method of two tools is considered.

# Differences between MRG and Skip/Direct

Block merging process considers the two spatially neighboring blocks of top-left sample position of the current PU as possible merging candidates. And the current PU derives its motion information from the one of two neighboring blocks when MRG is applied where motion information includes motion vectors, the direction of prediction (forward, backward, or bi-directional) and reference index for each direction. Consequently, an additional flag indicating which neighboring block is merged into current PU among two candidates is sent to decoder explicitly.

Different from MRG, Skip/Direct of TMuC software considers multiple neighboring blocks of the current PU as possible candidates. Those candidates are the same as the motion vector predictors of inter prediction mode where the maximum number of candidates is 5 by combining multiple motion vectors or using itself. Besides motion vectors, every other motion information is inferred in similar way of conventional video codec. That is, reference index is set to 0 and the direction of prediction is set to forward directional and bi-directional in P- and B-slice respectively in Skip mode. In case of a direct mode in B-slice, the direction of prediction is signaled additionally. In Skip mode, no texture is coded.

# Combination of MRG and Skip/Direct

To turn on both MRG and Skip/Direct, syntax is modified where MRG is enabled only if PU is not skipped by skip_flag. For that, skip_flag is coded first. In modified syntax, Direct mode is employed for non-merged PU and signaled by prediction mode as it was.

# Test results

In this section, two experimental results are given. The first one is testing the combination of MRG and Skip/Direct of A124. The second one is testing MRG off case with Skip/Direct of A124. Using results of the first test, we can get the clue that Skip/Direct brings extra bit savings in top of MRG or not. Using result of the second test, we can judge which method is more appropriate for test mode (TM) between Skip/Direct and MRG.

Anchor of experimental results are provided by software ad-hog group using reference configuration [2]. MRG is enabled in the reference configuration by default and then Skip/Direct are not utilized at all.

Table 1. BD-Rate improvement by MRG off (= Skip/Direct on)

|  | Random Access HE | Random Access LC | Low delay HE | Low delay LC |
| --- | --- | --- | --- | --- |
| Class A | -0.8 | -2.4 | - | - |
| Class B | -0.7 | -3.5 | 0.0 | -2.3 |
| Class C | -0.8 | -1.8 | -0.5 | -1.1 |
| Class D | -0.7 | -2.5 | 0.2 | -0.5 |
| Class E | - | - | 0.7 | -9.8 |
| Enc time[%] | 111 | 114 | 109 | 108 |
| Dec time[%] | 102 | 101 | 102 | 101 |
| Total | -0.7 | -2.6 | 0.1 | -2.9 |

Table 1 shows BD-Rate improvements when MRG is turned off in the reference configuration. The results are copied from TE12 report [3]. In the experiment, since MRG is turned off, Skip/Direct is utilized instead. As a result, the results of this experiment demonstrate the improved coding efficiency of Skip/Direct against MRG. The improved BD-Rate is -0.7% (random access), -2.6% (random access low complexity), 0.1% (low delay) and -2.9% (low delay low complexity), respectively. This implies that Skip/Direct have higher coding efficiency than MRG in current TMuC design.

Table 2. BD-Rate improvement by combination of Skip/Direct and MRG

|  | Random Access HE | Random Access LC | Low delay HE | Low delay LC |
| --- | --- | --- | --- | --- |
| Class A | -1.3 | -2.0 | - | - |
| Class B | -1.4 | -3.5 | -0.6 | -2.8 |
| Class C | -1.2 | -1.5 | -1.0 | -1.3 |
| Class D | -0.9 | -2.1 | -0.3 | -0.9 |
| Class E | - | - | 0.1 | -10.0 |
| Enc time[%] | 113 | 114 | 110 | 100 |
| Dec time[%] | 101 | 93 | 100 | 77 |
| Total | -1.2 | -2.4 | -0.5 | -3.3 |

Table 2 shows BD-Rate improvement when Skip/Direct is added on the reference configuration. The results are obtained by enabling the macro V0_7_COMPAT and SAMSUNG_MRG_SKIP_DIRECT in TMuC-0.81 [4]. In this setting, Skip/Direct mode is employed with MRG together. As you can see, for 4 different configurations, up to -3.3% BD-Rate improvements is obtained. Maximum gain, -10.0%, is achieved with Class E under low delay low complexity configuration.

# Conclusions

In this document, the coding efficiency of TMuC with Skip/Direct was presented. The performance of TMuC was improved by maximum -3.3% (low delay low complexity) with additionally enabled Skip/Direct. It means that even if MRG is turned on, Skip/Direct can bring extra bit savings. Moreover, Skip/Direct showed bit rate saving over MRG by itself. Here, we propose to include Skip/Direct in Test Mode (TM) to improve the overall performance of TM.

# References

- “Test Model under consideration”, JCTVC-B205, Geneva, CH, 21-28 July, 2010

- ftp://ftp.kw.bbc.co.uk/hevc/tmuc-0.7-anchors

- K. Sugimoto, A. Minezawa and S. Sekiguchi, “TE12.1 cross-check report on block merging (MRG) on/off”, JCTVC-C118, Guangzhou, CN, 7-15 October, 2010

- https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tag/0.81

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).