| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C200 WG11 Number: m18238 |
| --- | --- |

| Title: | TE 12.1: Transform unit quadtree/2-level test |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool experiment result report |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Woo-Jin Han Yeongtong-Gu Suwon-City Gyeonggi-Do South Korea Digital Media & Communications R&D Center Samsung Electronics Co., Ltd. | Email: | tammy.lee@samsung.com jianle.chen@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of TE12 residual quadtree transform (RQT), which to explore the performance of RQT in the Test Model under Consideration (TMuC). Experimental result of RQT off compared to TMuC shows 1.2%, 0.5% and 0.2% bitrate savings under intra only, random access and low delay condition respectively in high efficiency setting. It shows 0.6%, -0.2% and 0.8% bitrate savings under the same conditions respectively in low complexity setting. This contribution also analyzes the performance and complexity of current TMuC transform unit design by testing several variants of it: no transform unit across partition boundary and no transform unit splitting inside a partition.

- RQT performance

## Transform unit design in TMuC

In TMuC, a picture is divided into coding tree blocks (CTB) and each CTB is recursively subdivided into prediction units to be adapted the specific characteristics of the signal. A prediction unit is further divided into partitions to be adapted to the characteristics of the prediction signal. The residual of a prediction unit is processed by DCT-based coding with variable unit sizes to be adapted to the characteristics of the residual signal. The maximum transform unit size of inter-predicted residual and intra-predicted residual is constrained by the corresponding prediction unit size and partition size respectively.

With RQT which is turned on by default, the residual of a prediction unit is split into transform units by using a nested quadtree-based partitioning. Without RQT, only a part of residual quadtree is allowed such as the size of the corresponding prediction unit or the size of the corresponding partition block depending on prediction mode.

ON

OFF

…

OFF

ON

## Experiment results

The following results are generated by switching off RQT in the four inter-coding configurations and the two intra-coding configurations specified in JCTVC-B300. The following macro setting was used to disable RQT:

#define HHI_RQT 0

#define HHI_RQT_CHROMA_CBF_MOD 0

#define HHI_RQT_INTRA 0

For the high-efficiency configurations, TMuC 0.7 was used whereas TMuC 0.7.4 was used for the low-complexity configurations since the bug related to the case when RQT is disabled was fixed at TMuC 0.7.4. (See Ticket #85 - http://hevc.kw.bbc.co.uk/trac/ticket/85)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.1 | 1.1 | -1.3 | -0.2 | -1.0 | -3.7 |
| Class B | 1.1 | 0.2 | -0.7 | 0.4 | -0.2 | -1.9 |
| Class C | 1.3 | 0.3 | -0.3 | 1.3 | 0.0 | -0.6 |
| Class D | 0.8 | 0.0 | -0.3 | 1.1 | -0.2 | -0.5 |
| Class E | 1.5 | -0.4 | -0.3 | 0.0 | -7.1 | -4.8 |
| All | 1.2 | 0.2 | -0.5 | 0.6 | -1.4 | -2.0 |
| Enc Time[%] | 39% |  |  | 31% |  |  |
| Dec Time[%] | 99% |  |  | 93% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 2.0 | -1.4 | -1.0 | -2.3 | -4.1 |
| Class B | 0.7 | 0.4 | -1.6 | -0.6 | -2.5 | -3.2 |
| Class C | 0.5 | -0.2 | -1.1 | 0.4 | -1.7 | -2.1 |
| Class D | 0.5 | 0.1 | -0.3 | 0.2 | -1.9 | -1.7 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.3 | -1.1 | -0.2 | -2.1 | -2.6 |
| Enc Time[%] | 97% |  |  | 89% |  |  |
| Dec Time[%] | 100% |  |  | 87% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 2.8 | 0.4 | 0.9 | -4.6 | -5.1 |
| Class C | 0.0 | 0.1 | -1.0 | 1.5 | -3.1 | -3.2 |
| Class D | 0.0 | 1.2 | -0.6 | 1.7 | -2.8 | -3.1 |
| Class E | 0.7 | 0.3 | -0.5 | -1.7 | -9.1 | -8.1 |
| All | 0.2 | 1.3 | -0.4 | 0.8 | -4.6 | -4.7 |
| Enc Time[%] | 95% |  |  | 85% |  |  |
| Dec Time[%] | 100% |  |  | 83% |  |  |

In intra-coding configuration, RQT shows 1.2% and 0.6% performance gain while encoding time is increased significantly under high efficiency setting and low complexity setting respectively. In inter-coding configuration, it also shows performance gain 0.5% (random access), -0.2% (random access low complexity), 0.2% (low delay) and 0.8% (low delay low complexity). However, chroma performance is degraded by 1.4 ~ 4.7% under low complexity setting.

Furthermore, it was suggested through the reflector to analyze two specific cases: (1) transform unit across partition boundary (2) transform unit splitting inside the partition boundary. To analyze RQT performance further, those two variants were tested in the following section.

# Additional results

## TU across partition and TU splitting inside partition

Residual quadtree is the nested quadtree whose root node is defined according to prediction mode. It cannot exceed the corresponding prediction unit and minimum possible leaf node size is defined by the minimum transform size supported.

Since a partition is always smaller than or equal to a prediction unit, residual quadtree root is always smaller than or equal to the corresponding prediction unit. With the quadtree level synchronizing with partition boundary as the center, residual quadtree can be separated into two parts: the above level and the below level. On the above level, which can be denoted as TU across partition, transform unit spans partition boundary while on the below level, which can be denoted as TU splitting inside partition, transform unit is split into smaller than a partition.

At Dresden, both two features were included in the TMuC design since it was suggested that TU across partition aims to remove redundancy for multiple neighboring partitions having similar signal characteristics by using large transform for better energy compaction while TU splitting inside partition aims to adapt the transform size according to the local property of the residual signal.

Prediction Unit (PU)

Transform Unit (TU)

TU larger than Partition

Level on Partition boundary

TU smaller than Partition

## Experiment results

TMuC 0.8.1 was used for the experiments with the following macro setting to maintain the compatibility to TMuC 0.7.

#define V0_7_COMPAT 1

To investigate the coding efficiency of TU across partition, the simulation has been done with disabling TU across partition feature. It was done by setting the following macro provided by HHI:

#define HHI_RQT_FORCE_SPLIT_NxN           	1

#define HHI_RQT_FORCE_SPLIT_RECT          	1

#define HHI_RQT_FORCE_SPLIT_ASYM      	1

For testing the coding efficiency of TU splitting inside partition, the following macro was set:

#define HHI_RQT_DISABLE_SUB              	1

| Rand access | Disabling TU across partition |  |  | Disabling TU splitting inside partition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.1 | 0.0 | 1.0 | 0.5 | 0.4 |
| Class B | 0.4 | 0.2 | 0.0 | 1.2 | 0.8 | 0.8 |
| Class C | 0.4 | 0.1 | 0.2 | 1.0 | 0.5 | 0.7 |
| Class D | 0.4 | 0.2 | 0.3 | 1.0 | 0.3 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.1 | 0.1 | 1.1 | 0.6 | 0.6 |
| Enc Time[%] | 99% |  |  | 85% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| Rand access LoCo | Disabling TU across partition |  |  | Disabling TU splitting inside partition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.5 | 2.5 | 2.6 | 0.0 | -2.0 | -2.2 |
| Class B | 1.7 | 1.8 | 1.8 | 0.4 | -2.9 | -3.4 |
| Class C | 2.0 | 2.3 | 2.5 | 1.4 | -1.5 | -1.6 |
| Class D | 1.7 | 2.1 | 2.0 | 1.5 | -2.0 | -1.9 |
| Class E |  |  |  |  |  |  |
| All | 1.9 | 2.1 | 2.1 | 0.9 | -2.2 | -2.4 |
| Enc Time[%] | 96% |  |  | 77% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |

| Low delay | Disabling TU across partition |  |  | Disabling TU splitting inside partition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 0.1 | 0.2 | 0.9 | 1.4 | 1.8 |
| Class C | 0.6 | 0.0 | 0.1 | 0.9 | 0.5 | 0.6 |
| Class D | 0.5 | -0.3 | -0.2 | 0.8 | 0.6 | 0.5 |
| Class E | 0.9 | 0.9 | 0.4 | 0.3 | -0.1 | -0.3 |
| All | 0.7 | 0.1 | 0.1 | 0.8 | 0.7 | 0.8 |
| Enc Time[%] | 99% |  |  | 86% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| Low delay LoCo | Disabling TU across partition |  |  | Disabling TU splitting inside partition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.6 | 1.4 | 1.6 | 2.5 | -4.2 | -5.0 |
| Class C | 1.4 | 1.4 | 1.7 | 3.3 | -2.9 | -3.0 |
| Class D | 1.0 | 1.9 | 1.8 | 4.1 | -3.7 | -3.9 |
| Class E | 4.1 | 4.1 | 3.9 | -0.5 | -7.0 | -6.2 |
| All | 1.9 | 2.0 | 2.1 | 2.5 | -4.3 | -4.5 |
| Enc Time[%] | 91% |  |  | 75% |  |  |
| Dec Time[%] | 80% |  |  | 80% |  |  |

# Conclusion

According to the above results, two features, TU across partition and TU splitting inside partition, show similar benefits. In addition, it seems that the benefits from each tool are not overlapped each other since they provide different benefits according to the test configurations and the sequences. It means that both parts of residual quadtree do not conflict each other in coding efficiency and it is worth to keep both properties in the current TMuC design since RQT off (2-level design) is also a subset of RQT design and we believe that its flexibility is important to the first stage of the HEVC test model.

We also recommend considering the encoder complexity reduction method in [5], which restricts the maximum number of quadtree depth according to the unit size since the encoder complexity of the full-depth residual quadtree is rather high.

# References

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

- JCT-VC, “TE9: Simulation results for various max. number of transform quadtree depth”, JCTVC-C198, JCT-VC Meeting, Guangzhou, October 2010

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).