| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C201 WG11 Number: m18239 |
| --- | --- |

| Title: | TE12.2: Experimental results of SIFO (DIF) / DCT-IF chroma extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool test report |  |  |
| Author(s) or Contact(s): | Elena Alshina Jianle Chen Nikolay Shlyakhov Alexander Alshin Woo-Jin Han | Email: | elena_a.alshina@samsung.com jianle.chen@samsung.com n.shlyakhov@samsung.com alexander_b.alshin@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the results of chroma DCT-IF extension within the scope of tool experiment 12 evaluating TMuC Tools Error: Reference source not found. For the test, DCT-IF 6 tap is used to replace existing default chroma interpolation (bilinear filter for high efficiency and DIF for low complexity). The interpolation filter for luma is remained as it is for both high efficiency and low delay configurations. That is SIFO for high efficiency and DIF for low complexity respectively. Experiments results show that, in high efficiency test configuration, the average BD-rate reduction of Y, Cb, Cr components are respectively 0.2%, 5.5% and 6.6% for random access and 0.5%, 2.5% and 2.9% for low delay. In low complexity test configuration, the average BD-rate reduction of Y, Cb, Cr components are respectively 0.4, 12.7 and 14.1% for random access and 1.4, 12.4 and 15.3% for low delay. Encoding and decoding time is similar to that of anchor software and deviations exists from measurement inaccuracies.

- Introduction

In current TMuC, various chroma interpolation filters are used according to the luma interpolation filter type:

- InterFilterType = 0 (DCT-IF), bi-linear for chroma interpolation filter

- InterFilterType = 1 (MOMS4), MOMS4 for chroma interpolation filter

- InterFilterType = 2 (MOMS6), MOMS6 for chroma interpolation filter

- InterFilterType = 3 (DIF), DIF chroma interpolation filter

- InterFilterType = 4 (SIFO), bi-linear for chroma interpolation filter

Since the use of longer tap interpolation filter was reported to be effective in several proposals, the 6-tap DCT-IF filter for the chroma interpolation filter was tested.

- Simulation environment

## Software

The software used to run the simulations is TMuC 0.8.1 [2]. The coding conditions used for the simulations are specified in [2]. The code change for this test is mentioned as following

The following macro has to be enabled in TypeDef.h:

#define V0_7_COMPAT 1

#define SAMSUNG_CHROMA_IF_EXT  1

Minor code change was done to enable DCT-IF 6 tap for chroma interpolation when DIF is used for luma in low complexity test configuration. Same filter coefficient of DCT-IF 6-tap filter for luma was utilized without any change of filter coefficients.

For the luma interpolation filter, SIFO (high-efficiency configuration) and DIF (low-complexity configuration) were used according to the TMuC default configuration.

- Experimental results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. The values are averaged over all sequences from one class. Furthermore relative encoding and decoding times in percentage of the anchor times are also given.

The results are presented in three tables: Table 2-1, Table 2-2. Each table represents a test for a constraint set using the high efficiency (HE) and low complexity (LC) settings.

Table 2-1 Average BD-rates and relative encoding/decoding runtimes for random access constraint set.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -2.4 | -3.2 | -0.3 | -8.1 | -8.7 |
| Class B | -0.1 | -5.0 | -6.7 | -0.3 | -12.7 | -14.9 |
| Class C | -0.2 | -5.8 | -5.6 | -0.5 | -12.6 | -12.3 |
| Class D | -0.3 | -7.3 | -9.2 | -0.5 | -15.0 | -17.6 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -5.5 | -6.6 | -0.4 | -12.7 | -14.1 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 103% |  |  | 97% |  |  |

Table 2-2 Average BD-rates and relative encoding/decoding runtimes for low delay constraint set.

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -1.8 | -2.0 | -1.3 | -13.5 | -16.5 |
| Class C | -0.5 | -3.5 | -3.7 | -1.6 | -13.3 | -12.4 |
| Class D | -0.9 | -3.2 | -4.5 | -1.9 | -14.6 | -16.8 |
| Class E | -0.1 | -1.3 | -1.3 | -0.5 | -6.5 | -15.1 |
| All | -0.5 | -2.5 | -2.9 | -1.4 | -12.4 | -15.3 |
| Enc Time[%] | 100% |  |  | 93% |  |  |
| Dec Time[%] | 102% |  |  | 83% |  |  |

# Conclusions

Based on the provided results, the 6-tap DCT-IF shows a good coding gain compared to the current TMuC default configuration. Although we think that the decoder complexity measurement seems not very accurate due to the heterogeneous test machine setup, it is clear that the encoder and decoder running time are not increased so much. We recommend considering to use 6-tap DCT-IF as the chroma interpolation filter instead of the current TMuC chroma interpolation filters.

# References

- JCT-VC, “Test model under consideration”, Doc. JCTVC-A205, April 2010

- JCT-VC, “Test Model under Consideration Software 0.81”,
 http://hevc.kw.bbc.co.uk/trac/browser/tags/0.8.1

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).