| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C202 WG11 Number: m18240 |
| --- | --- |

| Title: | TE12.4: Experimental results of MDDT and ROT by Samsung and Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool test report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin Woo-Jin Han R. Joshi M. Coban | Email: | elena_a.alshina@samsung.com alexander_b.alshin@samsung.com wjhan.han@samsung.com rajanj@qualcomm.com mcoban@qualcomm.com |
| Source: | Samsung Electronics Co., Ltd. and Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides the simulation results when both MDDT and ROT are turned off against the TMuC anchor results to verify the efficiency of the current unified design in TMuC. Additionally, new unified design implemented in TMuC 0.8.1 is tested and compared to the previous design.

- Introduction

According to the Test model under consideration (TMuC) [3], MDDT and ROT are unified according to the transform sizes. MDDT is applied when the transform size is 4x4 or 8x8; for other transform sizes ROT is applied. This contribution provides the simulation result of the unified design in the current TMuC. Additionally, new unified design implemented in TMuC 0.8.1 is tested and compared to the previous design.

For the TE12 experiments, MDDT was turned off by turning off QC_MDDT macro and ROT was turned off by the configuration parameter.

For the high-efficiency configurations, TMuC 0.7.1 was used but TMuC 0.7.4 was used for the low-complexity configurations since the bug related to this case when MDDT is turned off was fixed in TMuC 0.7.4. It was fully discussed and cross-checkers agreed to use this version.

- Test results

MDDT = 0, ROT = 0 against TMuC anchor

|  | Intra |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  |  | U BD-rate | V BD-rate |
| Class A | 2.5 |  | 1.0 | 0.5 | 2.6 |  |  | 0.6 | 0.3 |
| Class B | 1.8 |  | 1.2 | 0.8 | 3.2 |  |  | 2.3 | 2.1 |
| Class C | 2.0 |  | 1.3 | 1.2 | 2.4 |  |  | 1.5 | 1.6 |
| Class D | 1.9 |  | 1.2 | 1.1 | 2.3 |  |  | 1.3 | 1.3 |
| Class E | 2.7 |  | 0.2 | 0.5 | 3.2 |  |  | 1.2 | 1.4 |
| All | 2.1 |  | 1.0 | 0.9 | 2.7 |  |  | 1.5 | 1.5 |
| Enc Time[%] | 55% |  |  |  | 53% |  |  |  |  |
| Dec Time[%] | 45% |  |  |  | 27% |  |  |  |  |
|  | Random access |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | 1.1 |  | 0.1 | -0.2 | 1.0 |  | 0.2 |  | -0.2 |
| Class B | 0.8 |  | 0.2 | 0.0 | 1.2 |  | 0.9 |  | 1.0 |
| Class C | 0.8 |  | 0.3 | 0.4 | 0.8 |  | 0.4 |  | 0.4 |
| Class D | 0.7 |  | 0.2 | 0.1 | 0.7 |  | 0.2 |  | 0.2 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | 0.8 |  | 0.2 | 0.1 | 0.9 |  | 0.5 |  | 0.5 |
| Enc Time[%] | 97% |  |  |  | 93% |  |  |  |  |
| Dec Time[%] | 34% |  |  |  | 17% |  |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.2 |  | 0.1 | -0.1 | 0.4 |  | 0.1 |  | 0.1 |
| Class C | 0.2 |  | 0.2 | 0.4 | 0.3 |  | 0.2 |  | 0.3 |
| Class D | 0.2 |  | -0.1 | 0.3 | 0.2 |  | 0.0 |  | 0.1 |
| Class E | 0.4 |  | 0.1 | 0.0 | 0.4 |  | -1.0 |  | 0.7 |
| All | 0.3 |  | 0.1 | 0.2 | 0.4 |  | -0.1 |  | 0.3 |
| Enc Time[%] | 98% |  |  |  | 90% |  |  |  |  |
| Dec Time[%] | 34% |  |  |  | 15% |  |  |  |  |

According to the results, current unified design of MDDT and ROT provides 2.1% gain in high-efficiency Intra and 2.7% gain in low-complexity intra configurations. Gains in the inter configurations are around 0.8-0.9% in random access scenario and 0.3-0.4% in low-delay scenario. Main reason of the smaller gain in low-delay scenario seems to come from the lack of intra modes in this scenario.

# Additional test results

Rather than unifying two coding tools according to the transform size, it is proposed to unify two coding tools by allowing the use of combination of MDDT and ROT for 4x4 and 8x8 transform sizes. In this case, MDDT is used when ROT index is 0. This variant has been implemented in TMuC 0.8.1 by the QC_MDDT_ROT_UNIFIED macro in TypeDef.h (default: off). Only several lines of change were made since it does not change any of the coding tools, but only changes the way the two coding tools are combined.

Additional test results were obtained using the following macro setting:

#define V0_7_COMPAT 1

#if QC_MDDT

#define QC_MDDT_ROT_UNIFIED       1     ///< better unification of MDDT and ROT

#endif

QC_MDDT_ROT_UNIFIED = 1 against TMuC anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | 0.4 | 0.6 | -0.8 | 0.0 | 0.1 |
| Class B | -0.9 | 0.5 | 0.7 | -0.9 | -0.2 | -0.2 |
| Class C | -1.5 | -0.2 | -0.4 | -1.6 | -1.4 | -1.5 |
| Class D | -1.4 | -0.1 | -0.4 | -1.4 | -1.4 | -1.8 |
| Class E | -1.2 | 1.5 | 1.1 | -1.1 | 1.1 | 0.3 |
| All | -1.2 | 0.4 | 0.3 | -1.2 | -0.5 | -0.7 |
| Enc Time[%] | 141% |  |  | 137% |  |  |
| Dec Time[%] | 97% |  |  | 89% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | 0.2 | 0.3 | -0.4 | -0.1 | 0.0 |
| Class B | -0.4 | 0.0 | 0.2 | -0.4 | -0.3 | -0.4 |
| Class C | -0.7 | -0.6 | -0.6 | -0.7 | -0.7 | -0.8 |
| Class D | -0.6 | -0.4 | -0.2 | -0.5 | -0.8 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.2 | -0.1 | -0.5 | -0.5 | -0.5 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 94% |  |  | 87% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | 0.2 | -0.1 | 0.0 | -0.1 |
| Class C | -0.2 | 0.1 | 0.1 | -0.1 | 0.0 | 0.1 |
| Class D | -0.1 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | -0.5 | 0.6 | 0.3 | -0.2 | -0.1 | 1.1 |
| All | -0.2 | 0.2 | 0.2 | -0.1 | 0.0 | 0.2 |
| Enc Time[%] | 92% |  |  | 94% |  |  |
| Dec Time[%] | 86% |  |  | 79% |  |  |

Performance Summary of two unified designs (against MDDT=0 and ROT=0)

|  | TMuC unified |  |  | Alternative unified design |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| HE Intra | -2.0 | -1.0 | -0.9 | -3.2 | -0.6 | -0.6 |
| HE Random access | -0.8 | -0.2 | 0.1 | -1.3 | -0.5 | -0.2 |
| HE Low delay | -0.3 | -0.1 | -0.2 | -0.5 | 0.2 | 0.1 |
| LoCo Intra | -2.7 | -1.5 | -1.5 | -3.8 | -2.0 | -2.2 |
| LoCo Random access | -0.9 | -0.5 | -0.5 | -1.4 | -1.0 | -1.0 |
| LoCo Low delay | -0.4 | 0.1 | -0.3 | -0.5 | 0.1 | -0.1 |

As shown in the table, the alternative unified design (QC_MDDT_ROT_UNIFIED=1) provides clearly better results compared to the current unified design in TMuC and now it has about 3.5% gain in intra configuration compared to MDDT=0 and ROT=0 case while providing good gain in random access scenario. Although encoder complexity in intra configuration is increased, it can be observed that the decoder complexity becomes smaller with the alternative unified design.

# Conclusions

Based on the results, we would like to recommend the alternative unified design for the first version of the HEVC test-model.

# References

- M.Karczewicz, P.Chen, R.Joshi, X.Wang, W-J.Chien, R.Panchal,  “Video coding technology proposal by Qualcomm Inc”, JCTVC-A121, Dresden,  Germany, April 2010.

- Ken McCann, Woo-Jin Han and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- JCT-VC, “Test model under consideration”, Doc. JCTVC-A205, April 2010.

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, Doc. JCTVC-B312, July 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).