| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C205 WG11 Number: m18243 |
| --- | --- |

| Title: | Low-complexity adaptive coefficients scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen | Email: | v.seregin@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes new scanning method for transform coefficients. The best scanning mode for every Transform Unit (TU) is defined explicitly based on RD estimation and corresponding mode index is signalled to the decoder side. The proposed technique was implemented on top of TMuC 0.7 and comparison was made to existing methods in TMuC 0.7 version. Performance is evaluated based on common test conditions specified in JCTVC-B300 [1]. Proposed method provides 0.5%, 0.3% and 0.5% improvements in high efficiency intra-only, random access, and low-delay configurations, respectively. In low complexity configurations proposed method maintains similar performance as TMuC.

# Introduction

Current TMuC contains three different transform coefficient scanning methods switching according to the CU mode, TU size and symbol mode of entropy coding method [2]:

- Statistic-adaptive scanning method coupled with MDDT transform.

- Context-adaptive scanning method as part of HHI_TRANSFORM_CODING.

- Conventional zigzag scanning method is exploited for inter CU in VLC mode.

In more details, for default TMuC configuration, if symbol mode differs from zero and HHI_TRANSFORM_CODING macro is enabled statistic-adaptive scanning is used for TU sizes smaller than 16x16 in intra CU and context-adaptive scanning is exploited for the TU sizes greater than 8x8 in intra CU and for all sizes in inter CU. If symbol mode is equal to zero, representing VLC mode, statistic-adaptive scanning is applied for all TU irrelevant to its size in intra. From this, it can be concluded that TMuC does not have general scanning method for every configuration.

This proposal is addressing this issue and provides an alternative scanning design combining the explicit signalling method proposed in our call-for-proposal response [3] and the mode-dependent feature from statistic-adaptive scanning method. Major goal is to apply the consistent scanning method for all cases while maintaining the coding efficiency of the current TMuC design.

# Adaptive transform coefficients scanning (ACS) method with explicit signalling

In this proposal Adaptive Coefficient Scanning (ACS) method is introduced, being part of A124 [3], with additional improvements mainly directed to the overhead reduction. Since transform coefficient may have sparse distribution, additional explicitly signalled scanning patterns can contract the significance map hence significantly improve coding efficiency.

Currently, ACS method consists of three scanning patterns: conventional zigzag, horizontal and vertical patterns (Fig. 1), and ACS index of best scanning pattern chosen by RD is signalled TU-wise to the decoder side.

In order to indicate supplemented scanning modes, overhead is signaled in case non-last DC coefficient exists, if there is only one DC coefficients all scanning patterns provide same result and overhead is not necessary. Overhead organized as following: 0 is sent to signal zigzag scanning pattern, or 1 following by 0 are sent to specify horizontal mode, or 1 following by 1 are put for vertical mode.

Fig. 1. ACS patterns for 4x4 transform unit.

When the horizontal or vertical scanning methods are chosen as the best scanning method, the mapping process from the coefficient array indices to the scanning order indices is not necessary. Thus, the complexity for the coefficient indexing can be reduced and also the memory bandwidth to access the scanning order mapping table can be saved.

On top of the ACS design introduced in JCTVC-A124, mode-dependency feature is considered additionally since we found that it is useful to reduce the overhead of ACS indices. More specifically, additional context modelling based on the intra modes for intra CUs is introduced taking into account the correlation of ACS indices. Consideration of this correlation is accomplished by classification of intra modes in several classes.

In current implementation, all possible intra modes are split into three classes: DC mode, horizontal and vertical classes. Intra modes of unified intra prediction (ADI and Angular) with up to 34 directions can be mapped by using IntraDirToContextIdx buffer, and one of possible classifications is provided below, where input is intra mode, and output is position in ACS context model.

IntraDirToContextIdx[34] = { 2, 1, 0, 2, 2, 2, 2, 1, 1, 1, 2, 2, 2, 2, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1 };

For each class of intra modes, ACS index is coded in separated position of context model specifying by class index, from 0 to 2 in this example. Additionally, ACS index coding for inter CUs are also separated from intra CUs.

# Experimental results of ACS and complexity analysis

For comparison of various scanning approaches, TMuC 0.7 with default configuration plays role of anchor, and every scanning configuration is evaluated against this anchor. For this purpose, all scanning methods implemented in the TMuC 0.7 were disabled and the following software configurations are tested:

- TMuC 0.7 with zigzag scanning

- TMuC 0.7 with ACS, where every scanning is replaced by ACS

Following table represents total average BD-rate saving for each testing configuration. ACS has improvements comparing with default TMuC by 0.5%, 0.3% and 0.5% in high-efficiency intra-only, random-access and low delay configurations, respectively. In low complexity configurations ACS shows similar performance compared to the TMuC.

Table 1. Average BD-rate saving (against TMuC anchor)

|  | TMuC 0.7 with zigzag |  |  | TMuC 0.7 with ACS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| HE Intra | 0.9 | 1.3 | 1.2 | -0.5 | 0.1 | 0.1 |
| HE Random access | 0.4 | 0.5 | 0.4 | -0.3 | 0.1 | 0.0 |
| HE Low delay | 0.2 | 0.5 | 0.6 | -0.5 | 0.4 | 0.6 |
| LoCo Intra | 0.8 | 0.6 | 0.6 | 0.6 | 0.2 | 0.2 |
| LoCo Random access | 0.4 | 0.3 | 0.2 | 0.2 | 0.3 | 0.1 |
| LoCo Low delay | 0.2 | -0.1 | 0.2 | -0.5 | -0.1 | -0.3 |

In the next table relative to the anchor encoding and decoding running time are summarised for the same configurations mentioned above.

Table 2. Relative to the anchor encoding and decoding running time

|  | TMuC 0.7 with zigzag |  | TMuC 0.7 with ACS |  |
| --- | --- | --- | --- | --- |
| Time | Enc | Dec | Enc | Dec |
| HE Intra | 89% | 44% | 141% | 46% |
| HE Random access | 94% | 32% | 111% | 33% |
| HE Low delay | 96% | 32% | 105% | 31% |
| LoCo Intra | 92% | 27% | 178% | 28% |
| LoCo Random access | 94% | 16% | 130% | 17% |
| LoCo Low delay | 88% | 14% | 117% | 15% |

Besides the shorten significant map sequence thus reduced parsing complexity provided by ACS, horizontal and vertical scanning patterns hit-ratio is also meaningful for complexity reduction due to buffer access saving.

Table 3. Hit-ratio of zigzag scanning pattern in ACS

|  | Zigzag hit-ratio |
| --- | --- |
| HE Intra | 49% |
| HE Random access | 58% |
| HE Low delay | 57% |
| LoCo Intra | 62% |
| LoCo Random access | 64% |
| LoCo Low delay | 62% |
| Total average | 58% |

From the result presented above, it is seen that ACS decoder has similar complexity to the software configuration with zigzag scanning only and saves 42% of buffer access due to horizontal and vertical scanning patterns.

Also, encoder for ACS can be further optimised reducing complexity. For example, before ACS index searching distribution of transform coefficients, considering the magnitude of coefficient and QP, can be performed by counting the coefficient above and below diagonal of TU. If there is domination in above or below diagonal part then searching can be reduced to zigzag and horizontal pattern or zigzag and vertical scanning correspondingly. Also, one of the reasons of relatively to the high efficiency configuration complexity of the encoder in low complexity case is the non-optimal RDOQ implementation, resulting in complexity increasing of ACS search as well.

# Conclusions

In this contribution, the simulation results of using one alternative scanning method are provided. For the alternative scanning method, the explicit signalling of the scanning index proposed in [3] are further improved to be combined with the mode-dependent property of the current TMuC. From the results, it was observed that the alternative scanning method can provide the similar performance to the TMuC while maintaining the decoder complexity as low as zigzag scanning method with less buffer access.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010

- JCT-VC, “Test Model under Consideration”, 2nd JCT-VC Meeting Document JCTVC-B205, July 2010.

- K. McCann et al. “Samsung’s Response to the Call for Proposals on Video Compression Technology”, 1st JCT-VC Meeting Document JCTVC-A124, April 2010.

# Patent rights declaration(s)

Samsung Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).