| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C206 |
| --- | --- |

| Title: | Chroma intra prediction by reconstructed luma samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin | Tel: Email: | +82-31-279-8826 jianle.chen@samsung.com v.seregin@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution demonstrates integerization method of the algorithm presented in JCTVC-B021 [1] where chroma samples are predicted from collocated reconstructed luma samples by linear model relationship. Only additions/multiplications operation is involved to calculate model parameter instead of division in the proposed approach. Experiments show that the average BD-rate reduction of Y, Cb, Cr components are 1.3%, 6.8% and 5.6% for intra coding condition and 0.7%, 6.7% and 4.6% for random access condition. Analysis shows that the complexity of proposed method is similar and even smaller comparing to method used in H.264/AVC.

# Introduction

A new intra chroma prediction method using inter-channel correlation was proposed in previous JCT-VC meeting [1] and ICIP 2009 [3]. The chroma values are predicted from collocated reconstructed luma values by linear model relationship. The model parameters are derived from causal reconstructed samples around the current block. Since parameters calculation of the new method involves float point division which is expensive to be implemented in both hardware and software, the integerization method is designed in this contribution in which floating point and division operations are instead by additions/multiplications.

# Proposed method

## Basic algorithm

Same to the method in [1] and [3], the proposed method utilizes the linear relationship to modelize the correlation of luma signal and chroma signal. The chroma values are predicted from reconstructed luma values of collocated block as follows.

(1),

where indicates the prediction of chroma samples in a block and indicates the reconstructed luma samples in the block. Parameters  and are derived from causal reconstructed samples around the current block.

As mentioned in [1], luma and chroma components have different sampling ratios in YUV420 sampling. The sampling ratio of chroma components is half of that of luma component and has 0.5 pixel phase difference in vertical direction. Reconstructed luma needs downsampling in vertical direction and subsample in horizontal direction to match size of chroma singal, as follows.

(2),

In this contribution, linear least square solution between causal reconstructed data of downsampled luma component and chroma component is used to derive model parameters  and .

(3),

(4),

whereand indicate reconstructed chroma samples and downsampled luma samples around the target block, indicates total samples number of neighboring data. As shown in fig.1, only left and above causal samples marked as gray circles are involved in the calculation to keep total samples number as power of 2. For a target N×N chroma block, when both left and above causal samples are available, total involved samples number is ; when only left or above causal samples are available, total involved samples number is .

Fig. 1 Locations of the samples used for the derivation of α and β

## Integer implementation

Float point operation is necessary in equation (3) to calculate linear model parameters  to keep high data accuracy. And float point multiplication is involved in equation (1) when is represented by float point value. In this section, the integer implementation of this algorithm is designed.

In the proposed method, fractional part of parameter is quantized with bits data accuracy. Parameter  value is represented by an up-scaled and rounded integer value  and . Then linear model of equation (1) is changed to.

(5)

Where  is rounding value of float point and can be calculated as follows.

(6)

This contribution proposes to replace division operation of equation (6) by table lookup and multiplication.  is firstly de-scaled to reduce the table size.  is also de-scaled to avoid product overflow.

Then, in  it is kept only most significant bits defined by  value and others bits are put to zero. The approximate value can be calculated as

Where  means rounding operation and can be calculated as

Where  means bit depth of value .

Same operation is done for .

Taking into account quantized representation of  and  formula (6) can be re-written as following.

Where  is represented as lookup table with length of to avoid the division.

In the simulation, the constant parameters are set as:

- equals to 13, which value is tradeoff between data accuracy and computational cost.

- equals to 6, results in lookup table size as 64, table size can be further reduced to 32 by up-scaling  when  (e.g. ).

- equals to 15, results in 16 bits data representation of table elements.

- is set as 15, to avoid product overflow and keep 16 bits multiplication

In final,  is clipped to , to remain 16 bits multiplication in equation (5). With this clipping, the actual  value is limited to  when  equals to 13, which is useful to prevent the error amplification.

With calculated parameter, parameteris calculated as follow.

The division of above equation can be simply replaced by shift since value I is power of 2,

# Simulation results and complexity analysis

## Integration to HEVC TMuC0.7 software

In current TMuC0.7 software, the Y component can have up to 34 intra prediction directions [4], the U and V component share the same intra prediction mode, and have only 5 possible intra prediction modes shown in tab.1: : DC, vertical, horizontal, diagonal down-right and luma angle mode. This contribution substitutes diagonal down-right with proposed LM mode. The mode order was changed as in Tab.1 since proposed LM mode shows best prediction efficiency.

To have the consistent comparison with H.264/AVC chroma prediction method, plane mode is tested in this contribution. For this test, plane mode replaces diagonal down-right in tab.1.

To identify the coding efficiency of LM mode and plane mode, the test of 4 modes combination (removing diagonal down-right mode) is also conducted in this contribution.

Tab.1 Intra chroma prediction modes of TMuC0.7

| mode num | TMuC 0.7 | Proposed method | Plane replacing down-right | Removing down-right |
| --- | --- | --- | --- | --- |
| 0 | Vertical | LM mode | Vertical | LM mode |
| 1 | Horizontal | Vertical | Horizontal | Vertical |
| 2 | DC | Horizontal | DC | Horizontal |
| 3 | Down-right | DC | Plane | Luma angle mode |
| 4 | Luma angle mode | Luma angle mode | Luma angle mode |  |

## Experimental results

Simulations are conducted with 6 test configurations specified for TMuC 0.7 in JCTVC-B300.

Experiments show that the average BD-rate reduction of Y, Cb, Cr components are 1.3%, 6.8% and 5.6% for intra coding condition and 0.7%, 6.7% and 4.6% for random access condition for high efficiency configuration.

Tab.1 average BD-rate saving and encoding/decoding time

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -7.2 | -2.9 | -0.9 | -7.3 | -3.1 |
| Class B | -1.3 | -8.1 | -4.5 | -1.0 | -8.1 | -3.7 |
| Class C | -1.8 | -7.9 | -8.5 | -1.5 | -6.6 | -7.5 |
| Class D | -1.2 | -5.6 | -5.5 | -1.1 | -5.2 | -4.6 |
| Class E | -0.7 | -4.8 | -5.6 | -0.4 | -3.6 | -4.8 |
| All | -1.3 | -6.8 | -5.6 | -1.0 | -6.3 | -4.8 |
| Enc Time[%] | 93% |  |  | 97% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -7.3 | -3.2 | -0.5 | -7.3 | -1.7 |
| Class B | -0.7 | -8.6 | -4.2 | -0.7 | -8.6 | -2.0 |
| Class C | -0.9 | -6.4 | -6.4 | -0.9 | -6.4 | -4.9 |
| Class D | -0.6 | -4.2 | -3.9 | -0.6 | -4.2 | -2.4 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -6.7 | -4.6 | -0.7 | -6.7 | -2.8 |
| Enc Time[%] | 92% |  |  | 95% |  |  |
| Dec Time[%] | 87% |  |  | 88% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -2.4 | -1.5 | -0.2 | -2.4 | -0.9 |
| Class C | -0.5 | -2.8 | -2.8 | -0.5 | -2.8 | -1.7 |
| Class D | -0.2 | -1.8 | -1.3 | -0.2 | -1.8 | -1.1 |
| Class E | 0.0 | -3.2 | -5.7 | 0.0 | -3.2 | -1.9 |
| All | -0.2 | -2.5 | -2.6 | -0.2 | -2.5 | -1.3 |
| Enc Time[%] | 96% |  |  | 94% |  |  |
| Dec Time[%] | 92% |  |  | 80% |  |  |

To compare results with plane mode and just 4 mode combination, 2 seconds duration is tested instead of 10 seconds frames to reduce running time.

Tab.2 shows average BD-rate saving for all test sequence of each method against the default TMuC0.7 software for all testing configurations. In intra test configuration, the average BD-rate saving of Y, Cb, Cr components are 1.3%, 6.5% and 5.5%. In random access configuration the average BD-rate saving of Y, Cb, Cr components are 0.7%, 6.2% and 4.1%. In the same condition with low complexity setting, the average BD-rate saving of Y, Cb, Cr components 1.0%, 6.0% and 4.7% for all intra configuration and 0.5%, 4.7% and 2.7% for all intra configuration.

Tab.2 also shows there is marginal affect when replace TMuC 0.7 chroma dow-right mode with plane mode, and removing chroma mode 3 shows very small loss.

Tab.3 shows BD-rate saving for each class in intra test configurations. It shows that consistent bit saving is obtained by proposed method for sequences with different resolution.

Tab.4 shows the relative encoding and decoding time to default TMuC0.7 software. It shows the proposed prediction method has the similar encoding and decoding time with existing method in all test configurations.

Tab.2 average BD-rate saving

| Test configuration | Proposed method |  |  | Plane replacing down-right |  |  | Removing down-right |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| HE_Intra | -1.3 | -6.6 | -5.5 | 0.1 | 0.1 | 0.1 | 0.1 | 0.4 | 0.4 |
| HE_Random_access | -0.7 | -6.2 | -4.1 | 0.0 | 0.2 | 0.2 | 0.1 | 0.4 | 0.6 |
| HE_Low_delay | -0.2 | -3.5 | -3.6 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| LoCo_Intra | -1.0 | -6.0 | -4.7 | 0.0 | 0.1 | 0.2 | 0.1 | 0.3 | 0.3 |
| LoCo_Random_access | -0.5 | -4.7 | -2.7 | 0.0 | 0.2 | 0.1 | 0.0 | 0.5 | 0.2 |
| LoCo_low_delay | -0.1 | -2.3 | -1.9 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 |

Tab.4 encoding and decoding time

| Test configuration | Proposed method |  | Plane replacing down-right |  | Removing down-right |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Running time |  |  |  |  |  |
|  | Enc | Dec | Enc | Dec | Enc | Dec |
| HE_Intra | 102% | 102% | 104% | 106% | 102% | 103% |
| HE_Random_access | 101% | 101% | 102% | 103% | 99% | 99% |
| HE_Low_delay | 97% | 94% | 96% | 93% | 98% | 96% |
| LoCo_Intra | 98% | 100% | 98% | 98% | 97% | 98% |
| LoCo_Random_access | 104% | 106% | 105% | 109% | 105% | 109% |
| LoCo_low_delay | 101% | 100% | 100% | 99% | 100% | 99% |

Tab.5 and Tab.6 shows the computational complexity comparison between proposed LM mode and plane mode. The total operation count of proposed LM mode is smaller than that of plane mode. And its complexity is expected to be similar to unified intra prediction method in HEVC TMuC.

Tab.5 computational complexity analysis of proposed LM mode

| Complexity of N×N chroma block | Proposed LM mode |  |  |
| --- | --- | --- | --- |
|  | Mul | Add | Shift |
| Calculation of and for each block | ~=3·N +6 | ~=6·N +4 | ~=4 |
| Down-sample for each pixel | 0 | ~=0.5 | ~=0.5 |
| Prediction for each pixel | 1 | 1 | 1 |
| Total operation for each pixel | 1+3/ N +6/ N2 | 1.5+6/ N +4/ N2 | 1.5+4/ N2 |

Tab.6 computational complexity analysis of plane mode

| Complexity of N×N chroma block | Plane mode |  |  |
| --- | --- | --- | --- |
|  | Mul | Add | Shift |
| Calculation of A, B and C for each block | ~=N +4 | ~=2·N +4 | ~=3 |
| Prediction for each pixel | 1+2/ N | 1+4/ N | 1 |
| Total operation for each pixel | 1+3/ N +4/ N2 | 1+6/ N +4/ N2 | 1+3/ N2 |

# Conclusions

This contribution proposes integerization method of new chroma intra prediction mode. Chroma samples are predicted from collocated reconstructed luma samples by linear model relationship. Only additions/multiplications operation is involved in the calculation and its overall complexity is comparable and even smaller than existing method in HEVC TMuC. Experimental results show that the average BD-rate reductions of Y, Cb, Cr components are 1.3%, 6.5% and 5.5% in Intra test configuration and the similar level of efficiency improvements of the chroma components are observed in other test configurations.

The proposed method can be easily extended to other YUV sampling format. For YUV 4:2:2 or YUV 4:4:4 sequences, the overall gains are expected to be more remarkable.

Since proposed method achieves solid coding gain without any additional complexity, we recommend adopting it in HEVC test model and setting up Core Experiment for chroma intra prediction topic.

# References

- J.S. Kim, S.W. Park, et al, “New intra chroma prediction using inter-channel correlation”, ITU-T/ISO JCT-VC, JCTVC-B021, Geneva, Switzerland, July 2010

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010

- S.H. Lee, N.I. Cho, “Intra Prediction Method Based on the Linear Relationship between the Channels for YUV 4:2:0 Intra Coding”, IEEE International Conference of Image Processing 2009

- J.-H. Min, S. Lee, I.-K. Kim, W.-J. Han, J. Lainema, and K. Ugur, “Unification of the Directional Intra Prediction Methods in TMuC,” JCTVC-B100, Geneva, Switzerland, Jul 2010.

- Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).