| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C207 WG11 Number: m18245 |
| --- | --- |

| Title: | Encoder improvement of unified intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Junghye Min Jianle Chen | Email: | yj1003.piao@samsung.com jh643.min @samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes a new encoding scheme for unified directional intra prediction which was introduced in JCTVC-B100. Experimental results shows unified intra prediction with new encoding scheme provides on average 2.57% bit savings compared to original ADI in TMuC 0.7 where current default encoding scheme of unified intra has 1.75% bit saving in intra high efficiency test.

# Introduction

In current TMuC, unified intra (JCTVC-B100) is the main intra coding tool which provides up to 34 directional prediction modes for different PUs. With the PU size of 4x4, 8x8, 16x16, 32x32 and 64x64, there are 17, 34, 34, 34 and 5 prediction modes available respectively. However, in current TMuC 0.7 software, only parts of directions are compared with RD optimization due to the high computational complexity while the others have less priority than those directions by comparing only with hadamard transform without considering other factors considered in RD. So the performance of unified intra still can be improved by encoder change. This proposal aims to optimize the encoder with more efficient mode decision algorithm for better coding efficiency with similar computational complexity of current TMuC design.

# Encoder Scheme

The unified intra in current TMuC determines the best prediction mode with RD optimization of the first Nmodes plus one more mode from the rough mode decision. Instead of comparing the reconstructed distortion, the most probable candidate mode is the one with minimum absolute sum of Hadamard Transformed coefficients of residual signal (HSAD) in the rough mode decision.

In the new unified intra, RD optimization is not applied to the fixed Nmodes but to the Nbest candidate modes selected by the rough mode decision where all modes are compared in this decision. Moreover, the mode bits are also considered in the rough mode decision when searching for the Ncandidate modes with least costs. The following table summarizes full prediction, unified intra and new unified intra.

Table I. Comparison of different encoding schemes

|  | Rough mode decision (RMD) | Modes for RD optimization |
| --- | --- | --- |
| Full RD | None | All modes 17 for 4x4 34 for 8x8 34 for 16x16 34 for 32x32 5 for 64x64 |
| TMuC 0.7 encoder scheme |  | FirstNmodes+1(best mode from RMD) 9+1 for 4x4 9+1 for 8x8 4+1 for 16x16 4+1 for 32x32 5 for 64x64 |
| New encoder scheme | MinN modes HSAD(Resmode)+λ⋅bit(mode)where mode∈ All modes | Nbest modes (from RMD) S1: 9 for 4x4 9 for 8x8 4 or 16x16 4 or 32x32 5 for 64x64 |
|  |  | Nbest modes (from RMD) S2: 9 for 4x4 9 for 8x8 9 or 16x16 9 or 32x32 5 for 64x64 |

# Experimental Results

To show the objective performance, unified intra prediction with full RD search, with default encoding scheme in TMuC0.7 and the new encoding scheme with two different parameter settings are compared with original ADI for all the PU sizes (UNIFIED_DIRECTION_INTRA=0, ANG_INTRA=0). The experiments are conducted with 6 test configurations specified for TMuC 0.7 in JCTVC-B300. Frame number of 1 second is tested instead of 10 seconds frames to prevent the extreme long running time. Table II and table III show the BD-rate savings of high efficiency coding and low complexity coding respectively. The average BD-rate saving is also illustrated in fig.1. The computational complexity of the new unified intra is presented by the relative encoding time with respect to the unified intra in table IV.

Table II. BD-rates of high efficiency coding

|  |  | Intra |  |  |  | Random access |  |  |  | Low delay |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sequence | Default | New_s1 | New_s2 | Full RD | Default | New_s1 | New_s2 | Full RD | Default | New_s1 | New_s2 | Full RD |
| Class A | Traffic | -0.82 | -1.36 | -1.74 | -2.49 | -0.36 | -0.62 | -0.99 | -1.36 |  |  |  |  |
| 4k | PeopleOnStreet | -1.08 | -1.79 | -2.12 | -2.94 | -0.47 | -0.82 | -0.91 | -1.20 |  |  |  |  |
| Class B | Kimono | 0.19 | 0.3 | -0.16 | -0.56 | -0.14 | -0.14 | -0.25 | -0.25 | -0.12 | -0.06 | -0.19 | -0.35 |
| 1080p | ParkScene | 0.84 | 0.5 | 0.07 | -1.10 | -0.02 | -0.21 | -0.51 | -1.11 | -0.02 | -0.19 | -0.38 | -0.88 |
|  | Cactus | -1.58 | -2.26 | -2.58 | -3.39 | -0.52 | -0.77 | -1.02 | -1.48 | -0.44 | -0.61 | -0.70 | -1.02 |
|  | BasketballDrive | -2.00 | -2.21 | -2.78 | -3.33 | -1.03 | -1.22 | -1.65 | -2.05 | -0.58 | -0.80 | -0.92 | -1.05 |
|  | BQTerrace | -1.86 | -2.25 | -2.38 | -2.94 | -0.17 | -0.41 | -0.42 | -0.83 | -0.11 | -0.17 | -0.45 | -0.78 |
| Class C | BasketballDrill | -6.01 | -7.06 | -7.32 | -7.85 | -1.05 | -1.61 | -2.16 | -2.26 | -0.70 | -1.16 | -1.45 | -1.54 |
| WVGA | BQMall | -1.30 | -1.55 | -1.75 | -2.51 | -0.72 | -0.75 | -0.97 | -1.28 | -0.24 | -0.43 | -0.41 | -0.64 |
|  | PartyScene | -0.51 | -0.81 | -0.88 | -1.50 | -0.54 | -0.68 | -0.71 | -1.17 | -0.09 | -0.34 | -0.48 | -0.56 |
|  | RaceHorsesC | -1.11 | -2.11 | -2.33 | -3.16 | -0.38 | -0.92 | -0.81 | -1.18 | -0.25 | -0.56 | -0.61 | -0.83 |
| Class D | BasketballPass | -4.50 | -5.11 | -5.38 | -5.84 | -1.06 | -1.79 | -1.42 | -2.07 | -0.17 | -0.87 | -0.56 | -0.60 |
| WQVGA | BQSquare | -1.52 | -1.96 | -2.02 | -2.58 | -0.44 | -0.48 | -0.65 | -0.82 | 0.09 | -0.38 | -0.32 | -0.55 |
|  | BlowingBubbles | -1.12 | -1.72 | -1.97 | -2.71 | -0.40 | -0.50 | -0.70 | -1.21 | -0.29 | -0.14 | -0.24 | -0.72 |
|  | RaceHorses | -1.76 | -2.60 | -2.80 | -3.48 | -0.44 | -0.85 | -0.85 | -1.23 | -0.18 | -0.44 | -0.43 | -0.89 |
| Class E | Vidyo1 | -3.22 | -4.01 | -4.63 | -5.40 |  |  |  |  | -0.43 | -0.97 | -0.96 | -1.31 |
| 720p | Vidyo3 | -2.46 | -2.61 | -2.97 | -3.61 |  |  |  |  | -0.68 | -0.44 | -0.77 | -1.31 |
|  | Vidyo4 | -1.67 | -1.94 | -2.50 | -3.16 |  |  |  |  | -0.71 | -1.10 | -0.83 | -1.21 |
|  | Class A | -0.95 | -1.58 | -1.93 | -2.71 | -0.41 | -0.72 | -0.95 | -1.28 |  |  |  |  |
|  | Class B | -0.88 | -1.18 | -1.57 | -2.26 | -0.38 | -0.55 | -0.77 | -1.15 | -0.25 | -0.37 | -0.53 | -0.82 |
|  | Class C | -2.23 | -2.88 | -3.07 | -3.76 | -0.67 | -0.99 | -1.16 | -1.47 | -0.32 | -0.62 | -0.74 | -0.89 |
|  | Class D | -2.23 | -2.85 | -3.04 | -3.65 | -0.59 | -0.90 | -0.90 | -1.33 | -0.14 | -0.46 | -0.39 | -0.69 |
|  | Class E | -2.45 | -2.85 | -3.36 | -4.06 |  |  |  |  | -0.61 | -0.84 | -0.85 | -1.28 |
|  | Average all | -1.75 | -2.25 | -2.57 | -3.25 | -0.52 | -0.79 | -0.93 | -1.3 | -0.31 | -0.54 | -0.61 | -0.89 |

Table III. BD-rates of low complexity coding

|  |  | Intra LoCo |  |  |  | Random access LoCo |  |  |  | Low delay LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sequence | Default | New_s1 | New_s2 | Full RD | Default | New_s1 | New_s2 | Full RD | Default | New_s1 | New_s2 | Full RD |
| Class A | Traffic | -1.32 | -1.68 | -2.25 | -3.31 | -0.55 | -0.73 | -1.05 | -1.57 |  |  |  |  |
| 4k | PeopleOnStreet | -2.47 | -3.09 | -3.58 | -4.59 | -1.02 | -1.19 | -1.38 | -1.75 |  |  |  |  |
| Class B | Kimono | -0.42 | 0.23 | -0.65 | -1.58 | -0.06 | 0.05 | -0.07 | -0.16 | -0.26 | -0.05 | -0.19 | -0.25 |
| 1080p | ParkScene | 0.42 | 0.45 | -0.09 | -1.61 | 0.24 | 0.26 | 0.08 | -0.65 | 0.23 | 0.09 | 0.01 | -0.53 |
|  | Cactus | -3.10 | -3.53 | -3.96 | -5.15 | -1.10 | -1.25 | -1.32 | -1.93 | -0.69 | -0.65 | -0.88 | -1.25 |
|  | BasketballDrive | -2.58 | -2.35 | -3.17 | -4.08 | -1.32 | -1.23 | -1.72 | -2.25 | -0.91 | -0.66 | -0.93 | -1.36 |
|  | BQTerrace | -3.28 | -3.69 | -3.85 | -4.70 | -0.72 | -0.84 | -0.91 | -1.34 | -0.56 | -0.44 | -0.51 | -0.84 |
| Class C | BasketballDrill | -7.90 | -8.72 | -8.97 | -9.68 | -2.21 | -2.36 | -2.82 | -3.37 | -1.38 | -2.03 | -2.19 | -2.04 |
| WVGA | BQMall | -2.00 | -2.32 | -2.64 | -3.67 | -1.03 | -1.06 | -1.30 | -1.67 | -0.20 | -0.28 | -0.34 | -0.68 |
|  | PartyScene | -0.50 | -0.76 | -0.84 | -1.75 | -0.36 | -0.64 | -0.59 | -1.08 | -0.25 | -0.44 | -0.44 | -0.62 |
|  | RaceHorsesC | -1.94 | -2.52 | -2.77 | -3.84 | -0.73 | -1.08 | -1.18 | -1.56 | -0.45 | -0.54 | -0.64 | -0.92 |
| Class D | BasketballPass | -5.32 | -5.81 | -6.11 | -6.83 | -1.42 | -1.36 | -1.77 | -2.23 | -1.14 | -0.97 | -0.81 | -1.32 |
| WQVGA | BQSquare | -1.87 | -2.29 | -2.34 | -3.15 | -0.22 | -0.28 | -0.20 | -0.68 | 0.09 | -0.15 | -0.06 | -0.26 |
|  | BlowingBubbles | -1.50 | -2.01 | -2.29 | -3.25 | -0.27 | -0.41 | -0.36 | -0.87 | -0.30 | -0.15 | -0.42 | -0.88 |
|  | RaceHorses | -2.47 | -3.11 | -3.28 | -4.29 | -0.94 | -0.97 | -1.02 | -1.40 | -0.34 | -0.56 | -0.36 | -0.56 |
| Class E | Vidyo1 | -4.57 | -5.12 | -5.95 | -6.99 |  |  |  |  | -1.47 | -1.60 | -1.77 | -2.14 |
| 720p | Vidyo3 | -3.39 | -3.68 | -4.21 | -5.07 |  |  |  |  | -1.28 | -0.73 | -1.09 | -1.20 |
|  | Vidyo4 | -2.40 | -2.59 | -3.39 | -4.27 |  |  |  |  | -0.93 | -0.94 | -1.15 | -1.40 |
|  | Class A | -1.90 | -2.39 | -2.92 | -3.95 | -0.78 | -0.96 | -1.22 | -1.66 |  |  |  |  |
|  | Class B | -1.79 | -1.78 | -2.34 | -3.42 | -0.59 | -0.60 | -0.79 | -1.27 | -0.44 | -0.34 | -0.50 | -0.85 |
|  | Class C | -3.08 | -3.58 | -3.80 | -4.74 | -1.08 | -1.28 | -1.47 | -1.92 | -0.57 | -0.82 | -0.90 | -1.07 |
|  | Class D | -2.79 | -3.31 | -3.50 | -4.38 | -0.71 | -0.75 | -0.83 | -1.30 | -0.42 | -0.46 | -0.41 | -0.76 |
|  | Class E | -3.45 | -3.79 | -4.52 | -5.45 |  |  |  |  | -1.22 | -1.09 | -1.33 | -1.58 |
|  | Average all | -2.59 | -2.92 | -3.35 | -4.32 | -0.78 | -0.87 | -1.04 | -1.50 | -0.61 | -0.63 | -0.74 | -1.02 |

Table IV. Average encoding time

|  | Default | New_s1 | New_s2 | FullRD |
| --- | --- | --- | --- | --- |
| Intra | 1.00 | 0.84 | 1.54 | 5.05 |
| Random access | 1.00 | 0.91 | 1.05 | 1.55 |
| Low delay | 1.00 | 0.90 | 1.09 | 1.52 |
| Intra_LoCo | 1.00 | 0.87 | 1.53 | 5.20 |
| Random access_LoCo | 1.00 | 0.94 | 1.13 | 1.73 |
| Low delay_LoCo | 1.00 | 0.94 | 1.10 | 1.70 |

Fig.1. Average BD-rates

# Conclusion

This contribution proposes a new encoding scheme for unified intra prediction. Experimental results show that the proposed method can improve coding efficiency of unified intra prediction performance by additional 0.82% bit saving. Even with 84% complexity of unified intra, 0.5% bit saving also can be achieved. We recommend adopting it to the TMuC software as default behaviour.

# Reference

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010

- J.-H. Min, S. Lee, I.-K. Kim, W.-J. Han, J. Lainema, and K. Ugur, “Unification of the Directional Intra Prediction Methods in TMuC,” JCTVC-B100, Geneva, Switzerland, Jul 2010.

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).