| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C208 WG11 Number: m18246 |
| --- | --- |

| Title: | Simulation results of motion partition, block merging and motion competition schemes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In current TMuC design, there are two approaches for frame partitioning: prediction unit (PU) splitting to make smaller motion partition and block merging (MRG) to make larger motion partition. In this proposal, anchor results are obtained by running TMuC-0.81 using reference configuration with disabling all motion partitions, motion competition and MRG. By enabling MRG on top of the anchor, improved BD-Rate is about -2.0% (random access), -0.2% (random access LC), -2.4% (low delay) and 0.0% (low delay LC). But higher BD-Rate savings can be obtained by enabling motion partition: -4.2% (random access), -5.8% (random access LC), -4.1% (low delay) and -5.1% (low delay LC). By enabling motion competition, improved BD-Rate savings are about -4.3% (random access), -5.6% (random access LC), -3.2% (low delay) and -5.2% (low delay LC). The best performance can be obtained by combining motion partitions and motion vector competition which shows -7.2% (random access), -9.2% (random access LC), -6.2% (low delay) and -8.3% (low delay LC) BD-Rate savings.

- Introduction

In current TMuC design, there are two different ways to make motion partition: partition unit (PU) splitting to make smaller motion partition and block merging (MRG) to make larger motion partition [1]. In PU splitting, motion competition scheme can be utilized to reduce the bits of motion vectors for each motion partitions. In this proposal, brief description of motion partition by PU splitting, MRG and motion competition schemes are given and performances of those methods are evaluated.

- Motion partition, block merging and motion competition

PU splitting is top-down approach: splitting motion partition to smaller one as a unit of fixed partition shape. Several fixed motion partition shape can be used in splitting. In current TMuC, motion partition can have square, rectangular, asymmetric and geometric shape. Motion information including motion vectors and reference index has to be mapped to each motion partition. Motion competition scheme can be utilized to reduce the bits of motion information.

On the contrary, MRG is bottom-up approach: making bigger motion partition by merging. Left and above motion partitions are candidates of merging process. The explicit flags are sent to decoder to indicate which motion partition is merged. In principle, highly flexible partition shape can be constructed by merging the several motion partitions. For optimality, it is beneficial to re-estimate the motion information of merged partition and re-encode the motion information after every merging. However, it has to be noted that in current TMuC software, re-estimation and re-encoding are not implemented yet.

# Experimental results

In this section, total 5 experimental results are provided. Anchor results are obtained by running TMuC-0.81 using reference configuration with disabling all motion partition, motion competition and MRG [2]. Table 1 summarizes the macros and command-line options to generate anchor results based on the TMuC-0.81. To reduce the testing time, limited number of frames are encoded which corresponds to 1 second duration of input sequences

Table 1. Macros and command-line options to generate anchor results based on the TMuC-0.81

| Configuration | Macro | Commandline |
| --- | --- | --- |
| Disabling motion partitions | #define HHI_DISABLE_INTER_NxN_SPLIT 1 #define HHI_RMP_SWITCH 1 | --RMP=0 --AMP=0 |
| Disabling motion competition | #define HHI_AMVP_OFF 1 |  |
| Disabling MRG | #define HHI_MRG 0 |  |

## Performance of MRG

Performances of MRG are obtained by setting HHI_MRG as 1 in the software. The summarized results are -2.0% (random access), -0.2% (random access LC), -2.4% (low delay) and 0.0% (low delay LC). It should be noted that performances of MRG with all motion partitions and motion competition are given by TE12 reports [3].

Table 2. Summarized BD-Rate when MRG is enabled.

|  | Random Access HE | Random Access LC | Low delay HE | Low delay LC |
| --- | --- | --- | --- | --- |
| Class A | -2.3 | -0.7 | - | - |
| Class B | -2.4 | 0.0 | -2.4 | -1.5 |
| Class C | -1.6 | -0.8 | -1.8 | -1.3 |
| Class D | -1.6 | 0.2 | -1.8 | -0.2 |
| Class E | - | - | -3.8 | 4.8 |
| Average | -2.0 | -0.2 | -2.4 | 0.0 |

## Performance of motion partition

Performance of motion partitions is obtained by enabling motion partitions (–AMP=1 in command-line after HHI_DISABLE_INTER_NxN_SPLIT and HHI_RMP_SWITCH are set 0) over anchor configuration. According to summarized results from Table 3, motion partitions bring -4.2% (random access), -5.8% (random access LC), -4.1% (low delay) and -5.1% (low delay LC) BD-Rate savings, respectively. Encoding time is increased about 3 times more than anchor.

Table 3. Summarized BD-Rate when motion partitions are enabled.

|  | Random Access HE | Random Access LC | Low delay HE | Low delay LC |
| --- | --- | --- | --- | --- |
| Class A | -4.7 | -7.1 | - | - |
| Class B | -3.1 | -4.8 | -2.9 | -4.3 |
| Class C | -4.4 | -5.9 | -3.8 | -4.6 |
| Class D | -5.0 | -6.3 | -4.7 | -5.0 |
| Class E | - | - | -5.7 | -7.3 |
| Total | -4.2 | -5.8 | -4.1 | -5.1 |

## Performance of motion competition

Performances of motion competitions are obtained by setting HHI_AMVP_OFF macro as 0. The obtained BD-Rate savings are -4.3% (random access), -5.6% (random access LC), -3.2% (low delay) and -5.2% (low delay LC). According to the results, coding efficiency of motion competition is slightly lower than that of motion partition. However, its coding efficiency is higher than that of MRG.

Table 4. Summarized BD-Rate when motion competition is enabled.

|  | Random Access HE | Random Access LC | Low delay HE | Low delay LC |
| --- | --- | --- | --- | --- |
| Class A | -5.2 | -7.0 | - | - |
| Class B | -4.5 | -5.7 | -2.9 | -4.9 |
| Class C | -3.8 | -4.9 | -3.3 | -4.6 |
| Class D | -4.2 | -5.3 | -3.0 | -4.5 |
| Class E | - | - | -3.9 | -7.2 |
| Total | -4.3 | -5.6 | -3.2 | -5.2 |

## Performance of combination of motion partition and motion competition

As mentioned in section 2, motion competition can be utilized to reduce the bits of motion information for each motion partitions. In this experiment, performances of the motion partition and motion competition are obtained by enabling two methods together. Table 5 shows the summarized results: -7.2% (random access), -9.2% (random access LC), -6.2% (low delay) and -8.3% (low delay LC), respectively. As you can see, motion competition brings extra gains from motion partitions.

Table 5. Summarized BD-Rate when motion both partitions and motion competitions are enabled.

|  | Random Access HE | Random Access LC | Low delay HE | Low delay LC |
| --- | --- | --- | --- | --- |
| Class A | -8.1 | -11.2 | - | - |
| Class B | -6.6 | -8.6 | -4.9 | -7.7 |
| Class C | -6.9 | -8.8 | -6.3 | -7.7 |
| Class D | -7.6 | -9.3 | -6.7 | -7.8 |
| Class E | - | - | -7.5 | -11.0 |
| Total | -7.2 | -9.2 | -6.2 | -8.3 |

## Performance of TMuC

In this experiment, performance of motion partition, motion competition and MRG is provided by enabling three tools at a time. In fact, this setting is identical to the reference configuration of TMuC software ad-hoc group. The summarized BD-Rate improvements in this experiment are -6.3% (random access), -6.7% (random access LC), -5.9% (low delay) and –5.6% (low delay LC), respectively.

It has to be noted that Skip/Direct modes are not utilized in this configuration. Performance of TMuC with Skip/Direct modes will be reported within TE11 scope.

Table 6. Summarized results when motion partition, motion competition and MRG are enabled.

|  | Random Access HE | Random Access LC | Low delay HE | Low delay LC |
| --- | --- | --- | --- | --- |
| Class A | -7.3 | -8.8 | - | - |
| Class B | -5.7 | -5.3 | -5.0 | -6.0 |
| Class C | -6.3 | -7.4 | -5.7 | -6.6 |
| Class D | -6.8 | -6.8 | -6.3 | -5.9 |
| Class E | - | - | -7.5 | -2.9 |
| Total | -6.3 | -6.7 | -5.9 | -5.6 |

# Conclusions

When the motion partition is disabled, by enabling MRG, improved BD-Rate savings were about -2.0% (random access), -0.2% (random access LC), -2.4% (low delay) and 0.0% (low delay LC). But higher BD-Rate savings could be obtained by enabling motion partition: -4.2% (random access), -5.8% (random access LC), -4.1% (low delay) and -5.1% (low delay LC). By enabling motion competition, improved BD-Rate savings were about -4.3% (random access), -5.6% (random access LC), -3.2% (low delay) and -5.2% (low delay LC). The best performance could be obtained by enabling motion partitions and motion vector competition together which shows -7.2% (random access), -9.2% (random access LC), -6.2% (low delay) and -8.3% (low delay LC) BD-Rate savings.

# References

- “Test Model under consideration”, JCTVC-B205, Geneva, CH, 21-28 July, 2010

- https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tag/0.81

- K. Sugimoto, A. Minezawa and S. Sekiguchi, “TE12.1 cross-check report on block merging (MRG) on/off”, JCTVC-C118, Guangzhou, CN, 7-15 October, 2010

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).