| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C209 WG11 Number: m18247 |
| --- | --- |

| Title: | Low-complexity 16x16 and 32x32 transforms and partial frequency transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoon Mi Hong Min-Su Cheon Il-Koo Kim | Email: | yoonmi.hong@samsung.com minsu.cheon@samsung.com ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes new 32-point fast DCT transform based on the Loeffler’s design principles. Fast integer realization of 16-point and 32-point transform is also provided based on the proposed transform. In addition, partial frequency transform scheme is utilized to reduce the complexity of transforms of current TMuC further. Proposed approaches reduce the number of operations significantly with negligible performance loss compared to the current TMuC.

- Introduction

H.264/AVC supports the DCT-like integer transforms for 4x4 and 8x8 blocks. However, its direct extension to large sizes such as 16x16, 32x32 and 64x64 is not easy because of the dedicated design for 4x4 and 8x8 only. Recently, integer transforms for 16x16, 32x32 and 64x64 sizes based on Chen’s fast algorithm was proposed and implemented in the current TMuC design [1]. While Chen’s factorization has fully recursive and regular butterfly structure which is beneficial to hardware design, it is not as fast as the algorithm based on Loeffler’s [2], [3]. Unfortunately, Loeffler’s fast algorithm has been known only for 4-point, 8-point and 16-point DCTs. This contribution presents its extension to 32-point DCT and its integer realization which is less complex than transform in current TMuC. Integer realization of existing 16-point transform is also provided to reduce the complexity further.

In addition, partial frequency transform is proposed. Since most of coefficients or energy is concentrated on the low frequency for large transform, complexity can be reduced further if only low frequency part is used for large transform cores. The performance does not change severely even though high frequency coefficients are set to be zero.

# New fast algorithm for 32-point discrete cosine transform (DCT)

The signal flow graph of proposed 32-point fast DCT is illustrated in Figure 1. Note that 2N-point DCT reuses N-point DCT recursively. Here,  and  denote  and , respectively.

Figure 1. Signal flow graph of proposed 32-point fast DCT

# Integer realization of 16-point and 32-point transforms

For effective hardwired design, all cosine and sine values in Figure 1 are approximated to the type of q/2p where p is a precision value adjusted according to dynamic range limitation and coding efficiency. The error between the original DCT and its approximation is compensated by scaling factors and those factors are integrated into the forward and inverse quantization step to reduce the number of operations further.

In inverse quantization process, 6 inverse quantization matrices which have same size with input block are utilized in order to compensate differences introduced by incorporating scaling into inverse quantization steps. In this proposal, each inverse quantization matrices are simplified by replacing those using a scalar value in order to reduce memory requirement. No significant loss is introduced by this simplification. Similar approach is proposed by JCTVC-C226, independently.

Compared to Chen’s transform, the proposed transforms have less number of stages that the input should go through. The forward and inverse transforms can be implemented with approximated multiplication factors by following the flow-graph in Figure 1 from left to right direction, and vice versa. The inverse transform process for 16-point and 32-point inputs are described below.

- 16-point one-dimensional inverse transform process is defined as the following 5 stages. Note that stage 0 re-orders the input so that it is well-fitted with the flow-graph. Xi (i = 0...15) are elements of input vector and Yi (i=0...15) are elements of output vector. In each stage, sets of intermediate values Ei, Di, Ci, Bi (i = 0...15) are derived and fed into the next stage.

| Stage 0 | Stage 1 | Stage 2 |
| --- | --- | --- |
| D0 = X0 D1 = -X8 D2 = X4 D3 = X12 D4 = X2 E5 = X6 D6 = X14 E7 = -X10 E8 = X3 E9 = X13 D10 = X9 E11 = X15 E12 = X1 D13 = X7 E14 = -X11 E15 = X5 | D5 = (45*(E5 - E7)) >> 6 D7 = (45*(E5 + E7)) >> 6 D8 = (24*E8 + 59*E9) >> 6 D9 = (-59*E8 + 24*E9) >> 6 D11 = (45*(E11 - E12)) >> 6 D12 = (45*(E11 + E12)) >> 6 D14 = (24*E14 + 59*E15) >> 6 D15 = (-59*E14 + 24*E15) >> 6 | C0 = (45*(D0 - D1)) >> 6 C1 = (45*(D0 + D1)) >> 6 C2 = (24*D2 - 59*D3) >> 6 C3 = (59*D2 + 24*D3) >> 6 C4 = D4 + D5 C5 = D4 - D5 C6 = -D6 + D7 C7 = D6 + D7 C8 = D8 + D14 C14 = D8 - D14 C9 = D9 + D15 C15 = D9 - D15 C10 = D10 + D11 C11 = D10 - D11 C12 = D12 + D13 C13 = D12 - D13 |

| Stage 3 | Stage 4 | Stage 5 |
| --- | --- | --- |
| B0 = C0 + C3 B3 = C0 - C3 B1 = C1 + C2 B2 = C1 - C2 B4 = (6*C4 - 31*C7) >> 5 B7 = (31*C4 + 6*C7)>> 5 B5 = (35*C5 - 53*C6) >> 6 B6 = (53*C5 + 35*C6) >> 6 B8 = C8 + C11 B11 = C8 - C11 B9 = C9 + C10 B10 = C9 - C10 B12 = C12 + C15 B15 = C12 - C15 B13 = C13 + C14 B14 = C13 - C14 | A0 = B0 + B7 A7 = B0 - B7 A1 = B1 + B6 A6 = B1 - B6 A2 = B2 + B5 A5 = B2 - B5 A3 = B3 + B4 A4 = B3 - B4 A8 = (49*B8 - 40*B15) >> 6 A15 = (40*B8 + 49*B15) >> 6 A9 = (30*B9 + 56*B14) >> 6 A14 = (-56*B9 + 30*B14) >> 6 A10 = (61*B10 - 18*B13) >> 6 A13 = (18*B10 + 61*B13) >> 6 A11 = (6*B11 + 63*B12) >> 6 A12 = (-63*B11 + 6*B12) >> 6 | Y0 = A0 + A15 Y1 = A1 + A14 Y2 = A2 + A13 Y3 = A3 + A12 Y4 = A4 + A11 Y5 = A5 + A10 Y6 = A6 + A9 Y7 = A7 + A8 Y8 = A7 - A8 Y9 = A6 - A9 Y10 = A5 - A10 Y11 = A4 - A11 Y12 = A3 - A12 Y13 = A2 - A13 Y14 = A1 - A14 Y15 = A0 - A15 |

- 32-point one-dimensional inverse transform process is defined as the following 7 stages. Again, stage 0 re-distributes the input vector. Xi (i = 0...31) are elements of input vector and Yi (i=0...31) are elements of output vector. In each stage, sets of intermediate values Fi, Ei, Di, Ci, Bi, Ai, Zi (i = 0...31) are derived and fed into next stage.

| Stage 0 | Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- | --- |
| D0= X0 E24 = X1 E12 = X2 F16 = -X3 D4 = X4 F31 = X5 E8 = X6 E26 = -X7 D2 = X8 E21 = X9 E15 = X10 F29 = X11 E5 = X12 F18 = -X13 D13 = X14 D22 = X15 D1 = -X16 D25 = X17 D10 = X18 F19 = -X19 E7 = -X20 F28 = -X21 E14 = -X22 E20 = -X23 D3 = X24 E27 = -X25 E9 = X26 F30 = -X27 D6 = X28 F17 = -X29 E11 = X30 E23 = -X31 | E16 = (251*F16 + 49*F17) >> 8 E17 = (-49*F16 + 251*F17) >> 8 E18 = (212*F18 + 142*F19) >> 8 E19 = (-142*F18 + 212*F19) >> 8 E28 = (212*F28 + 142*F29) >> 8 E29 = (-142*F28 + 212*F29) >> 8 E30 = (251*F30 + 49*F31) >> 8 E31 = (-49*F30 + 251*F31) >> 8 | D5 = (181*(E5 - E7)) >> 8 D7 = (181*(E5 + E7)) >> 8 D8 = (97*E8 + 236*E9) >> 8 D9 = (-236*E8 + 97*E9) >> 8 D11 = (181*(E11 - E12)) >> 8 D12 = (181*(E11 + E12)) >> 8 D14 = (97*E14 + 236*E15) >> 8 D15 = (-236*E14 + 97*E15) >> 8 D16 = E16 + E18 C18 = E16 - E18 C17 = E17 + E19 D19 = E17 - E19 D20 = (236*E20 - 97*E21) >> 8 D21 = (97*E20 + 236*E21) >> 8 D23 = (181*(E23 - E24)) >> 8 D24 = (181*(E23 + E24)) >> 8 D26 = (236*E26 - 97*E27) >> 8 D27 = (97*E26 + 236*E27) >> 8 D28 = - E28 + E30 C30 = E28 + E30 C29 = - E29 + E31 D31 = E29 + E31 | C0 = (181*(D0 - D1)) >> 8 C1 = (181*(D0 + D1)) >> 8 C2 = (97*D2 - 236*D3) >> 8 C3 = (236*D2 + 97*D3) >> 8 C4 = D4 + D5 C5 = D4 - D5 C6 = -D6 + D7 C7 = D6 + D7 C8 = D8 + D14 C14 = D8 - D14 C9 = D9 + D15 C15 = D9 - D15 C10 = D10 + D11 C11 = D10 - D11 C12 = D12 + D13 C13 = D12 - D13 C16 = (181*(D16 - D19)) >> 8 C19 = (181*(D16 + D19)) >> 8 C20 = D20 + D26 C26 = D20 - D26 C21 = D21 + D27 C27 = D21 - D27 C22 = D22 + D23 C23 = D22 - D23 C24 = D24 + D25 C25 = D24 - D25 C28 = (181*(D28 - D31)) >> 8 C31 = (181*(D28 + D31)) >> 8 |

| Stage 4 | Stage 5 | Stage 6 | Stage 7 |
| --- | --- | --- | --- |
| B0 = C0 + C3 B3 = C0 - C3 B1 = C1 + C2 B2 = C1 - C2 B4 = (49*C4 - 251*C7) >> 8 B7 = (251*C4 + 49*C7) >> 8 B5 = (142*C5 - 212*C6) >> 8 B6 = (212*C5 + 142*C6) >> 8 B8 = C8 + C11 B11 = C8 - C11 B9 = C9 + C10 B10 = C9 - C10 B12 = C12 + C15 B15 = C12 - C15 B13 = C13 + C14 B14 = C13 - C14 B16 = C16 + C28 B28 = C16 - C28 B17 = C17 + C29 B29 = C17 - C29 B18 = C18 + C30 B30 = C18 - C30 B19 = C19 + C31 B31 = C19 - C31 B20 = C20 + C23 B23 = C20 - C23 B21 = C21 + C22 B22 = C21 - C22 B24 = C24 + C27 B27 = C24 - C27 B25 = C25 + C26 B26 = C25 - C26 | A0 = B0 + B7 A7 = B0 - B7 A1 = B1 + B6 A6 = B1 - B6 A2 = B2 + B5 A5 = B2 - B5 A3 = B3 + B4 A4 = B3 - B4 A8 = (197*B8 - 162*B15) >> 8 A15 = (162*B8 + 197*B15) >> 8 A9 = (120*B9 + 225*B14) >> 8 A14 = (-225*B9 + 120*B14) >> 8 A10 = (244*B10 - 74*B13) >> 8 A13 = (74*B10 + 244*B13) >> 8 A11 = (25*B11 + 254*B12) >> 8 A12 = (-254*B11 + 25*B12) >> 8 A16 = B16 + B23 A23 = B16 - B23 A17 = B17 + B22 A22 = B17 - B22 A18 = B18 + B21 A21 = B18 - B21 A19 = B19 + B20 A20 = B19 - B20 A24 = B24 + B31 A31 = B24 - B31 A25 = B25 + B30 A30 = B25 - B30 A26 = B26 + B29 A29 = B26 - B29 A27 = B27 + B28 A28 = B27 - B28 | Z0 = A0 + A15 Z1 = A1 + A14 Z2 = A2 + A13 Z3 = A3 + A12 Z4 = A4 + A11 Z5 = A5 + A10 Z6 = A6 + A9Z7 = A7 + A8 Z8 = A7 - A8 Z9 = A6 - A9 Z10 = A5 - A10 Z11 = A4 - A11 Z12 = A3 - A12 Z13 = A2 - A13 Z14 = A1 - A14 Z15 = A0 - A15 Z16 = (171*A16 + 189*A31) >> 8 Z31 = (-189*A16 + 171*A31) >> 8 Z17 = (205*A17 - 152*A30) >> 8 Z30 = (152*A17 + 205*A30) >> 8 Z18 = (131*A18 + 219*A29) >> 8 Z29 = (-219*A18 + 131*A29) >> 8 Z19 = (231*A19 - 109*A28) >> 8 Z28 = (109*A19 + 231*A28) >> 8 Z20 = (86*A20 + 241*A27) >> 8 Z27 = (-241*A20 + 86*A27) >> 8 Z21 = (248*A21 - 62*A26) >> 8 Z26 = (62*A21 + 248*A26) >> 8 Z22 = (37*A22 + 253*A25) >> 8 Z25 = (-253*A22 + 37*A25) >> 8 Z23 = (255*A23 - 12*A24) >> 8 Z24 = (12*A23 + 255*A24) >> 8 | Y0 = Z0 + Z31 Y31 = Z0 - Z31 Y1 = Z1 + Z30 Y30 = Z1 - Z30 Y2 = Z2 + Z29 Y29 = Z2 - Z29 Y3 = Z3 + Z28 Y28 = Z3 - Z28 Y4 = Z4 + Z27 Y27 = Z4 - Z27 Y5 = Z5 + Z26 Y26 = Z5 - Z26 Y6 = Z6 + Z25 Y25 = Z6 - Z25 Y7 = Z7 + Z24 Y24 = Z7 - Z24 Y8 = Z8 + Z23 Y23 = Z8 - Z23 Y9 = Z9 + Z22 Y22 = Z9 - Z22 Y10 = Z10 + Z21 Y21 = Z10 - Z21 Y11 = Z11 + Z20 Y20 = Z11 - Z20 Y12 = Z12 + Z19 Y19 = Z12 - Z19 Y13 = Z13 + Z18 Y18 = Z13 - Z18 Y14 = Z14 + Z17 Y17 = Z14 - Z17 Y15 = Z15 + Z16 Y16 = Z15 - Z16 |

# Partial frequency transform for low complexity implementation of large transform

In order to reduce complexity of transform further, partial frequency transform is described in this section. In the partial frequency transform, only low frequency coefficients are calculated. The high frequency coefficients are not calculated at all. Specifically, if only 8 low frequency coefficients are utilized, only X0, X1,...,X7 among the input vectors of the inverse transform are fed into the next stage and the rest of them, that is, X8, ..., XN (N=16, 32, 64) are set to be zero and not calculated. Then, calculations to derive intermediate values in the next stage can be saved. Moreover, the number of 1-D transforms to generate 2-D transform results in the inverse transform process is also reduced. In general, for 2-D inverse transform for NxN block, the 1-D transform process is repeated N times for vertical direction and horizontal direction, respectively. However, if only 8x8 low frequency coefficients are used, the process can be repeated only 8 times for one direction and N times for the other direction. Consequently, the number of operations for 2-D inverse transform is significantly reduced.

# Complexity measurement

Compared to Chen’s factorization, Loeffler’s algorithm for 16-point transform and its extension to 32-point transform has less number of rotations, which lead less number of operations. The comparison of complexity in terms of the number of operations for 16x16 and 32x32 transforms is summarized in Table 1.

Table 1. Number of operations for 16-point and 32-point transforms

|  |  | Chen 16T | Loeffler 16T | Chen 32T | New 32T |
| --- | --- | --- | --- | --- | --- |
| Number of operations | Addition | 74 | 72 | 194 | 186 |
|  | Multiplication | 44 | 42 | 116 | 104 |
|  | Shift | 26 | 24 | 66 | 58 |
|  | Total | 144 | 138 | 376 | 348 |

For low complexity implementation, as an example, the complexity reduction is compared in Table 2 and  when only 8x8 or 16x16 low frequency area is used for large transforms based on Loeffler’s algorithm. Note that for 32x32, using only one 16x16 low frequency transform with 32x32 transform core has less number of operations than using four times 16x16 transform cores. Table 4 shows the complexity reduction for Chen’s 64x64 transform.

Table 2. Comparison of complexity for partial frequency transform for 16x16 block

|  |  | Loeffler 16x16 | 8x8 low frequency transform | Ratio |
| --- | --- | --- | --- | --- |
| Number of operations per pixel | Addition | 9.00 | 5.25 | 0.58 |
|  | Multiplication | 5.25 | 3.38 | 0. 64 |
|  | Shift | 3.00 | 2.25 | 0.75 |
|  | Total | 17.25 | 10.88 | 0.63 |

Table 3. Comparison of complexity for partial frequency transform for 32x32 block

|  |  | Loeffler 32x32 | 16x16 low frequency transform | Ratio | 8x8 low frequency transform | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Number of operations per pixel | Addition | 11.63 | 7.31 | 0.63 | 4.84 | 0.42 |
|  | Multiplication | 6.50 | 4.03 | 0.62 | 2.73 | 0.42 |
|  | Shift | 3.63 | 2.72 | 0.75 | 1.80 | 0.50 |
|  | Total | 21.75 | 14.06 | 0.65 | 9.38 | 0.43 |

Table 4. Comparison of complexity for partial frequency transform for 64x64 block

|  |  | Chen 64x64 | 16x16 low frequency transform | Ratio | 8x8 low frequency transform | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Number of operations per pixel | Addition | 15.06 | 6.91 | 0.46 | 4.82 | 0.32 |
|  | Multiplication | 9.13 | 3.87 | 0.42 | 2.67 | 0.29 |
|  | Shift | 5.06 | 2.54 | 0.50 | 1.72 | 0.34 |
|  | Total | 29.25 | 13.32 | 0.46 | 9.21 | 0.31 |

# Estimation of dynamic range

Note that all approximated multiplication factors in the flow-graph are in the range [-1, 1] because these values are originally cosine and sine values. For more concrete and theoretical analysis of dynamic range, maximum row/column sum matrix norm and  are considered. It turns out that Chen’s and Loeffler’s 16-point factorization have the same maximum row sum norm = 11.25 and almost same maximum column sum norm =10.28 and 10.29, respectively.

The comparison of dynamic range estimation for 16x16 and 32x32 transforms is summarized in Table 5.

Table 5. Dynamic range estimation for 16-point and 32-point transforms

|  | Chen 16T | Loeffler 16T | Chen 32T | New integer 32T |
| --- | --- | --- | --- | --- |
| Max. column sum norm | 10.28 | 10.29 | 20.52 | 20.51 |
| Max. row sum norm | 11.25 | 11.25 | 22.63 | 22.63 |

Note that the increment of dynamic range after 1-D 16-point and 32-point transform is within 4 and 5 bit for both Chen’s and Loeffler’s transform, respectively.

Alternatively, dynamic range can be investigated by the method proposed in [5]. This method estimates the output range assuming the worst-case inputs. For example, when the input residuals are in [-256, 255], one of the worst-case input vectors to 1-D transform is (255, 255, ···, 255)T . As pointed out in [4], the maximum increase of dynamic range comes from the DC sub-band. In this case, the DC outputs of 2-D 16x16 and 32x32 transforms would be in the range [-218, 255*210] and [-222, 255*214], respectively. It should be noted that both large transforms are well performed within 32-bit arithmetic.

# Experimental results

At first, performance of 64x64 and 32x32 transforms are provided for sequences which are not used for TMuC evaluation. Table 6 summarizes the performance degradation when 64x64 and 32x32 transforms are disabled from reference configuration. HouseAndClouds and Geogia are 4Kx2K sequences provided by Samsung to response for call for test material. The others are common 1920x1080p sequences used by video experts. Only partial number of frames which is equivalent to 1 second duration is tested based on TMuC-0.5 for these experiments. According to the results, maximum 6.21% loss for RollingTomatoes (low delay) and minimum 0.87% loss for TableSetting (low delay) are observed when 64x64 transform is not used. Even larger losses are observed for both random access and low delay for all sequences when 64x64 and 32x32 transforms are not used together. Maximum loss is about 16.42% for RollingTomatoes (random access).

Table 6. Performance degradation when 64x64 and 32x32 transforms are disabled.

| Sequences | Random access |  | Low delay |  |
| --- | --- | --- | --- | --- |
|  | 64x64 | 32x32 + 64x64 | 64x64 | 32x32 + 64x64 |
| HouseAndClouds 4096x2048 | 3.50 | 10.64 | 3.68 | 9.29 |
| Geogia 4096x2048 | 1.19 | 4.44 | 1.26 | 4.38 |
| PlayingCards 1920x1080 | 2.31 | 5.99 | 2.55 | 7.03 |
| RollingTomatoes 1920x1080 | 5.93 | 16.42 | 6.21 | 15.46 |
| Sunflower 1920x1080 | 1.34 | 5.92 | 1.56 | 5.39 |
| TableSetting 1920x1080 | 1.73 | 6.68 | 0.87 | 5.56 |

These results clearly show that the 32x32 transform definitely provides good coding gain while the 64x64 transform also has reasonable application for those sequences. It implies that the current sequences used in the official test configuration may not have enough varieties, thus we have to consider adding more test sequences into the test set.

In the next experiment, new transforms based on Loeffler’s and simplified inverse quantization matrices are implemented in TMuC-0.5. As one can see in Table 7, the overall performance change from the TMuC is negligible with less number of operations and reduced memory requirement.

Table 7. Performance of proposed transform

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 95% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | -0.2 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 92% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.3 | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | 0.2 | 0.1 | 0.1 | 0.1 | 0.0 |
| Class E | 0.0 | -0.8 | 0.2 | 0.1 | 0.1 | 0.5 |
| All | 0.0 | -0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 92% |  |  |
| Dec Time[%] | 100% |  |  | 79% |  |  |

The partial transforms with low frequency was also tested on TMuC-0.7.3 with the same configuration. Two simulation results, up to 8x8 cases and up to 16x16 cases, were tested compared to TMuC anchor. It should be noted that the test case of 8x8 partial transforms is roughly same to the current TMuC design in SymbolMode 0 Error: Reference source not foundError: Reference source not found(VLC) case. However, the performance loss compared to the full-range transform is not trivial. Compared to it, the test case of 16x16 partial transforms shows the loss less than 0.5% compared to the full-range transform. From these results, we believe that 16x16 partial transform can be one consistent solution for all symbol coding modes. The extension of VLC mode for 16x16 transform coefficients is provided in another proposal, JCTVC-C210.

Table 8. Performance of 8x8 partial frequency transform

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.0 | 0.8 | 0.6 |
| Class B | 2.3 | 1.9 | 2.1 |
| Class C | 0.9 | 0.9 | 0.8 |
| Class D | 1.2 | 1.3 | 1.1 |
| Class E | 2.5 | 2.4 | 2.6 |
| All | 1.6 | 1.5 | 1.5 |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.1 | 1.1 |
| Class B | 1.9 | 2.1 | 2.2 |
| Class C | 1.1 | 0.8 | 0.6 |
| Class D | 0.8 | 0.8 | 0.7 |
| Class E |  |  |  |
| All | 1.2 | 1.3 | 1.2 |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 2.8 | 2.0 | 1.6 |
| Class C | 1.7 | 0.9 | 0.8 |
| Class D | 1.5 | 0.1 | 0.3 |
| Class E | 2.8 | 1.5 | 1.9 |
| All | 2.2 | 1.1 | 1.1 |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 99% |  |  |

Table 9. Performance of 16x16 partial frequency transform

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.3 | 0.3 |
| Class B | 0.7 | 1.0 | 1.0 |
| Class C | 0.2 | 0.4 | 0.3 |
| Class D | 0.3 | 0.4 | 0.3 |
| Class E | 0.6 | 1.4 | 1.2 |
| All | 0.4 | 0.8 | 0.6 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.4 | 0.2 |
| Class B | 0.6 | 1.0 | 0.8 |
| Class C | 0.3 | 0.1 | 0.2 |
| Class D | 0.2 | 0.4 | 0.3 |
| Class E |  |  |  |
| All | 0.3 | 0.5 | 0.5 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.8 | 0.5 | 0.3 |
| Class C | 0.4 | 0.1 | 0.2 |
| Class D | 0.2 | -0.1 | 0.0 |
| Class E | 0.5 | 0.5 | -0.2 |
| All | 0.5 | 0.3 | 0.1 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

A new fast algorithm for 32-point DCT based on Loeffler’s fast DCT is presented. Furthermore, 16-point and 32-point integer transforms are obtained by approximating all multiplication factors in the flow-graph to some dyadic rationales. The memory requirement is also reduced by introduction of simplified inverse quantization matrices. The complexity and dynamic range are also investigated and all transforms proposed here are well performed within 32-bit architecture.

Complexity can be reduced further with the use of partial transform proposed here. Experimental results on TMuC software show that proposed changes do not significantly change the performance while reducing the complexity.

We propose the new large transforms as replacements of current transforms in the TMuC, and their partial transform design up to 16x16 for further complexity reduction at the cost of small BD-rate increase. In addition, the 32x32 transform should be kept, and rather than removal of 64x64 transform, we suggest keeping it in the HEVC Test Model with the proposed complexity reduction method to maintain the consistency and the flexibility of the initial version of the Test Model.

# References

- K. McCann, W. J. Han, I. K. Kim, J. H. Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y. M. Hong, M. S. Cheon, and N. Shlyakhov,  Samsung's response to the call for proposals on video compression technology,  in 1st JCT-VC meeting , no. JCTVC-A124, Apr. 2010.

- W.-H. Chen, C. H. Smith, and S. C. Fralick, A fast computational algorithm for the discrete cosine transform, IEEE Trans. Commun., vol. COMM-25, pp. 1004-1009, Sep. 1977.

- C. Loeffler, A. Lightenberg, and G. Moschytz, Practical fast 1-D DCT algorithms with 11 multiplications,  Proc. IEEE ICASSP, vol. 2, pp. 988–991, Feb. 1989.

- J. Liang and T. D. Tran, Fast multiplierless approximations of the DCT with the lifting scheme, IEEE Trans. Signal Process., no. 12, pp. 3032-3044, Dec. 2001.

- X. Wan, Y. Wang, and W. Chen, Dynamic range analysis for the implementation of fast transform, IEEE Trans. Circuits Syst. Vid. Technol., vol. 5, pp. 178–180, Apr. 1995.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).