| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C210 WG11 Number: m18248 |
| --- | --- |

| Title: | Efficient coefficient coding method for large transform in VLC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Min-Su Cheon Il-Koo Kim | Email: | sunil.lee@samsung.com minsu.cheon@samsung.com ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents a new coefficient coding method for large transform in VLC mode. Tandberg, Ericsson, and Nokia’s coefficient coding method, which is currently implemented in the TMuC, has been extended for the efficient coding of the coefficients from large transforms including 16x16, 32x32, and 64x64. Experimental results show that the proposed method provides good gain for random access and intra only configurations without noticeable complexity increments.

- Introduction

The current coefficient coding method of the TMuC in VLC mode is based on Tandberg, Ericsson, and Nokia’s (TEN) method [1], [2]. Although TEN’s proposal also used large spatial transforms including 16x16, 32x32, and 64x64, only 8 low frequency basis vectors were used even when transforms larger than 8x8 were applied. Since at most 64 transform coefficients from 8x8 low frequency band needed to be coded, TEN’s VLC engine for coefficient coding was designed for size of up to 8x8 or 64 transform coefficients. Contrary to the TEN’s partial transform, the spatial transforms of the TMuC proposed by Samsung [3] exploit all the basis vectors, thus the transform coefficients from the whole frequency band need to be coded. However, since TEN’s VLC lacks an efficient method to code the coefficients from large transforms, only 64 low frequency coefficients from adaptively scanned 1D array (TU from intra PU) or 8x8 low frequency band (TU from inter PU) are coded in the current TMuC, which means that high frequency coefficients from large transforms are truncated without any rate-distortion consideration in VLC mode. Since this kind of truncation may lead to the decrease in the overall performance in VLC mode, especially when there exist many high frequency coefficients outside 8x8 low frequency band, an efficient coefficient coding method for large transforms in VLC mode should be proposed.

In this contribution, a new coefficient coding method in VLC mode for transforms larger than 8x8 is proposed. In the proposed method, first, TEN’s coefficient coding engine for 8x8 transform is extended to 16x16 transform by introducing new mapping tables for last non-zero coefficient position and run-level value. Then, the proposed method is further extended to 32x32 and 64x64 transforms while minimizing the increase of the number and the size of the mapping tables.

- Algorithm description

Basically, the proposed coefficient coding method is an extended version of the TEN’s method. For efficient coding of last non-zero coefficient and run-level value from large transforms, new mapping tables to assign VLC table indices and codewords have been introduced. In the following sections, the proposed method will be explained in detail.

## Overall description of the proposed coefficient coding method

First, TEN’s coefficient coding engine for 8x8 transform is extended to 16x16 transform by introducing new mapping tables for assigning VLC table indices and codewords to last non-zero coefficient (last_coeff) and run-level value from 16x16 transform. An adaptive sorting table is also used to code the last non-zero coefficient position (last_pos) of 16x16 transform. Then, the proposed method is further extended to 32x32 and 64x64. When the last_coeff of 32x32 and 64x64 transforms is coded, last_pos and a flag indicating whether level of last_coeff is greater than 1 (levelFlag) are not jointly coded, and no adaptive sorting table is used. This simplification can eliminate the long mapping tables for 32x32 and 64x64 transforms (2048 for 32x32 and 8192 for 64x64) and can also reduce the complexity of the adaptive sorting without performance loss. The zero run longer than 256, which can happen in 32x32 and 64x64 transforms, is simply coded by writing an escape codeword indicating that the run is longer than 256, and then directly writing the run subtracted by 256. This simplification can also eliminate the long mapping table required to assign the codeword to the run, which can be up to 4096 in 64x64 transform. The new thresholds for switching from run-mode to level-mode have been also introduced for the large transforms. Note that no VLC tables are added in the proposed method, that is, 11 VLC tables defined by TEN are selectively used to code the coefficients from the large transforms, considering their statistical characteristics. The overall procedure to encode the transform coefficients is summarized in the following.

  - Organize the quantized transform coefficients, each represented by a level and sign in a 1D array using the scanning method currently defined in the TMuC (Qualcomm’s adaptive scanning for TU from intra PU [4], zigzag for TU from inter PU).

  - Coefficients are encoded backwards along the 1D array from the highest frequency coefficient towards the DC coefficient.

  - Each event is encoded using “UVLC-like” tables which are exactly the same as TEN’s VLC tables. Different VLC tables are used for different events.

  - First, the last_coeff is coded:

    - If transform size <= 16x16, code the last_pos and levelFlag as a combined event.

    - Otherwise (if transform size >= 32x32), code last_pos and levelFlag separately as individual single events.

    - If level>1, code the value of level as a single event.

    - Code the sign as one bit.

  - If level=1, continue encoding non-zero levels in run-mode.

  - If level>1, switch to level-mode if any of the following is true:

    - Cumulative sum of all level of coefficients coded so far >1 (excluding the level of last_coeff), is bigger than a transform size-dependent threshold.

    - The position of the current level is bigger than a transform size-dependent threshold position

  - In run-mode, each non-zero coefficient is coded:

    - If run<=255, encode the preceding run and levelFlag as a combined event.

    - Otherwise (if run>255), encode the run of 256 and levelFlag as a combined event using the escape code (specifically, 511 and 512), then encode the remaining run (run-256).

    - Encode the sign and level of the non-zero coefficient (if levelFlag=1).

  - In level-mode, encode each coefficient (including zero coefficients) as level and sign.

For the combined last_coeff/levelFlag event, an adaptive sorting table is used only when the transform size is equal to or smaller than 16x16.

## Efficient coding of last non-zero coefficient in large transforms

Table 1 shows the mapping table of the VLC table indices used to code the last_pos in 16x16 transform. As shown in the table, different VLC table is used for the last_pos coding according to the color component, slice type, PU type, and the last_nonzero_pos_vlc_idx16 variable. The definition, initialization, and update of the variable last_nonzero_pos_vlc_idx16 are exactly the same as those of the variable last_nonzero_pos_vlc_idx given in Section 3.6.1.2 of [5]. Note that the VLC3 and VLC4 tables are used to code the last_pos in 32x32 and 64x64 transforms, respectively, without the adaptation.

Table 1. VLC table indices for last non-zero coefficient in 16x16 transform

| last_nonzero_pos_vlc_idx16 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chroma (U) | 10 | 10 | 10 | 10 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 |
| Chroma (V) | 10 | 10 | 10 | 10 | 10 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 |
| Intra Y | 10 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 |
| Inter Y in P slice | 10 | 10 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 4 |
| Inter Y in B slice | 10 | 10 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 4 |

For the combined coding of last_pos and levelFlag using the adaptive sorting in 16x16 transform, the codeword mapping table with 512 entries is introduced. The table is initialized with the values from 0 to 511, and adaptively updated as proposed in [1]. Note that the initial value of the table can be further optimized by considering the statistical characteristics of the coefficients from 16x16 transforms. Note that no codeword mapping tables are required for 32x32 and 64x64 transforms, since the adaptive sorting is not exploited for those transform sizes.

Table 2. VLC table indices for isLevelOne_run in large transforms (16x16, 32x32, and 64x64)

| Max run | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VLC table index | 8 | 1 | 1 | 1 | 1 | 1 | 1 | 5 | 5 | 5 | 5 | 2 | 2 | 2 | 2 | 6 |
| Max run | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| VLC table index | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 3 | 3 |
| Max run | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| VLC table index | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Max run | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| VLC table index | 3 | 7 | 7 | 7 | 7 | 4 | 7 | 7 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

## Efficient run-mode coding in large transforms

Table 2 shows the mapping table of the VLC table indices used to code the syntax element isLevelOne_run in large transforms including 16x16, 32x32, and 64x64. As defined in Section 3.6.1.4 of [5], the syntax element isLevelOne_run describes the number of zero coefficients between the current nonzero coefficient and the next non-zero coefficient in the inverse scan order and levelID which indicates whether |level| of that coefficient is 1 or >1. As shown in the table, the VLC table index is chosen according to the max run which is given as (k-1) where k is the position of the current non-zero coefficient. Note that, when max run is greater than 63, VLC3 is used to code the isLevelOne_run.

The codeword mapping table for the syntax element isLevelOne_run in 16x16 transform is introduced in the proposed method. The same mapping table is also used for 32x32 and 64x64 transforms when run is  shorter than 256. When run is longer than 256 in 32x32 and 64x64 transforms, first an escape code is coded (511 for the run with level equal to 1, 512 for the run with level greater than 1), then the remaining run ( = run - 256) is coded. This simplification can eliminate the long codeword mapping table required to assign the codeword to the isLevelOne_run, which can be up to 4096 in 64x64 transform.

- Experimental results

The test condition is the same as TE12 test condition for low-complexity configuration [6]. Note that the detailed experimental results are available in the attached Excel file.

## Performance of the full coefficient coding

Table 3 summarizes the BD-rate savings provided by the full coefficient coding using the proposed method, where all the coefficients from the whole frequency band are coded. Average BD-rate savings are 1.6% and 1.1% in intra only and random access configurations, respectively, while the BD-rate loss of 0.4% is observed in low delay configuration. As shown in the table, the noticeable performance improvement is achieved in intra only and random access configurations, where large transforms are frequently selected and there exist many high frequency coefficients. On the other hand, the proposed method does not provide any BD-rate saving in low delay configuration, where large transform is not selected often and most TUs include only few non-zero coefficients in the low frequency band.

Table3. BD-rate saving of the full coefficient coding using the proposed method

| Config. Class | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.3 | -1.2 | -1.5 | -0.9 | 0.1 | 0.0 |  |  |  |
| Class B | -2.6 | -1.9 | -1.6 | -2.0 | 0.2 | 0.8 | 0.4 | 2.2 | 2.7 |
| Class C | -0.8 | -0.6 | -0.8 | -0.7 | 0.0 | -0.1 | 0.1 | 0.6 | 0.8 |
| Class D | -0.6 | -0.7 | -0.9 | -0.3 | 0.4 | 0.1 | 0.2 | 0.9 | 0.3 |
| Class E | -2.0 | -2.2 | -2.2 |  |  |  | 1.2 | 4.4 | 3.2 |
| Average | -1.6 | -1.3 | -1.3 | -1.1 | 0.2 | 0.3 | 0.4 | 1.9 | 1.7 |
| Enc Time | 129% |  |  | 111% |  |  | 104% |  |  |
| Dec Time | 94% |  |  | 93% |  |  | 80% |  |  |

The proposed method is also effective for the sequences in class B, where 2.6% and 2.0% BD-rate savings are achieved in intra only and random access configurations, respectively. These results also show that the proposed method improves the coding performance when there exist many TUs larger than 8x8 and many high frequency coefficients. The complexity of the proposed method measured in terms of encoding and decoding time shows that the proposed method slightly increases the encoder complexity, especially in the intra only configuration; however, the decoder complexity is even decreased since too frequent TU splitting is prevented.

## Performance of the 16x16 partial coefficient coding

The performance of the proposed method is also evaluated when only the transform coefficients from 16x16 low frequency band are coded in 32x32 and 64x64 transforms. Table 4 summarizes the BD-rate savings provided by the 16x16 partial coefficient coding using the proposed method. Average BD-rate savings are 1.6%, 0.8%, and -0.3% in intra only, random access, and low delay configurations, respectively. As the full coefficient coding, the noticeable performance improvement is observed in intra only and random access configurations. Although the performance is slightly degraded as compared with that of the full coefficient coding, the gap is not so large, which means that most coding gain of the proposed method comes from the extension of the coefficient coding engine to 16x16 transform.

Table 4. BD-rate saving of the 16x16 partial coefficient coding using the proposed method

| Config. Class | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.5 | -1.4 | -1.8 | -0.9 | -0.3 | -0.6 |  |  |  |
| Class B | -2.4 | -1.7 | -1.9 | -1.5 | -0.4 | -0.5 | 0.1 | 1.3 | 1.0 |
| Class C | -0.8 | -0.6 | -0.7 | -0.5 | -0.1 | -0.2 | 0.1 | 0.5 | 0.7 |
| Class D | -0.6 | -0.6 | -0.8 | -0.3 | -0.1 | -0.2 | 0.1 | 0.4 | 0.1 |
| Class E | -1.8 | -2.0 | -1.8 |  |  |  | 1.1 | 2.1 | 2.0 |
| Average | -1.6 | -1.2 | -1.4 | -0.8 | -0.3 | -0.3 | 0.3 | 1.0 | 0.9 |
| Enc Time | 106% |  |  | 96% |  |  | 94% |  |  |
| Dec Time | 82% |  |  | 75% |  |  | 68% |  |  |

The complexity of the 16x16 partial coefficient coding using the proposed method is virtually similar to the TMuC anchor while meaningful complexity reduction is observed at the decoder.

- Conclusion

In this proposal, a new coefficient coding method for large transforms in VLC mode is proposed, and its performance is presented. As provided in Section 3, the proposed method provides noticeable performance improvement in intra only and random access configurations without complexity increase, while introducing small loss in low delay configuration.

The experimental results show that the proposed method can effectively improve the coding performance when large transforms with many high frequency coefficients are selected, e.g. for the sequences in class B. We propose to consider the proposed method in the Test Model (TM) since the proposed method is a straightforward extension of the existing coefficient coding method in VLC mode and provides the coding gain without noticeable complexity increase.

- References

- K. Ugur, K. R. Andersson, and A. Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson,” JCTVC-A119, Dresden, Germany, April 2010.

- JCT-VC, “Test model under consideration”, JCTVC-A205, Dresden, Germany, April 2010.

- Ken McCann, Woo-Jin Han, and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- M.Karczewicz, P.Chen, R.Joshi, X.Wang, W-J.Chien, R.Panchal, “Video coding technology proposal by Qualcomm Inc,” JCTVC-A121, Dresden, Germany, April 2010.

- K. Ugur, K. R. Andersson, and A. Fuldseth, “Appendix to Description of video coding technology proposal by Tandberg Nokia Ericsson,” JCTVC-A119_ Appendix_Decoder_Description, Dresden, Germany, April 2010.

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, Geneva, CH, July 2010

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).