| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C211 WG11 Number: m18249 |
| --- | --- |

| Title: | Inner block oriented ALF processing for memory & bandwidth reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yumi Sohn Kyohyuk Lee Baekeun Lee Il Koo Kim | Email: | yumi.sohn@samsung.com kyohyuk.lee@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This proposal shows adaptive loop filter with low memory, bandwidth and computational complexity requirements. ALF in TMuC requires spatial neighboring reconstructed data for CU based ALF processing. This document proposes applying filtering only to restricted inner LCU area where there is no need to use out of LCU data for filtering. This approach results in marginal performance degradation with memory, bandwidth and computational complexity reduction. The experimental results show that only 1.0% performance degradation in low delay configuration and 0.8% performance degradation in random access configuration, respectively. In expense of marginal performance degradation, we can reduce about 24Kbytes external DDR memory and 0.77Kbytes internal SRAM. And we can reduce memory bandwidth about 5.98MHz in case of 4Kx2K@30p video sequence and 64 bit BUS. Furthermore we can reduce computational complexity for ALF processing itself maximum to 24% too.

- Introduction

Current ALF implementation is applied after deblock filter is done for whole frame. To minimize decoding cycle of HEVC in HW or multi-core platform, LCU synchronous decoding process is essential. For LCU synchronous processing, spatial neighboring reconstructed data is required, since outside data of LCU is used for input of ALF process. For LCU synchronous processing of the ALF, decoder requires reconstructed above and left pixel data of current LCU. When we decode 4Kx2K video sequence, we need additional 24Kbytes (8*4096*4 + 8*2048*2*2) DDR memory for the bottom side of horizontal LCU stride and 0.77Kbytes ((64*4+68*4)*8 + (32*2+34*2)*8*2 ) internal SRAM for the right side of previous LCU & bottom side of upper LCU to process the current LCU. For the exploitation of the above pixel data, decoder has to access external DDR memory for data upload. So, additional memory bandwidth, 5.98 MHz in case of 4Kx2K@30p video and 64 bit BUS, occurs too. In this proposal, out of LCU boundary data is not used for ALF processing of current LCU. This approach reduces external DDR and internal SRAM memory requirement, memory bandwidth and computational complexity with marginal performance degradation.

- Proposed Method

LCU synchronous decoding process is required for efficient implementation and minimization of decoding cycles. Current decoding process requires outside region of current LCU for applying ALF in whole LCU region, since 9, 7 or 5 tap is utilized in LCU boundary region. Storing of spatial neighboring reconstructed pixel data is necessary and it causes additional external DDR and internal SRAM memory requirement. If there is no enough internal memory to keep the spatial neighbor data, which is common in most of platforms, the additional memory bandwidth for data transmission is needed. To reduce memory and bandwidth requirement, we propose to restrict filtering region only to reduced inner area of LCU. To explain the proposed processing, let’s define LCU boundary as followings. Fig 1 shows definition of LCU boundary (It is denoted by grey region).

Fig 1. The definition of LCU boundary

Then proposed ALF processing is as followings.

- If current CU includes LCU boundary, then apply ALF in non boundary LCU region

  - Input data is restricted on in LCU data. That is to say, outside of LCU region is not used for ALF input. Then output size is restricted up to LCU size – (filter_tap-1)/2.

  - The ALF output size depends on filter tap size and CU adaptive ALF flag.

    - Maximum size is 56x56 in 9x9 filter , 58x58 in 7x7 filter and 60x60 in 5x5 filter

  - Non-ALF output region in LCU does not change

  - Merge with ALF output region and ALF boundary region (No ALF filtering)

- Otherwise, apply ALF processing same with the current TMuC

Fig 2 shows current TMuC ALF processing region and Fig 3 shows proposed reduced ALF processing region. (Yellow marked region indicated output of ALF process).

Fig 2. ALF processing region in TMuC

Fig 3. Proposed reduced ALF region

# Simulation result

For comparison of inner block ALF processing approaches, TMuC 0.7 with default configuration plays role of anchor. The difference condition is like below:

Anchor :

- VAR_SIZE macro is 3 which mean using 7x7 regions to calculate pixel variance.

Inner block oriented ALF process method :

- Filtering region is restricted only to the reduced inner block of LCU boundary.

- VAR_SIZE macro is 1 which means using 3x3 regions to calculating pixel variance.

Following table represents total average BD-rate saving. Detailed test results are available in attached excel file. To estimate the performance of new method, we use ALF_OFF results which were released in TE12-ALF_Qualcomm. It shows that the large portion of ALF performance is maintained without boundary filtering. Performance degradations were under 1% in both cases of random access and low delay.

|  | ALF_OFF (TE12-ALF-Qualcomm) |  | Proposed method |  |
| --- | --- | --- | --- | --- |
| Class | HE Random Access | HE Low Delay | HE Random Access | HE Low Delay |
| Class A | -4.9 % |  | -1.0 % |  |
| Class B | -5.8 % | -6.0 % | -0.9 % | -0.2 % |
| Class C | -2.6 % | -4.1 % | -0.6 % | -0.9 % |
| Class D | -1.7 % | -2.7 % | -0.6 % | -0.9 % |
| Class E |  | -8.5 % |  | -2.5 % |
| All | -3.7 % | -5.2 % | -0.8 % | -1.0 % |

Table1. Performance degradation of the proposed method

# Conclusion

To minimize decoding cycle of HEVC, we have to adopt LCU synchronous ALF processing. In this document, we propose inner block oriented ALF processing method that does not need additional 24 Kbytes external DDR and 0.77 Kbytes internal SRAM memory. And the proposal can reduce 5.98 MHz memory bandwidth in case of 4Kx2K@30p video and 64 bit BUS platform environment. Furthermore we can reduce 24% computational complexity of ALF filtering itself. Performance degradation of the proposal was negligible under TMuC test conditions, random access and low delay. So, we propose JCT-VC to adopt inner block oriented ALF processing method in HEVC Test Model (TM).

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).