| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C212 WG11 Number: m18250 |
| --- | --- |

| Title: | Cross-verification of JCTVC-C086: experimental results of ALF on low complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Woo-Jin Han | Email: | wjhan.han@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution is the cross-verification report of JCTVC-C086: experimental results of ALF on low complexity. More specifically, two low-complexity inter configurations using 6-tap DCT-IF and ALF together were tested and compared to the results of JCTVC-C086.

- Cross-verification

TMuC 0.7.3 software has been used without any changes. For the configuration parameter, the following values were specified to enable 6-tap DCT-IF and ALF additionally on top of the TE12 default parameters.

Configuration parameters

InterpFilterType         	: 0           # 0: DCT-IF, 1: 4-tap MOMS, 2: 6-tap MOMS, 3: DIF

DIFTap                  	: 6           # Number of filter taps for DCT-IF (4, 6, 8, 10, 12)

ALF                    	: 1           # Adaptive loop filter  (0: OFF, 1: ON)

Cross-check results

|  |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -4.9 |  | -6.9 | -6.5 |
| Class B | -6.0 |  | -10.8 | -10.4 |
| Class C | -7.1 |  | -7.9 | -9.8 |
| Class D | -11.4 |  | -11.4 | -12.1 |
| Class E |  |  |  |  |
| All | -7.6 |  | -9.6 | -10.1 |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low delay LoCo |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A |  |  |  |  |
| Class B | -5.8 |  | -10.7 | -11.7 |
| Class C | -7.9 |  | -10.8 | -12.5 |
| Class D | -10.9 |  | -14.8 | -15.5 |
| Class E | -3.2 |  | -12.9 | -15.0 |
| All | -7.1 |  | -12.2 | -13.4 |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 90% |  |  |  |

- Conclusions

All results were compared to those of JCTVC-C086 and there are no mismatches except negligible differences. During the cross-check activity, one interesting point is that this configuration does not increase the running time compared to the anchor whereas JCTVC-C086 reports the 20% increase of decoding time. It may be due to the heterogeneous arrangement of our test server configurations; however, it is rather clear that this configuration does not increase the running time significantly.