| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C214_r1 WG11 Number: m18254 |
| --- | --- |

| Title: | TE12.5: Results for Adaptive loop filter using prediction and residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report and Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a part of the TE12: It presents the results of the Adaptive loop filter using the prediction and residual. An average Y-BD-rate gain of 1.4% for Intra High Efficiency, 1.3% for Random Access High Efficiency, 0.8% for Low Delay High Efficiency,  x.x % for Intra Low Complexity, 5.9 % for Random Access Low Complexity, and 5.2 % for Low Delay Low Complexity compared to the official anchors can be achieved. It is proposed to include it in the TM.

# Introduction

The Adaptive loop filter using the prediction and residual was first presented as part of the proposal JCTVC-A114 [1] in Dresden. At the Dresden meeting, it was decided to include it into the TMuC.

# Adaptive loop filter using the prediction and residual

The adaptive loop filter using the prediction and residual aims at reducing quantization noise. Based on the prediction signals, the quantized prediction errore', and the reconstructed filter signals'', the following linear filter operations are performed:

s'''=i=1Mwi⋅si''+i=1NwM+i⋅si+i=1OwM+N+i⋅ei'+c

All filter coefficientswi, withi=1,…,M+N+O as well as the offset c are estimated once per frame by minimizing the mean squared error between the original input signalsand the filtered signals''':

D=Es−s'''2→min

The estimated filter coefficients are coded and transmitted to the decoder. The filter sizes M,N,O as well as the quantization step sizes used in the filter coefficients coding step are adaptive and determined by minimization of the Lagrangian costs of overall data rate R and mean squared errorD:

R⋅λ+D→min

# Software integration

The adaptive loop filter using the prediction and residual was integrated into the version 0.7 of the TMuC software. It can be enabled by setting the macro WIENER_3_INPUT  to 1. The corresponding software is publicly available at:

http://hevc.kw.bbc.co.uk/trac/browser/branches/0.7-panasonic

In order to guarantee a defined software version, the integration branch was write protected by the Chairmen of the Software Development AhG.

Currently, this software is being merged into the trunk and harmonized with the tools also being merged into the trunk in parallel. For this merge, the integration branch

http://hevc.kw.bbc.co.uk/trac/browser/branches/0.8-panasonic

is used.

The software integration includes the loop filter of JCTVC-A114 [1] as well as a harmonized version with the filter of Qualcomm, which is the one enabled in the common test conditions [5] by setting the macro QC_ALF to 1. In the following, the loop filter of Qualcomm is also denoted as QC_ALF filter. Our integration allows the following macro configurations:

- Macro QC_ALF=1  and macro WIENER_3_INPUT=1 :
Adaptive loop filter using the prediction and residual within the QC_ALF filter

- Macro QC_ALF=1  and macro WIENER_3_INPUT=0:
Only the QC_ALF filter is used

- Macro QC_ALF=0  and macro WIENER_3_INPUT=1:
Adaptive loop filter using the prediction and residual as originally implemented in JCTVC-A114

# Experiments

Software and configuration

Under consultation with the Chairmen of the AhGs of Software Development and TE12, experiments were performed using the defined software of the integration branch:

http://hevc.kw.bbc.co.uk/trac/browser/branches/0.7-panasonic

The settings for the experiment are:

- Macro QC_ALF=1 and macro WIENER_3_INPUT=1 :
Adaptive loop filter using the prediction and residual within the QC_ALF filter

- Configuration parameter ALF=1

Tested are all six test cases, which is Intra, Random access, and Low delay, each in high efficiency and low complexity operation mode.

The configuration files of the SVN repository, which equal to those defined in [5], were used for the experiments.

The simulations were run on a Linux cluster, which includes processors of the various types, such as:

- Intel Xeon E5472  @ 3.00GHz

- Intel Core i7 970 @ 3.20GHz

- Intel Core(TM)2 CPU 6600  @ 2.40GHz

- Intel Pentium D CPU 3.20GHz

The fast cpus were assigned with priority to the high efficiency operation mode. The slower cpus were mainly assigned to the low complexity operation mode.

Experimental results (comparison to the official anchors)

As a reference, the official anchors available at

ftp://ftp.kw.bbc.co.uk/hevc/tmuc-0.7-anchors

have been used.

For the high efficiency operation mode, the anchors use the QC_ALF filter enabled. The results reflect the gain of the adaptive loop filter using the prediction and residual compared to the loop filter QC_ALF.

For the low complexity operation mode, the anchors do not use any loop filter, disabled by setting the parameter ALF=0. The results reflect the gain of the Adaptive loop filter using the prediction and residual within the QC_ALF filter compared to no loop filter.

The following tables show the results of the experiment.

The attached excel file “JCTVC-C214_TE12_3-Input-ALF_PANASONIC.xls” includes all further details.

Unfortunately, a small implementation bug caused encoder-decoder mismatches for some frames in software of the branch 0.7-panasonic. This bug had the effect that some coefficients were coded with a wrong code words, each of much too high length. This bug is already fixed in the version in the branch 0.8-panasonic.

Additional experimental results (comparison to the loop filter QC_ALF for the Low Complexity operation mode)

As an additional reference for the Low Complexity operation mode, the coding results of the TE12 experiment for the enabled QC_ALF filter, available at

ftp://ftp.kw.bbc.co.uk/hevc/results/201007-geneva-te12/TE12-ALF-Intel.xls ,

have been used for additional comparisons. The following tables show the results compared to this additional reference.

The attached excel file “JCTVC-C214_Additional_Comparison.xls” includes all further details.

Sequence dependent Y-BD-rate gains

Using our simulation results and the results of the QC_ALF  filter, available at

ftp://ftp.kw.bbc.co.uk/hevc/results/201007-geneva-te12/TE12-ALF-Intel.xls ,

sequence dependent Y-BD-rate gain are calculated for the QC_ALF filter (macro QC_ALF=1, macro WIENER_3_INPUT=0, configuration parameter ALF=1) and for the adaptive loop filter using the prediction and residual used within the QC_ALF filter (macro QC_ALF=1, macro WIENER_3_INPUT=1, configuration parameter ALF=1). The reference applies no loop filter (configuration parameter ALF=0). The results for the six test cases are shown in the following Figures. All negative numbers show increased coding efficiency compared to the reference, which applies no loop filter.

From the experimental results shown above, it can be seen that the adaptive loop filter using the prediction and residual integrated into the QC_ALF filter increases the coding efficiency for each single test sequence in each of the tested configurations. In the High Efficiency operation mode, the harmonized loop filter shows an average Y-BD-rate gain of 3.5% for Intra, 4.7% for Random Access, and 5.6% for Low Delay compared to using no loop filter. In the Low Complexity operation mode, it shows an average Y-BD-rate gain of x.x % for Intra, 5.9 % for Random Access, and 5.2 % for Low Delay compared to using no loop filter.

# Conclusion

The Adaptive-loop filter using prediction and residual was first proposed in Dresden as part of the proposal JCTVC- A114 [1] and was introduced in the TMuC [2]. The current implementation of the Adaptive loop filter using prediction and residual in the current TMuC software using the configuration proposed by TE12 [4] shows an average Y-BD-rate gain of 1.4% for Intra High Efficiency, 1.3% for Random Access High Efficiency, 0.8% for Low Delay High Efficiency,  x.x% for Intra Low Complexity, 5.9 % for Random Access Low Complexity, 5.2 % for Low Delay Low Complexity compared to the official anchors.

In the High Efficiency operation mode, the harmonized loop filter shows an average Y-BD-rate gain of 3.5% for Intra, 4.7% for Random Access, and 5.6% for Low Delay compared to using no loop filter. In the Low Complexity operation mode, it shows an average Y-BD-rate gain of x.x % for Intra, 5.9 % for Random Access, and 5.2 % for Low Delay compared to using no loop filter.

Since the Adaptive loop filter using prediction and residual increases the coding efficiency consistently for each single test sequences in each of the tested configuration, we see a value in this technique for the standardization and therefore propose to include it in the TM.

# Acknowledgement

I would like to thank Qualcomm for supporting the harmonization work of the adaptive loop filter using prediction and residual and the QC_ALF. Furthermore, I would like to thank Hisao Sasai and Toshiyasu Sugio for their very valuable support in generating the simulation results.

# References

- I. Amonou et. al., Description of video coding technology proposal by France Telecom, NTT, NTT DOCOMO, Panasonic and Technicolor, JVCVC-A114, Dresden, Germany, 15-23 April 2010

- JCTVC-B205, Test Model under Consideration, Geneva, Switzerland, 21-28 July 2010

- Frank Bossen, Kemal Ugur, David Flynn, Heiko Schwarz and Peisong Chen, AHG report: Software development and TMuC software technical evaluation, JCTVC-B003, Geneva, Switzerland, 21-28 July 2010

- Ken McCann, Tool Experiment 12: Evaluation of TMuC Tools, JCTVC-B312, Geneva, Switzerland, 21-28 July 2010

- Frank Bossen, Common test conditions and software reference configurations, JCTVC-B300, Geneva, Switzerland, 21-28 July 2010

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).