| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C218 WG11 Number: m18258 |
| --- | --- |

| Title: | Encoding complexity reduction for Intra prediction by Disabling NxN Partition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim Yongjoon Jeon Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6272-9179 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes simplification of intra prediction process to reduce encoding complexity. Each PU consists of only one partition except PUs corresponding to minimum CU. It shows approximately 4 % encoding time reduction with 0.21% loss in BD-rate. If ROT and CIP are not applied, the reduction ratio is around 40% with 0.36% loss compared to the anchor in which also ROT and CIP are not applied.

- Introduction

In the description of TMuC [1], a Coding Unit(CU) splits into four smaller CUs recursively. When a CU is not allowed to split any more the CU is considered as Prediction Unit(PU). A PU can be split into partitions. For each PU, a signal indicates the type of partition is encoded.

For intra prediction, there are two types of partitioning; 2Nx2N and NxN. The partition type 2Nx2N indicates that the PU is not split and the partition type NxN indicates that the PU is split into four partitions. For each partition, the RD cost of each intra prediction mode is calculated to decide the most efficient intra prediction mode. Since a CU can be split into four smaller CUs, a partition of a CU whose partition type is NxN has same size of a partition of one size smaller CU whose partition type is 2Nx2N.

# Proposed Methods

In the proposed algorithm, a PU does not split into partitions except PUs whose size is minimum CU size. Therefore the indication flag is encoded only when a PU has minimum CU size. The syntax table should be changed as shaded in the Table 1.

Table 1 Modified syntax for the proposed method

| prediction_unit( x0, y0, currPredUnitSize ) { | C |
| --- | --- |
| … |  |
| if( PredMode == MODE_INTRA ) { |  |
| … | 2 |
| if(entropy_coding_mode_flag) |  |
| if(currCodingUnitSize== MinCodingUnitSize) |  |
| intra_split_flag | 2 |
| else intra_split_flag=0 |  |
| combined_intra_pred_flag | 2 |
| for( i = 0; i < ( intra_split_flag ? 4 : 1 ); i++ ) { |  |
| prev_intra_luma_pred_ flag | 2 |
| if( !prev_intra_pred_luma_flag ) |  |
| rem_intra_luma_pred_mode | 2 |
| if( adaptive_intra_smoothing_flag ) |  |
| intra_luma_filt_flag | 2 |
| } |  |
| } |  |
| if( chroma_format_idc != 0 ) |  |
| intra_chroma_pred_mode | 2 |
| } |  |

# Experimental results

Table 2 and Table 3 show the experimental results under the condition of Intra High Efficiency [2] with the configuration file provided by TMuC. Table 2 shows encoding complexity reduction in time and Table 3 shows BD rate decrease caused by disabling NxN partition.

Since the time consumed for Rotation Transform is huge [3] as reported for TE12 [4], the effect of disabling NxN partition is attenuated. Table 4 and Table 5 show the experimental results when ROT and CIP is disabled. It takes 63.88% of the time that anchor takes with only 0.34% BD-rate loss.

Table  2 Encoding Time of the proposed methods

|  |  | Encoding time ratio(%) | average |
| --- | --- | --- | --- |
| classC | BasketballDrill | 100.15 | 95.88 |
|  | BQMall | 91.82 |  |
|  | PartyScene | 93.61 |  |
|  | RaceHorses | 97.94 |  |
| classD | BasketballPass | 93.61 | 96.26 |
|  | BQSquare | 98.35 |  |
|  | BlowingBubbles | 97.94 |  |
|  | RaceHorses | 95.15 |  |
| average |  |  | 96.07 |

Table  3 BD rates of the proposed methods

|  |  | BD-rate | average |
| --- | --- | --- | --- |
| classB | Kimono | 0.11 | 0.14 |
|  | ParkScene | 0.07 |  |
|  | Cactus | 0.18 |  |
|  | BasketballDrive | 0.19 |  |
|  | BQTerrace | 0.14 |  |
| classC | BasketballDrill | 0.59 | 0.27 |
|  | BQMall | 0.14 |  |
|  | PartyScene | 0.08 |  |
|  | RaceHorses |  |  |
| classD | BasketballPass | 0.09 | 0.09 |
|  | BQSquare | 0.07 |  |
|  | BlowingBubbles | 0.13 |  |
|  | RaceHorses | 0.06 |  |
| classE | Vidyo1 | 0.52 | 0.36 |
|  | Vidyo3 | 0.28 |  |
|  | Vidyo4 | 0.28 |  |
| average |  |  | 0.21 |

Table  4 Encoding time of the proposed method when both CIP and ROT are both off

|  |  | Encoding time ratio (%) | average |
| --- | --- | --- | --- |
| classA | Traffic_2560x1600_30_crop | 63.91 | 63.81 |
|  | PeopleOnStreet_2560x1600_30_crop | 63.70 |  |
| classB | Kimono | 64.46 | 64.16 |
|  | ParkScene | 64.49 |  |
|  | Cactus | 64.23 |  |
|  | BasketballDrive | 63.84 |  |
|  | BQTerrace | 63.79 |  |
| classC | BasketballDrill | 63.66 | 63.54 |
|  | BQMall | 63.67 |  |
|  | PartyScene | 63.20 |  |
|  | RaceHorses | 63.64 |  |
| classD | BasketballPass | 64.33 | 63.95 |
|  | BQSquare | 63.53 |  |
|  | BlowingBubbles | 63.92 |  |
|  | RaceHorses | 64.04 |  |
| classE | Vidyo1 | 63.91 | 63.93 |
|  | Vidyo3 | 63.82 |  |
|  | Vidyo4 | 64.05 |  |
| average |  |  | 63.88 |

Table  5 BD rates of the proposed method when both CIP and ROT are both off

|  |  | BD-rate | average |
| --- | --- | --- | --- |
| classA | Traffic_2560x1600_30_crop | 0.36 | 0.39 |
|  | PeopleOnStreet_2560x1600_30_crop | 0.41 |  |
| classB | Kimono | 0.24 | 0.29 |
|  | ParkScene | 0.27 |  |
|  | Cactus | 0.34 |  |
|  | BasketballDrive | 0.37 |  |
|  | BQTerrace | 0.26 |  |
| classC | BasketballDrill | 0.78 | 0.39 |
|  | BQMall | 0.27 |  |
|  | PartyScene | 0.11 |  |
|  | RaceHorses | 0.22 |  |
| classD | BasketballPass | 0.20 | 0.22 |
|  | BQSquare | 0.09 |  |
|  | BlowingBubbles | 0.32 |  |
|  | RaceHorses | 0.27 |  |
| classE | Vidyo1 | 0.68 | 0.51 |
|  | Vidyo3 | 0.40 |  |
|  | Vidyo4 | 0.46 |  |
| average |  |  | 0.36 |

# Patent rights declarations

LG electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Test Model under Consideration”, JCTVC-B305, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010

- E. Alshina, A. Alshin, W.-J. Han, R. Joshi , M. Coban, “TE12.4: Experimental results of MDDT and ROT by Samsung and Qualcomm “, JCTVC-C202(m18240), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, GuangZhou, China, Oct. 2010

- Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools Test Model under Consideration”, JCTVC-B312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008