| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C220 WG11 Number: m18260 |
| --- | --- |

| Title: | Summary report for TE3 on inter prediction in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andreas Krutz and Thomas Sikora Technische Universität Berlin, Sekr. E-N 1 Einsteinufer 17, 10587 Berlin, Germany | Email: | krutz@nue.tu-berlin.de sikora@nue.tu-berlin.de |
| Source: | Technische Universität Berlin |  |  |

_____________________________

# Abstract

This document summarizes the activity of TE3 – Inter prediction in HEVC [1]. The TE was separated into four subtests that are Subtest 1 – Warped motion compensated and second order prediction, Subtest 2 – Flexible motion partitioning, Subtest 3 – Multi hypothesis inter prediction, and Subtest 4 - Improved inter prediction with enhanced MC filter. The results that have been generated so far are summarized in this document. Due to the short period of time, integration in the new software and running the experiments was a big task. Therefore, not all participants who intended to actively contribute to this TE at the last meeting have shown results for their proposed tools.

# TE Documents

| Participant | Document | Title | Type |
| --- | --- | --- | --- |
| LGE | JCTVC-C033/ m18054 | TE3: Motion compensation with adaptive warped reference | Proposal, verified by TUB (JCTVC-C282/m18325) |
| TUB | JCTVC-C282/ m18325 | TE3 subtest 1: Cross-check of results from LG | Verification |
| Technicolor | JCTVC-C031/ m18052 JCTVC-C034/ m18055 | TE3 subtest 2: Report on simplified geometry block partitioning TE3 subtest 2: Cross-check of results from Huawei | Proposal, verified by Huawei (JCTVC-C170/m18199) Verification |
| Huawei & Hisilicon | JCTVC-C099/ m18122 JCTVC-C170/ m18199 | TE3.2: Huawei & HiSilicon report on flexible motion partitioning coding TE3.2: Report on cross-verification of simplified geometry block partitioning from Technicolor | Proposal, verified by Technicolor (JCTVC-C034/m18055) Verification |
| Mitsubishi Electric | JCTVC-C233/ m18272 | TE3 subtest 3: Local intensity compensation (LIC) for inter prediction | Proposal, verified by Poznan University (JCTVC-C240/m18272) |
| Poznan Univ. | JCTVC-C240/ m18281 | TE3: Cross-check results of local intensity compensation tool from Mitsubishi Electric | Verification |
| Sony | JCTVC-C164/ m18190 JCTVC-C165/ m18191 | TE3 subset4: Results on Bi/Single filter switching in FIF TE3 subset4: Cross verification on high accuracy interpolation filter | Proposal, verified by Toshiba Verification |
| Toshiba | JCTVC-C078/ m18101 JCTVC-C247/ m18288 | TE3 subtest 4: High accuracy interpolation filter (HAIF) TE3 subset 4: Cross-verification on bi/single filter switching in FIF | Proposal, verified by Sony (JCTVC-C165/m18191) Verification |
| Ghent Univ. | JCTVC-C236/ m18277 | Report of complexity analysis of geometric partitioning | Report |

# Subtest 1: Warped Motion Compensated Prediction

## Motion compensation with adaptive warped reference [2]

This proposal presents the experimental results of TE3 subtest 1, which aims to improve temporal prediction performance by using warping motion. This tool uses the additional reference picture which is warped version of the original reference picture based on homography relation between the current picture and the reference picture. Experimental result shows 3.1% overall bitrate saving under ‘low delay & high efficiency’ condition and 3.5% overall bitrate saving under ‘random access & high efficiency’ condition compared with TMuC 0.7.3 (refer to Table 1 and 2). Especially for some sequence having warping motion, bitsaving is increased up to 41.1%.

Table 1. BD rate (%) reduction of low delay & high efficiency condition

| Class | seq. | BD-rate(%) |
| --- | --- | --- |
| B | BasketballDrive | 0.1 |
|  | BQTerrace | 0.7 |
|  | Cactus | -3.1 |
|  | Kimono | 0.1 |
|  | ParkScene | 0.2 |
|  | Bluesky | -8.4 |
|  | Station2 | -41.1 |
| Class B Avg. |  | -7.4 |
| C | BasketballDrill | -0.2 |
|  | BQMall | 0.6 |
|  | PartyScene | -0.1 |
|  | RaceHorses | 0.5 |
|  | Flowervase | -0.7 |
| Class C Avg. |  | 0.0 |
| D | BasketballPass | 0.6 |
|  | BlowingBubbles | 0.2 |
|  | BQSquare | 0.3 |
|  | RaceHorses | 0.8 |
|  | Flowervase | -6.1 |
| Class D Avg. |  | -0.8 |
| E | Vidyo1 | 0.0 |
|  | Vidyo3 | -0.9 |
|  | Vidyo4 | -0.1 |
|  | City | -0.2 |
|  | Jets | -12.4 |
| Class E Avg. |  | -2.7 |
| Total Avg. |  | -3.1 |

Table 2. BD rate (%) reduction of random access & high efficiency condition.

| Class | seq. | BD-rate(%) |
| --- | --- | --- |
| A | Traffic | 0.0 |
|  | PeopleOnStreet | 0.0 |
| Class A Avg. |  | 0.0 |
| B | BasketballDrive | -0.2 |
|  | BQTerrace | 0.1 |
|  | Cactus | -8.4 |
|  | Kimono | 0.0 |
|  | ParkScene | 0.1 |
|  | Bluesky | -14.2 |
|  | Station2 | -28.9 |
| Class B Avg. |  | -7.4 |
| C | BasketballDrill | 0.0 |
|  | BQMall | 0.1 |
|  | PartyScene | -0.2 |
|  | RaceHorses | 0.0 |
|  | Flowervase | -0.4 |
| Class C Avg. |  | -0.1 |
| D | BasketballPass | 0.1 |
|  | BlowingBubbles | 0.1 |
|  | BQSquare | -0.2 |
|  | RaceHorses | 0.1 |
|  | Flowervase | -2.3 |
| Class D Avg. |  | -0.4 |
| E | City | -3.6 |
|  | Jets | -16.4 |
| Class E Avg. |  | -10.0 |
| Total avg. |  | -3.5 |

Table 3. Complexity comparison (with TMuC 0.7.3)

| Test condition | Encoding complexity | Decoding complexity |
| --- | --- | --- |
| Low delay & high eff. | 1.3 | 1.3 |
| Random access & high eff. | 1.4 | 1.3 |
| Avg | 1.3 | 1.3 |

We have sent our decoder, bitstreams and performance excel sheet to the cross-checker, TUB.

# Subtest 2: Flexible Motion Partitioning

## Flexible motion partitioning coding [3]

This contribution reports Huawei & HiSilicon's implementation status of flexible motion partitioning on TMuC platform. At the current stage, the strict horizontal and vertical partitioning cases have been implemented in the TMuC0.7. In general, this new design has followed the principle of Asymmetric Motion Partitioning (AMP) in TMuC, and more horizontal and vertical partitioning cases have been added to TMuC0.7. The addition of these new horizontal and vertical partitions is a straightforward extension of the existing representation of the AMP in TMuC. As a result, this new partitioning design is compatible with all of the tools in the TMuC0.7 software.

Using the TE3.2 test condition, the new partitioning method has achieved about xxx and xxx bitrate saving on average for random access and lowdelay case. The simulation results are summarized in the following table.

|  | Random access, high efficiency |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.89 | -1.90 | -1.71 |
| Class B | -1.48 | -1.73 | -1.71 |
| Class C | -1.54 | -1.69 | -1.54 |
| Class D | -1.73 | -1.86 | -2.12 |
| Class E |  |  |  |
| All | -1.62 | -1.78 | -1.78 |
| Enc Time[%] | 222% |  |  |
| Dec Time[%] | 103% |  |  |

|  | Low delay, high efficiency |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -1.68 | -2.40 | -2.51 |
| Class C | -1.60 | -1.92 | -2.11 |
| Class D | -1.67 | -2.05 | -2.22 |
| Class E | -3.76 | -4.16 | -3.63 |
| All | -2.0 | -2.5 | -2.5 |
| Enc Time[%] | 223% |  |  |
| Dec Time[%] | 102% |  |  |

Complexity analysis: without a fast algorithm, the current design's encoding time is about 2 times as that of TE3.2 anchor. On the other hand, the decoding time is basically the same as that of the anchor.

Conclusion:

According the test results, the flexible motion partitioning method proposed in [1] can enhance the performance of the existing motion partitioning in TMuC through a straightforward addition of more horizontal and vertical partition configurations. And we believe the coding performance of TMuC can be further improved with the addition of non-rectangular partitioning cases. Therefore, we suggest that these new horizontal and vertical partitioning cases should be added to the TMuC software.

## Simplified geometry block partitioning [4]

This contribution relates to Simplified Geometry adaptive block partitioning (SGEO) as proposed initially in document JCTVC-B085. Geometry adaptive block partitioning (GEO), proposed by Qualcomm in JCTVC-A121, has been recently implemented in to TMuC. In its initial implementation, GEO is parameterized as follows:

- Block 16x16:  = ,  = 1  494 modes

- Block 32x32: = ,  = 1  238 modes

- Block 64x64: only AMP is used

This proposal consists of reducing the number of possible combinations as follows:

- Block 16x16: Δρ=2, ρmax=6, Δθ=45  26 modes

- Block 32x32 Δρ=4, ρmax=12, Δθ=45  26 modes

- Block 64x64: only AMP is used

The simulations are not complete yet, however the following tables give the results obtained so far:

|  | Random access high efficiency |  |  | Low delay high efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SGEO vs TMuCAMP0 | SGEO vs TMuC | SGEO vs GEO | SGEO vs TMuCAMP0 | SGEO vs TMuC | SGEO vs GEO |
| A Traffic | -3.0 | -1.6 | 1.1 |  |  |  |
| A PeopleOnStreet | -4.8 | -3.3 |  |  |  |  |
| B Kimono | -1.5 | -0.5 | 0.4 | -1.3 | -0.3 | 0.4 |
| B ParkScene | -1.9 | -0.9 | 0.8 | -2.1 | -0.7 | 0.8 |
| B Cactus | -3.5 | -2.6 | 0.4 | -3.7 | -2.4 | 0.5 |
| B BasketballDrive | -2.7 | -1.6 |  | -2.6 | -1.6 | 0.3 |
| B BQTErrace | -1.7 | -0.6 |  | 0.3 | 1.5 |  |
| C BasketballDrill | -3.3 | -2.3 | 0.8 | -3.6 | -2.7 | 0.8 |
| C BQMall | -4.4 | -2.8 | 1.8 | -4.4 | -2.7 | 1.2 |
| C PartyScene | -3.0 | -2.0 | 0.5 | -2.3 | -1.2 | 0.4 |
| C RaceHorses | -3.4 | -1.8 | 1.6 | -2.7 | -1.6 | 1.2 |
| D BasketballPass | -2.9 | -1.8 | 0.8 | -2.9 | -1.9 | 0.7 |
| D BQSquare | -2.3 | -1.1 | 0.1 | -1.2 | 0.4 | 0.3 |
| D BlowingBubbles | -3.6 | -2.4 | 0.8 | -3.7 | -2.1 | 0.8 |
| D RaceHorses | -3.9 | -2.3 | 1.5 | -3.8 | -2.4 | 1.3 |
| E Vidyo1 |  |  |  | -4.8 | -2.1 | 0.9 |
| E Vidyo3 |  |  |  | -4.8 | -1.5 | 1.5 |
| E Vidyo4 |  |  |  | -2.7 | -0.6 | 0.7 |
| Average All | -3.1 | -1.8 | 0.9 | -2.9 | -1.4 | 0.8 |

|  | Random access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.9 | -5.2 | -5.1 |  |  |  |
| Class B | -2.3 | -3.1 | -3.3 | -1.9 | -3.3 | -3.8 |
| Class C | -3.5 | -4.8 | -5.1 | -3.3 | -4.6 | -4.7 |
| Class D | -3.2 | -4.4 | -4.6 | -2.9 | -4.5 | -4.5 |
| Class E |  |  |  | -4.1 | -3.7 | -3.3 |
| All | -3.1 | -4.2 | -4.4 | -1.9 | -3.3 | -3.8 |

Table 1: SGEO vs TE03 reference (TE12 anchors config with AMP off).

|  | Random access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.4 | -3.7 | -3.7 |  |  |  |
| Class B | -1.2 | -2.0 | -2.2 | -0.7 | -1.7 | -2.0 |
| Class C | -2.2 | -3.6 | -3.6 | -2.1 | -3.1 | -3.1 |
| Class D | -1.9 | -3.2 | -3.1 | -1.5 | -2.8 | -2.8 |
| Class E |  |  |  | -1.4 | -0.1 | -0.7 |
| All | -1.8 | -3.0 | -3.0 | -1.4 | -2.0 | -2.2 |

Table 2: SGEO vs TE12 anchors.

|  | Random access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.2 | 1.0 | 1.0 |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 1.2 | 1.3 | 1.4 | 0.9 | 1.0 | 1.2 |
| Class D | 0.8 | 0.6 | 0.6 | 0.8 | 0.6 | 0.5 |
| Class E |  |  |  | 1.0 | 1.7 | 1.1 |
| All |  |  |  |  |  |  |

Table 3: SGEO vs GEO.

Regarding complexity, the average encoding time ratio between SGEO and GEO is of around 80% for both Random Access and Low Delay.

Considering the minor loss of SGEO compared to GEO, and the noticeable encoding complexity reduction, it is suggested to incorporate SGEO into the TMuC software.

# Subtest 3: Multi-hypothesis inter prediction

## Local intensity compensation (LIC) for inter prediction [5]

This contribution presents the status of implementation of the Local Intensity Compensation (LIC) tool, as a successor to the previously reported Macroblock Weighted Prediction (MBWP), in TMuC 0.7.1. Intensity compensation is done in a form of a locally adapted weighted prediction where parameters are selected for partitions within a prediction unit. The implementation has been completed partially, and the work on its additional features and fine tuning is ongoing. Although the first results with low complexity configurations show loss of up to 0.7% it is also observed that LIC can be beneficial for some sequences. This document also describes future work which addresses improvements to this method.

# Subtest 4: Improved Inter Prediction with enhanced MC filter

## Bi/Single filter switching in FIF [6]

This contribution reports the results on enhanced MC filter, Bi/Single filter switching in fixed interpolation filter, in Tool Experiment3 (TE3) subtest4. In the test, the coding performance and complexity was measured under common test conditions, which is defined in JCTVC-B300[1]. The summary of the test is as follows. At this stage, filter 8 tap filter is not optimized yet and the results of 8 tap is preliminary. At the next stage, further study on filter design for 8 tap (or longer tap), harmonization with other interpolation filter proposals should be discussed in this subtest.

Table 4 Summary of result for random access loco case

|  | Random access LoCo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DIF (6-tap) vs. |  |  |  |  |  |
|  | Bi/Single (6-tap) |  |  | DCT-IF (6-tap) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | -4.5 | -4.5 | 0.5 | -4.6 | -4.7 |
| Class B | -0.3 | -5.1 | -7.2 | -0.7 | -5.4 | -7.1 |
| Class C | -4.4 | -4.4 | -4.7 | -2.9 | -4.7 | -4.9 |
| Class D | -9.6 | -9.0 | -8.9 | -6.9 | -8.3 | -8.5 |
| Class E |  |  |  |  |  |  |
| All | -3.8 | -5.9 | -6.6 | -2.8 | -5.9 | -6.6 |
| Enc Time[%] | 105% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |

Table 5 Summary of result for low delay loco case

|  | Low delay LoCo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DIF (6-tap) vs. |  |  |  |  |  |
|  | Bi/Single (6-tap) |  |  | DCT-IF (6-tap) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -7.2 | -9.1 | 1.1 | -7.4 | -9.4 |
| Class C | -3.0 | -6.7 | -6.9 | -1.5 | -6.9 | -7.2 |
| Class D | -5.9 | -10.9 | -11.5 | -5.1 | -10.5 | -11.2 |
| Class E | -0.7 | -12.1 | -16.6 | 3.5 | -10.9 | -15.8 |
| All | -2.4 | -8.9 | -10.6 | -0.7 | -8.7 | -10.5 |
| Enc Time[%] | 105% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |

Table 6 Summary of result for random access case

|  | Random access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SIFO (12-tap) vs. |  |  |  |  |  |
|  | Bi/Single (8-tap) |  |  | DCT-IF (8tap) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.1 | 0.2 | 0.2 | 0.1 | 0.2 |
| Class B | 0.2 | -0.1 | -0.1 | 0.1 | -0.2 | -0.1 |
| Class C | 0.6 | -0.3 | -0.1 | 0.3 | -0.3 | -0.1 |
| Class D | 0.6 | -0.1 | 0.0 | 0.2 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | -0.1 | 0.0 | 0.2 | -0.2 | 0.0 |
| Enc Time[%] | 59% |  |  | 61% |  |  |
| Dec Time[%] | 95% |  |  | 96% |  |  |

Table 7 Summary of result for low delay case

|  | Low delay |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SIFO (12-tap) vs. |  |  |  |  |  |
|  | Bi/Single (8-tap) |  |  | DCT-IF (8tap) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 0.5 | 0.6 | 0.8 | 0.7 | 0.6 |
| Class C | 0.9 | 0.6 | 0.8 | 1.3 | 0.7 | 0.8 |
| Class D | 0.8 | 1.1 | 1.0 | 1.3 | 1.0 | 1.4 |
| Class E | 0.8 | -0.2 | -0.1 | 1.0 | 0.7 | 0.0 |
| All | 0.8 | 0.5 | 0.6 | 1.1 | 0.8 | 0.7 |
| Enc Time[%] | 66% |  |  | 65% |  |  |
| Dec Time[%] | 100% |  |  | 96% |  |  |

## High accuracy interpolation filter (HAIF) [7]

In this contribution, experimental results of high accuracy interpolation filters are reported.  This is one of proposals in subtest 4 of tool experiment 3 on inter prediction.  The purposes of this tool experiment are to improve the performance of inter prediction with enhanced motion compensation filters.  The experimental results show that the interpolation filter which derives each fractional pixel position directly and have more steep frequency characteristics with long tap length can achieve good performance and the interpolation filter and the in-loop filter have strong relation as a filter processing technology.  At the nest stage, computational reduction for the interpolation filter should be discussed in this subtest.

Table 8	Average results of luminance BD-rate (%) on high efficiency anchor

|  |  | HAIF (8-tap) | HAIF86 (8/6-tap) | DCT-IF (6-tap) |
| --- | --- | --- | --- | --- |
| Random access | Class A | 0.32 | 0.37 | 0.44 |
|  | Class B | 0.17 | 0.50 | 1.02 |
|  | Class C | 0.93 | 1.35 | 2.21 |
|  | Class D | 1.51 | 2.55 | 4.44 |
|  | Class E | N/A | N/A | N/A |
|  | All | 0.75 | 1.26 | 2.17 |
|  | Enc Time[%] | 62.22 | 63.04 | 56.96 |
|  | Dec Time[%] | 94.25 | 100.38 | 92.01 |
| Low delay | Class A | N/A | N/A | N/A |
|  | Class B | 0.83 | 1.22 | 1.64 |
|  | Class C | 1.99 | 2.79 | 3.07 |
|  | Class D | 3.84 | 5.82 | 6.23 |
|  | Class E | 0.58 | 0.75 | 2.17 |
|  | All | 1.82 | 2.68 | 3.24 |
|  | Enc Time[%] | 67.34 | 68.33 | 61.20 |
|  | Dec Time[%] | 95.43 | 101.13 | 93.89 |

Table 9	Average results of luminance BD-rate (%) on low complexity anchor

|  |  | HAIF (8-tap) | HAIF86 (8/6-tap) | DCT-IF (6-tap) |
| --- | --- | --- | --- | --- |
| Random access | Class A | -0.74 | 0.11 | 0.53 |
|  | Class B | -1.91 | -1.31 | -0.69 |
|  | Class C | -4.54 | -4.21 | -2.94 |
|  | Class D | -9.34 | -8.79 | -6.85 |
|  | Class E | N/A | N/A | N/A |
|  | All | -4.44 | -3.89 | -2.77 |
|  | Enc Time[%] | 111.64 | 111.83 | 100.30 |
|  | Dec Time[%] | 102.51 | 102.69% | 97.93 |
| Low delay | Class A | N/A | N/A | N/A |
|  | Class B | -1.53 | -1.25 | 1.09 |
|  | Class C | -4.38 | -3.92 | -1.52 |
|  | Class D | -8.90 | -7.90 | -5.14 |
|  | Class E | -1.70 | -0.91 | 3.50 |
|  | All | -4.11 | -3.52 | -0.67 |
|  | Enc Time[%] | 112.63 | 112.94 | 101.10 |
|  | Dec Time[%] | 101.55 | 101.77 | 98.06 |

Table 10	Average results of luminance BD-rate (%) on low complexity low delay anchor

|  |  | HAIF (8-tap) | ALF | HAIF (8-tap) + ALF |
| --- | --- | --- | --- | --- |
| Random access | Class A | -0.74 | -3.92 | -5.01 |
|  | Class B | -1.91 | -4.67 | -6.41 |
|  | Class C | -4.54 | -4.39 | -8.06 |
|  | Class D | -9.34 | -5.93 | -13.00 |
|  | Class E | N/A | N/A | N/A |
|  | All | -4.44 | -4.83 | -8.42 |
|  | Enc Time[%] | 111.64 | 101.55 | 114.86 |
|  | Dec Time[%] | 102.51 | 114.28 | 118.54 |
| Low delay | Class A | N/A | N/A | N/A |
|  | Class B | -1.53 | -4.40 | -6.53 |
|  | Class C | -4.38 | -4.89 | -8.88 |
|  | Class D | -8.90 | -4.50 | -12.20 |
|  | Class E | -1.70 | -2.90 | -4.59 |
|  | All | -4.11 | -4.27 | -8.17 |
|  | Enc Time[%] | 112.63 | 101.41 | 114.99 |
|  | Dec Time[%] | 101.55 | 116.55 | 120.37 |

# 6  Conclusion and further activities of the TE

The activity has determined tools regarding inter prediction in the new TMuC software. Promising results have been achieved in all subtests.

7 References

| [1] | A. Krutz, T. Sikora, A. Glantz, S. Park, J. Lim, E. Francois, P. Chen, X. Zheng, H. Yu, S. Paschalakis, N. Sprljan, S. Li, A. Tabatabai, T. Suzuki, T. Chujoh, W.-H. Peng, S. Matsuo, “Tool Experiment 3: Inter Prediction in HEVC,” Doc. JCTVC-B303, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010 |
| --- | --- |
| [2] | S. Park, J. Park, B. Jeon, “TE3: Motion compensation with adaptive warped reference,” Doc. JCTVC-C033/m18054, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010 |
| [3] | X. Zheng, H. Yu, “TE3.2: Huawei & HiSilicon report on flexible motion partitioning coding,” Doc. JCTVC-C099/m18122, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010 |
| [4] | E. Francois, D. Thoreau, P. Bordes, “TE3 subtest 2: Report on simplified geometry block partitioning,” Doc. JCTVC-C031/m18052, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010 |
| [5] | S. Sprljan, S. Paschalakis, P. Wu, “TE3 subtest 3: Local intensity compensation (LIC) for inter prediction,” Doc. JCTVC-C233/m18273, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010 |
| [6] | K. Kondo, T. Suzuki, “TE3 subset4: Results on Bi/Single filter switching in FIF,” Doc. JCTVC-C164/m18190, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010 |
| [7] | T. Chujoh, K. Kanou, T. Yamakage, “TE3 subtest 4: High accuracy interpolation filter (HAIF),” Doc. JCTVC-C078/m18101, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010 |