| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C224 WG11 Number: m18264 |
| --- | --- |

| Title: | Frame coding in vertical raster scan order |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Hyungjoon Kim Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-4073 h-kim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HEVC, the CUs (Coding Units) in a frame are encoded in horizontal raster scan order. We observed that the frame coding in horizontal raster scan order is not always effective for intra prediction and requires large on-chip memory to cover large vertical motion for motion estimation (ME) and motion compensation (MC), which is very challenging especially for UHD (ultra high definition) video such as 4k x 2k and 8k x 4k. In this contribution, it is proposed to support the frame coding in vertical raster scan order along with horizontal raster scan order. Then the best coding scan order can be selected adaptively for each frame based on coding efficiency, horizontal/vertical motion, on-chip memory saving and etc.

- Introduction

The conventional frame coding scheme only in horizontal raster scan order has two problems; 1) horizontal raster scan is not always effective for intra prediction and 2) with sliding window memory for ME and MC, it requires large on-chip memory to cover large vertical motion especially for UHD videos. While UHD video coding is one of the application areas of HEVC, the frame coding in horizontal raster scan order is not cost effective at all for UHD videos with high vertical motions. The support of vertical raster scan order coding is effective in resolving these problems.

(a)                                                                      (b)

Figure 1: (a) Frame coding in vertical raster scan order and (b) sub-block coding order in a CU

Figure 1 shows the vertical raster scan order where each block represents a CU and the sub-block scan order in a CU with vertical raster scan. As shown in Figure 1 (a) and (b), the available neighboring CUs and the sub-block coding order in a CU change as well with vertical raster scan. That means there should no coding gain loss on average due to vertical raster scan coding order.

In this contribution, we show the cases where vertical raster scan order works to improve coding efficiency, and we propose to support the frame coding in vertical raster scan order along with horizontal raster scan order and to adaptively select the best coding scan order based on any criteria such as coding efficiency, horizontal/vertical motion, on-chip memory saving and etc.

# Intra prediction improvement by vertical raster scan

The frame coding in vertical raster scan is effective for the frames which have lots of vertical discontinuities as shown in Figure 2. If we apply the horizontal raster scan order to this frame, we should pay more bits, for example, to encode MB mode due to incorrect mode prediction caused by discontinuity. On the contrary, we could reduce such overhead bits if we apply the vertical raster scan order.

Figure 2: Frame with vertical discontinuities

We tested the frame coding scheme in vertical rater scan order and compared it with horizontal raster scan order using TMuC-0.7.3. In this experiment, rather than applying vertical raster scan order, we rotated input sequences by 90 degree, which we believe provides the same effect. And we did not use any adaptive scan order decision methods.

Table 2 shows bit rate saving by the vertical raster scan coding for particular sequences and the average bit rate saving for all 14 test sequences. We could save bits up to 4 % for the Traffic (Class A) sequence while there is no saving on average. That is, there are losses by vertical raster scan coding order for some sequences. The best scan order can be determined in the R-D optimized way and we can reduce more bits with adaptive frame-level decision.

Table 1: Bit saving by vertical raster scan over horizontal raster scan

|  | BD-Rate Y (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Intra | Intra Low complexity | Random access | Random access Low complexity |
| Traffic | 3.3 | 4.0 | 3.7 | 3.4 |
| PeopleOnStreet | 2.9 | 3.2 | 1.9 | 1.6 |
| BQTerrace | 2.8 | 3.5 | 2.1 | 3.2 |
| Average for all sequences | 0.0 | 0.0 | 0.0 | -0.1 |

# ME improvement by vertical raster scan

The bottleneck for the HD video design is memory bandwidth. For HD videos, the sliding (growing) search window is commonly used to minimize the memory bandwidth required for loading the reference samples for ME and MC. For more detail on the sliding window scheme, please refer to [1].

A typical vertical sliding window scheme for horizontal raster scan coding order is shown in Figure 3. If a video contains large vertical motion, the vertical sliding window scheme cannot cover the large vertical motion. In this scenario, inter mode cannot be used because the required reference area is not available, which eventually causes encoding efficiency degradation. However, this problem could be easily solved by employing a horizontal sliding window scheme with vertical raster scan illustrated in Figure 4. By adaptively choosing raster scan coding order, we can improve coding efficiency of inter frames at the same on-chip memory requirement. On the other hand, we can reduce the required on-chip memory size without coding gain loss.

Figure 3: Vertical sliding window scheme for horizontal raster scan coding order

Figure 4: Horizontal sliding window scheme for vertical raster scan coding order

For horizontal and vertical search range srX and srY, on-chip memory size (byte) for sliding window for 8-bit Luma can be calculated as follows:

MemSizeHorOrder = picWidth · (2 · srY + N)

and

MemSizeVerOrder = picHeight · (2 · srX + N),

where MemSizeHorOrder and MemSizeVerOrder are on-chip memory sizes for vertical sliding window (horizontal raster coding order) and horizontal sliding window (vertical raster coding order), respectively. N * N is the largest CU size, and picWidth and picHeight are the horizontal and vertical size of the frame.

Based on the equations, Table 2 lists the search ranges with horizontal and vertical raster scan coding orders for different available on-chip memory sizes for 4k x 2k (3840 x 2160) videos. For a given on-chip memory size, srY is always limited in vertical sliding window with horizontal raster scan coding only, which potentially causes encoding efficiency degradation for large vertical motion as illustrated in Figure 3. By introducing the vertical raster scan coding with horizontal sliding window, we can not only remove the limitation on srY but also have srX larger than srY for horizontal raster scan coding. The adaptive use of vertical raster scan order based on horizontal/vertical motion will prevent quality degradation for large vertical motion.

Table 2: Available on-chip memory size vs. search ranges

|  | 500 KBytes |  | 750 Kbytes |  | 1,000 KBytes |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | srX | srY | srX | srY | srX | srY |
| Vertical sliding window | Unlimited | +/-33 | Unlimited | +/-65 | Unlimited | +/-98 |
| Horizontal sliding window | +/-83 | Unlimited | +/-141 | Unlimited | +/-199 | Unlimited |

# Conclusions

The frame coding in vertical raster scan order is likely to give better coding efficiency for particular contents. We would like to recommend starting a tool/core experiment or AhG activity to study vertical scanning more.

# References

[1] Minhua Zhou, “Sub-picture based raster scan coding order for HEVC UHD video coding”, JCTVC-B062, 21-28 July 2010, Geneva, Switzeland.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).