| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C225 WG11 Number: m18265 |
| --- | --- |

| Title: | TE12: Summary of evaluation of TMuC tools in TE12 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ken McCann (Samsung/ZetaCast) | Email: | ken@zetacast.com |
| Source: | TE12 Coordinator |  |  |

_____________________________

# Abstract

This contribution is a summary of tool experiment 12 (TE12) on exploring the performance of individual tools in the Test Model under Consideration (TMuC)[1].  Detailed results are reported by the TE12 participants in individual input documents.

- Introduction

The incremental contribution of each individual tool in TMuC is highly dependent on the settings of the other tools in use; in effect, it is a multi-dimensional non-linear system.   The starting points used in TE12 were the default configurations of the TMuC specified in JCTVC-B300[2], since these represent the current best estimate of realistic use cases.

Each tool in the TMuC was tested individually as follows.

- Some tools are on/off in nature (e.g. ROT).  This type of tool was tested by being turned on (if off in the default configuration) or off (if on in the default configuration) unless it has a direct alternative (see below).

- Some tools have direct alternatives either within the TMuC itself or else added to the software for comparison purposes (e.g. PIPE and the LCEC entropy coding within the TMuC, with CABAC for comparison).  This type of tool was tested by being replaced by each direct alternative in turn (e.g. PIPE was tested against both LCEC and CABAC).

- Some tools can have minimum or maximum limits defined (e.g. Large Transform).  This type of tool was tested by having either the minimum or maximum limit made stricter as appropriate (e.g. maximum transform size being reduced from 64x64 to 32x32).

Tools for which there is no alternative in the TMuC (e.g. inter transforms) were not tested.

In most cases, only one tool at a time was exercised by giving it an alternative setting from that of the default configuration.  However, some limited testing of combinations of tools was also performed, in order to improve understanding of the interaction between tools (e.g. between ROT and MDDT).

- Document List

A total of 70 documents directly relating to TE12 were registered.  In the following list, the number “x” in the “TE12.x” part of the title represents the test category that the paper refers to.  In some cases these have been modified from the registered title by the TE12 Coordinator.  The categories are as follows:

1. Unit definitions

2. Motion Representation

3. Intra-frame Prediction

4. Transforms

5. Filtering

6. Entropy Coding

7. Other decoder tools

8. Encoder-only tools

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C029 | m18040 | TE12.1: Report on 64x64 versus 32x32 maximum coding unit size for low complexity configuration | P. Pandit (Harmonic) |
| JCTVC-C030 | m18051 | TE12.1: Report on AMP evaluation | E. Francois (Technicolor), L. Guillo (INRIA), A. Ichigaya (NHK), H. Yu (Huawei) |
| JCTVC-C041 | m18062 | TE12.2: Performance of partition based illumination compensation (PBIC) | A. Fujibayashi, S. Kanumuri, F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C044 | m18065 | TE12.1, 12.3, 12.8: Results for experiments on max CU size, RDOQ and AIS | F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C047 | m18068 | TE12.4: Transform skip flag test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C048 | m18069 | TE12.6: PIPE and LCEC tested against CABAC by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C049 | m18070 | TE12.4: Transform coding HHI tested against Samsung proposal by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C050 | m18071 | TE12.2: Interpolation filter SIFO and DIF tested against MOMS by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C051 | m18072 | TE12.6: PIPE tested against LCEC by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C052 | m18073 | TE12.1: Block merging (MRG) test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C053 | m18074 | TE12.3: Adaptive intra smoothing (AIS) test (slow vs. fast) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C054 | m18075 | TE12.3: Adaptive intra smoothing (AIS) test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C056 | m18079 | TE12.1: Evaluation of transform unit (TU) size | M. Zhou, V. Sze (TI) |
| JCTVC-C057 | m18080 | TE12.7: Evaluation of IBDI and TPE (transform precision extension) | M. Zhou (TI) |
| JCTVC-C058 | m18081 | TE12.6: Evaluation of entropy coders: PIPE tested against CABAC | V. Sze, M. Budagavi (TI) |
| JCTVC-C059 | m18082 | TE12.4: Evaluation of transform coefficient coding (HHI_TRANSFORM_CODING) with tool breakdown | V. Sze, M. Budagavi, M. Zhou (TI) |
| JCTVC-C087 | m18110 | TE12.2: Experimental results of interpolation filter | T. Chujoh, K. Kanou, T. Yamakage (Toshiba) |
| JCTVC-C088 | m18111 | TE12.7: Experimental results of IBDI | T. Chujoh, T. Yamakage (Toshiba) |
| JCTVC-C092 | m18115 | TE12.3: Report on combined intra prediction evaluation | K. Chono, K. Senzaki, H. Aoki, J. Tajime, Y. Senda (NEC) |
| JCTVC-C104 | m18127 | TE12.1: Huawei & HiSilicon report on AMP evaluation | X. Zheng (HiSilicon), J. Zhou, H. Yu (Huawei) |
| JCTVC-C105 | m18128 | TE12.1: Huawei report on max transform unit size evaluation | J. Zhou, D. Wang, H. Yu (Huawei) |
| JCTVC-C109 | m18132 | TE12.1: Results for PU-based merging by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C110 | m18133 | TE12.4: Transform unit quadtree tested against 2-level by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C118 | m18141 | TE12.1 cross-check report on block merging (MRG) on/off | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C119 | m18142 | TE12.2 report on MV prediction AMVP/IMVP | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C120 | m18143 | TE12.3 cross-check report on AIS on/off | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C129 | m18153 | TE12.5: Cross verification for adaptive loop filter (ALF) (on vs. off) | Y.-J. Chiu, L. Xu, W. Zhang, H. Jiang (Intel) , |
| JCTVC-C136 | m18160 | TE12.4: Mode dependent direnctional transform (MDDT) test (off vs. on) by HKUST | X. Zhang (HKUST) |
| JCTVC-C148 | m18174 | TE12.5: Crosscheck on deblocking filter in TMuC0.7 by MediaTek | Y.-L. Chang, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-C149 | m18175 | TE12.1: Report on asymmetric motion prediction unit (AMP) on/off | A. Ichigaya, K. Iguchi (NHK) |
| JCTVC-C153 | m18179 | TE12.5: Report on deblocking filter | P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C159 | m18185 | TE12.8: Cross verification of RDOQ | K. Andersson (Ericsson) |
| JCTVC-C160 | m18186 | TE12.6: Crosscheck on LCEC phase 2 | P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C162 | m18188 | TE12.2: Results on MC interpolation filters | K. Kondo, T. Suzuki (Sony) |
| JCTVC-C163 | m18189 | TE12.2: Results on MV prediction | K. Sato, T. Suzuki (Sony) |
| JCTVC-C171 | m18200 | TE12.3: Experimental results of edge based prediction | A. Tabatabai, C. Auyeung (Sony) |
| JCTVC-C174 | m18203 | TE12.3: Report on combined intra prediction | T. Yamamoto, A. Segall (SHARP) |
| JCTVC-C181 | m18210 | TE12.4: Cross-check of rotational transform (ROT) | K. Misra, A. Segall (Sharp) |
| JCTVC-C182 | m18211 | TE12.2: Cross-check of adaptive motion vector resolution (AMVRES) technology in the TMuC software | A. Segall (Sharp) |
| JCTVC-C184 | m18215 | TE12.6: PIPE tested against LCEC by Cisco | A. Fuldseth (Cisco) |
| JCTVC-C188 | m18219 | TE12.3: Results for planar prediction tests by Nokia | J. Lainema, K. Ugur, O. Bici (Nokia) |
| JCTVC-C196 | m18233 | TE12.6: Results for LCEC_PHASE2 tests by Nokia | K. Ugur, A. Hallapuro, J. Lainema (Nokia) |
| JCTVC-C197 | m18235 | TE12.2: Results for SIFO-6Tap with DIF tests by Nokia and Qualcomm | K. Ugur, J. Lainema (Nokia), R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C200 | m18238 | TE12.1: Experimental results of transform unit quadtree/2-level test | T. Lee, J. Chen, W.-J. Han (Samsung) |
| JCTVC-C201 | m18239 | TE12.2: Experimental results of SIFO (DIF) / DCT-IF | E. Alshina, J. Chen, N. Shlyakhov, A. Alshin, W.-J. Han (Samsung) |
| JCTVC-C202 | m18240 | TE12.4: Experimental results of MDDT and ROT by Samsung and Qualcomm | E. Alshina, A. Alshin, W.-J. Han (Samsung), R. Joshi, M. Coban (Qualcomm) |
| JCTVC-C203 | m18241 | TE12.4: Experimental results of transform coefficient coding | J. Chen, V. Seregin, W.-J. Han (Samsung) |
| JTVC-C213 | m18251 | - TE12.3: Additional results for combined intra prediction | M. Mrak, T.Davies, D. Flynn, A. Gabriellini (BBC) |
| JCTVC-C214 | m18254 | TE12.5: Results for adaptive loop filter using prediction and residual | M. Narroschke (Panasonic) |
| JCTVC-C216 | m18256 | TE12.3: Results for edge based prediction | V. Drugeon (Panasonic) |
| JCTVC-C225 | m18265 | TE12 : Summary of evaluation of TMuC tools in TE12 (this document) | K. McCann (Samsung/Zetacast) |
| JCTVC-C230 | m18270 | TE12.5: Cross check result of Panasonic's 3-input-ALF | I. S. Chong, M. Karczewicz (Qualcomm) |
| JCTVC-C231 | m18271 | TE12.1: Results for PU based merging | T.K. Tan |
| JCTVC-C232 | m18272 | TE12.7: Verification of JCTVC-C041 (partition based illumination compensation - PBIC) | N. Sprljan, S. Paschalakis, P. Wu (Mitsubishi Electric) |
| JCTVC-C235 | m18276 | TE12.5: Evaluation of adaptive in-loop filter | I. S. Chong, M. Karczewicz (Qualcomm) |
| JCTVC-C238 | m18279 | TE12.8: Results on RDOQ in high efficiency settings | J. Zan, J. Meng, M. T. Islam, D. He (RIM) |
| JCTVC-C239 | m18280 | TE12.2: Report on interpolation filter: SIFO/DIF | R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C241 | m18282 | TE12.2: Crosscheck on interpolation filter: SIFO(DIF)/MOMS | R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C244 | m18285 | TE12.3: Report on planar intra prediction | D. Flynn (BBC) |
| JCTVC-C246 | m18287 | TE12.2: Cross-verification of partition-based illumination compensation (PBIC) | P. Onno (Canon) |
| JCTVC-C248 | m18289 | TE12.2: Cross verification on DCT-IF for chroma | T. Chujoh, K. Kanou, T. Yamakage (Toshiba) |
| JCTVC-C252 | m18293 | TE12.7: Report on evaluation of internal bit depth increase and transform precision extension | R. Joshi, M. Karczewicz (Qualcomm) |
| JCTVC-C258 | m18299 | TE12.4: Evaluation of transform skip flag (RQT_ROOT) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C259 | m18300 | TE12.1: Results for experiments on Max CU size | Y. Yu, K. Panusopone, L. Wang, S.-T. Hsiang, F. Ishtiaq (Motorola) |
| JCTVC-C264 | m18305 | TE12.1: Evaluation of block merging (MRG) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C265 | m18306 | TE12.2: Evaluation of adaptive motion vector resolution (AMVRES) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C268 | m18310 | TE12.4: Report on evaluation of MDDT and ROT | R. Joshi, M. Coban, M. Karczewicz (Qualcomm) |
| JCTVC-C281 | m18324 | TE12.3: Evaluation of fast adaptive intra smoothing | M. Coban, Y. Zheng, M. Karczewicz (Qualcomm) |
| JCTVC-C286 | m18329 | TE12.1: Technicolor & INRIA report on AMP evaluation | E. Francois (Technicolor), L. Guillo (INRIA) |
| JCTVC- C297 | m18344 | TE12.3: Cross-verification of JCTVC-C213: additional results for combined intra prediction | W.-J. Han (Samsung) |

- Tests Performed

  - Unit Definitions

The following tests on unit definitions were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Max Coding unit size | Min size = 8x8 Max size = 64x64 | Min size = 8x8 Max size = 32x32 | MaxCUWidth : 32 MaxCUHeight : 32 MaxPartitionDepth : 3 |
| Transform unit quadtree/2-level | Quadtree | 2-level | #define HHI_RQT 0 #define HHI_RQT_CHROMA_CDF_MOD 0 #define HHI_RQT_INTRA 0 |
| Max transform unit size | Max size = 64 | Max size = 32 | QuadTreeTULog2MaxSize : 5 |
| Asymmetric Motion Prediction Unit (AMP) | Enabled | Disabled | AMP: 0 |
| Block Merging (MRG) on/off | Enabled | Disabled | MRG: 0 |
| PU-Based Merging (tested with TMuC 0.8-hhi-bugfix) | Disabled | Enabled | Tested with TMuC 0.8-hhi-bugfix |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. acc, high-efficiency | Rand. acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| Max CU size 64x64/32x32 | NTT Docomo | Harmonic | NTT Docomo | Harmonic | NTT Docomo | Harmonic |
|  | Motorola | Motorola | Motorola | Motorola | Motorola | Motorola |
| Transform unit quadtree/2-level | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| Max TU size 64x64/32x32 | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon |
|  | TI | TI | TI | TI | TI | TI |
| Asymmetric PU (AMP) on/off |  |  | Technicolor/INRIA | NHK | Technicolor/INRIA | NHK |
|  |  |  | NHK, Huawei/HiSilicon | Technicolor/INRIA Huawei/HiSilicon | NHK, Huawei/HiSilicon | Technicolor/INRIA Huawei/HiSilicon |
| Block Merging (MRG) on/off |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  |  |  | Mitsubishi, HHI | Mitsubishi, HHI | Mitsubishi, HHI | Mitsubishi, HHI |
| PU Based Merging (TMuC 0.8-hhi-bugfix) |  |  | HHI | HHI | HHI | HHI |
|  |  |  | DOCOMO | DOCOMO | DOCOMO | DOCOMO |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C029 | m18040 | TE12.1: Report on 64x64 versus 32x32 maximum coding unit size for low complexity configuration | P. Pandit (Harmonic) |
| JCTVC-C030 | m18051 | TE12.1: Report on AMP evaluation | E. Francois (Technicolor), L. Guillo (INRIA), A. Ichigaya (NHK), H. Yu (Huawei) |
| JCTVC-C044 | m18065 | TE12.1, 12.3, 12.8: Results for experiments on max CU size, RDOQ and AIS | F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C052 | m18073 | TE12.1: Block merging (MRG) test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C056 | m18079 | TE12.1: Evaluation of transform unit (TU) size | M. Zhou, V. Sze (TI) |
| JCTVC-C104 | m18127 | TE12.1: Huawei & HiSilicon report on AMP evaluation | X. Zheng (HiSilicon), J. Zhou, H. Yu (Huawei) |
| JCTVC-C105 | m18128 | TE12.1: Huawei report on max transform unit size evaluation | J. Zhou, D. Wang, H. Yu (Huawei) |
| JCTVC-C109 | m18132 | TE12.1: Results for PU-based merging by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C118 | m18141 | TE12.1 cross-check report on block merging (MRG) on/off | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C149 | m18175 | TE12.1: Report on asymmetric motion prediction unit (AMP) on/off | A. Ichigaya, K. Iguchi (NHK) |
| JCTVC-C200 | m18238 | TE12.1: Experimental results of transform unit quadtree/2-level test | T. Lee, J. Chen, W.-J. Han (Samsung) |
| JCTVC-C231 | m18271 | TE12.1: Results for PU based merging | T.K. Tan |
| JCTVC-C259 | m18300 | TE12.1: Results for experiments on Max CU size | Y. Yu, K. Panusopone, L. Wang, S.-T. Hsiang, F. Ishtiaq (Motorola) |
| JCTVC-C264 | m18305 | TE 12.1: Evaluation of block merging (MRG) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C286 | m18329 | TE12.1: Technicolor & INRIA report on AMP evaluation | E. Francois (Technicolor), L. Guillo (INRIA) |

  - Motion Representation

The following tests on motion representation were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Motion vector prediction (AMVP/IMVP) | AMVP method | IMVP method | IMP : 1 #define HHI_AMVP_OFF 1 |
| Interpolation filter: test defaults against each other | SIFO (LoCo: DIF) | DIF (LoCo: SIFO) | InterpFilterType: 3 (LoCo: InterpFilterType : 4) |
| Interpolation filter: test defaults against DCT-IF | SIFO (LoCo: DIF) | DCT-IF | InterpFilterType: 0 |
| Interpolation filter: test defaults against MOMS | SIFO (LoCo: DIF) | MOMS | InterpFilterType : 2 |
| Interpolation Filter: test SIFO 6-tap with DIF (TMuC 0.8) | SIFO (LoCo: DIF) | SIFO 6-tap with DIF | Used TMuC 0.8 |
| Adaptive motion vector resolution (AMVRES) | Enabled | Disabled | AMVRES : 0 |
| Partition-based intensity compensation (PBIC) (TmuC 0.7.4) | Disabled | Enabled | #define DCM_PBIC PBIC : 1 |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. Acc, high-efficiency | Rand. Acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| MV prediction AMVP / IMVP |  |  | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi |
|  |  |  | Sony | Sony | Sony | Sony |
| Interpolation filter: SIFO / DIF |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  |  |  | NTT | NTT | NTT | NTT |
| Interpolation filter: SIFO (DIF) / DCT-IF |  |  | Toshiba | Toshiba | Toshiba | Toshiba |
|  |  |  | Sony, Samsung | Sony, Samsung | Sony, Samsung | Sony, Samsung |
| Interpolation filter: SIFO (DIF) / MOMS |  |  | HHI | HHI | HHI | HHI |
|  |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
| Interpolation Filter: SIFO-6 Tap with DIF |  |  | Nokia | Nokia | Nokia | Nokia |
|  |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
| Adaptive MV resolution on/off |  |  | Qualcomm |  | Qualcomm |  |
|  |  |  | Sharp |  | Sharp |  |
| Partition-based intensity comp (TmuC 0.7.4) |  |  | Canon | Canon | Canon | Canon |
|  |  |  | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C041 | m18062 | TE12.2: Performance of partition based illumination compensation (PBIC) | A. Fujibayashi, S. Kanumuri, F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C050 | m18071 | TE12.2: Interpolation filter SIFO and DIF tested against MOMS by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C087 | m18110 | TE12.2: Experimental results of interpolation filter | T. Chujoh, K. Kanou, T. Yamakage (Toshiba) |
| JCTVC-C119 | m18142 | TE12.2 report on MV prediction AMVP/IMVP | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C162 | m18188 | TE12.2: Results on MC interpolation filters | K. Kondo, T. Suzuki (Sony) |
| JCTVC-C163 | m18189 | TE12.2: Results on MV prediction | K. Sato, T. Suzuki (Sony) |
| JCTVC-C182 | m18211 | TE12.2: Cross-check of adaptive motion vector resolution (AMVRES) technology in the TmuC software | A. Segall (Sharp) |
| JCTVC-C197 | m18235 | TE12.2: Results for SIFO-6Tap with DIF tests by Nokia and Qualcomm | K. Ugur, J. Lainema (Nokia), R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C201 | m18239 | TE12.2: Experimental results of SIFO (DIF) / DCT-IF | E. Alshina, J. Chen, N. Shlyakhov, A. Alshin, W.-J. Han (Samsung) |
| JCTVC-C232 | m18272 | TE12.2: Verification of JCTVC-C041 (partition based illumination compensation – PBIC) | N. Sprljan, S. Paschalakis, P. Wu (Mitsubishi Electric) |
| JCTVC-C239 | m18280 | TE12.2: Report on interpolation filter: SIFO/DIF | R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C241 | m18282 | TE12.2: Crosscheck on interpolation filter: SIFO(DIF)/MOMS | R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C246 | m18287 | TE12.2: Cross-verification of partition-based illumination compensation (PBIC) | P. Onno (Canon) |
| JCTVC-C248 | m18289 | TE12.2: Cross verification on DCT-IF for chroma | T. Chujoh, K. Kanou, T. Yamakage (Toshiba) |
| JCTVC-C265 | m18306 | TE12.2: Evaluation of adaptive motion vector resolution (AMVRES) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |

## Intra-frame Prediction

The following tests on intra-frame prediction were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Adaptive Intra Smoothing (AIS) on/off | Enabled | Disabled | AIS : 0 |
| Adaptive Intra Smoothing (AIS) fast/slow | Fast | Slow | #define AIS_TEST_BEST 1 |
| Combined Intra Prediction (CIP) on/off | Enabled | Disabled | CIP : 0 |
| Enhanced CIP on/off | Disabled | Enabled | Tested with 0.7-BBC branch |
| Planar prediction on/off | Enabled | Disabled | #define PLANAR_INTRA 0 |
| Edge-based prediction on/off | Enabled | Disabled | EdgePredictionEnable : 0 |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. Acc, high-efficiency | Rand. Acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| Adapt. Intra smooth (AIS) on/off | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| Adapt. Intra smooth (AIS) fast/slow | HHI | HHI | HHI | HHI | HHI | HHI |
|  | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
| Combined Intra pred. (CIP) on/off | NEC | NEC | NEC | NEC | NEC | NEC |
|  | Sharp | Sharp | Sharp | Sharp | Sharp | Sharp |
| Enhanced CIP on/off | BBC | BBC | BBC | BBC | BBC | BBC |
|  | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
| Planar prediction on/off | BBC | BBC | BBC | BBC | BBC | BBC |
|  | Nokia | Nokia | Nokia | Nokia | Nokia | Nokia |
| Edge-based prediction on/off | Panasonic | Panasonic | Panasonic | Panasonic | Panasonic | Panasonic |
|  | Sony | Sony | Sony | Sony | Sony | Sony |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C044 | m18065 | TE12.1, 12.3, 12.8: Results for experiments on max CU size, RDOQ and AIS | F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C053 | m18074 | TE12.3: Adaptive intra smoothing (AIS) test (slow vs. fast) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C054 | m18075 | TE12.3: Adaptive intra smoothing (AIS) test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C092 | m18115 | TE12.3: Report on combined intra prediction evaluation | K. Chono, K. Senzaki, H. Aoki, J. Tajime, Y. Senda (NEC) |
| JCTVC-C120 | m18143 | TE12.3 cross-check report on AIS on/off | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C171 | m18200 | TE12.3: Experimental results of edge based prediction | A. Tabatabai, C. Auyeung (Sony) |
| JCTVC-C174 | m18203 | TE12.3: Report on combined intra prediction | T. Yamamoto, A. Segall (SHARP) |
| JCTVC-C188 | m18219 | TE12.3: Results for planar prediction tests by Nokia | J. Lainema, K. Ugur, O. Bici (Nokia) |
| JTVC-C213 | m18251 | - TE12.3: Additional results for combined intra prediction | M. Mrak, T.Davies, D. Flynn, A. Gabriellini (BBC) |
| JCTVC-C216 | m18256 | TE12.3: Results for edge based prediction | V. Drugeon (Panasonic) |
| JCTVC-C244 | m18285 | TE12.3: Report on planar intra prediction | D. Flynn (BBC) |
| JCTVC-C281 | m18324 | TE12.3: Evaluation of fast adaptive intra smoothing | M. Coban, Y. Zheng, M. Karczewicz (Qualcomm) |
| JCTVC- C297 | m18344 | TE12.3: Cross-verification of JCTVC-C213: additional results for combined intra prediction | W.-J. Han (Samsung) |

## Transforms

The following tests on transforms were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Rotational transform (ROT) | Enabled | Disabled | ROT : 0 |
| Mode depend. Directional transform (MDDT) | Enabled | Disabled | #define QC_MDDT 0 |
| Combined ROT and MDDT test | Both enabled | Both disabled | ROT : 0 #define QC_MDDT 0 |
| Transform coefficient coding | HHI | Samsung | #define HHI_TRANSFORM_CODING 0 |
| Transform Skip Flag (TmuC 0.7.2) | Disabled | Enabled | #define HHI_RQT_ROOT |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. Acc, high-efficiency | Rand. Acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| Rotational transform (ROT) on/off | Sharp | Sharp | Sharp | Sharp | Sharp | Sharp |
|  | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
| Mode dep. Directional trans. (MDDT) on/off | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
|  | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm |
| Combined ROT+MDDT on/off | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
| Transform coding HHI/Samsung | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| Transform Skip Flag on/off (TmuC 0.7.2) |  |  | HHI | HHI | HHI | HHI |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C047 | m18068 | TE12.4: Transform skip flag test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C049 | m18070 | TE12.4: Transform coding HHI tested against Samsung proposal by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C059 | m18082 | TE12.4: Evaluation of transform coefficient coding (HHI_TRANSFORM_CODING) with tool breakdown | V. Sze, M. Budagavi, M. Zhou (TI) |
| JCTVC-C110 | m18133 | TE12.4: Transform unit quadtree tested against 2-level by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C136 | m18160 | TE12.4: Mode dependent direnctional transform (MDDT) test (off vs. on) by HKUST | X. Zhang (HKUST) |
| JCTVC-C181 | m18210 | TE12.4: Cross-check of rotational transform (ROT) | K. Misra, A. Segall (Sharp) |
| JCTVC-C202 | m18240 | TE12.4: Experimental results of MDDT and ROT by Samsung and Qualcomm | E. Alshina, A. Alshin, W.-J. Han (Samsung), R. Joshi, M. Coban (Qualcomm) |
| JCTVC-C203 | m18241 | TE12.4: Experimental results of transform coefficient coding | J. Chen, V. Seregin, W.-J. Han (Samsung) |
| JCTVC-C258 | m18299 | TE12.4: Evaluation of transform skip flag (RQT_ROOT) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C268 | m18310 | TE12.4: Report on evaluation of MDDT and ROT | R. Joshi, M. Coban, M. Karczewicz (Qualcomm) |

## Filtering

The following tests on filtering were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Deblocking filter | Enabled | Disabled | LoopFilterDisable : 1 |
| Adaptive loop filter (ALF) on/off | Diamond (LoCo: disabled) | Disabled (LoCo: diamond) | ALF : 0 (LoCo: ALF : 1) |
| ALF default/3-input ALF (tested with 0.7-panasonic branch) | Diamond (LoCo: disabled) | 3-input ALF | Tested with 0.7-panasonic branch |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. acc, high-efficiency | Rand. acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| Deblocking filter on/off | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  | MediaTek | MediaTek | MediaTek | MediaTek | MediaTek | MediaTek |
| Adaptive loop filter (ALF) on/off | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  | Intel | Intel | Intel | Intel | Intel | Intel |
| ALF default/3-input (0.7-panasonic branch) | Panasonic | Panasonic | Panasonic | Panasonic | Panasonic | Panasonic |
|  | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C129 | m18153 | TE12.5: Cross verification for adaptive loop filter (ALF) (on vs. off) | Y.-J. Chiu, L. Xu, W. Zhang, H. Jiang (Intel) , |
| JCTVC-C148 | m18174 | TE12.5: Crosscheck on deblocking filter in TMuC0.7 by MediaTek | Y.-L. Chang, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-C153 | m18179 | TE12.5: Report on deblocking filter | P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C214 | m18254 | TE12.5: Results for adaptive loop filter using prediction and residual | M. Narroschke (Panasonic) |
| JCTVC-C230 | m18270 | TE12.5: Cross check result of Panasonic's 3-input-ALF | I. S. Chong, M. Karczewicz (Qualcomm) |
| JCTVC-C235 | m18276 | TE12.5: Evaluation of adaptive in-loop filter | I. S. Chong, M. Karczewicz (Qualcomm) |

## Entropy Coding

The following tests on entropy coding were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Entropy coders: tested against each other | PIPE (LoCo: LCEC) | LCEC (LoCo: PIPE) | SymbolMode : 0 MultiCodewordThreshold: 0 (LoCo: SymbolMode : 2, MultiCodewordThreshold : 96000) |
| Entropy coders: tested against CABAC | PIPE (LoCo: LCEC) | CABAC | SymbolMode : 1 MultiCodewordThreshold : 0 |
| LCEC Phase 2 on/off (TMuC 0.7.4) | Disabled | Enabled | #define LCEC_PHASE2 |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. acc, high-efficiency | Rand. acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| PIPE tested against LCEC | Cisco Norway | Cisco Norway | Cisco Norway | Cisco Norway | Cisco Norway | Cisco Norway |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| PIPE (and LCEC) test against CABAC | TI | TI | TI | TI | TI | TI |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| LCEC Phase 2 on/off (TMuC 0.7.4) |  | Nokia |  | Nokia |  | Nokia |
|  |  | Qualcomm |  | Qualcomm |  | Qualcomm |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C048 | m18069 | TE12.6: PIPE and LCEC tested against CABAC by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C051 | m18072 | TE12.6: PIPE tested against LCEC by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C058 | m18081 | TE12.6: Evaluation of entropy coders: PIPE tested against CABAC | V. Sze, M. Budagavi (TI) |
| JCTVC-C160 | m18186 | TE12.6: Crosscheck on LCEC phase 2 | P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C184 | m18215 | TE12.6: PIPE tested against LCEC by Cisco | A. Fuldseth (Cisco) |
| JCTVC-C196 | m18233 | TE12.6: Results for LCEC_PHASE2 tests by Nokia | K. Ugur, A. Hallapuro, J. Lainema (Nokia) |

## Other

The following tests on other decoding tools were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Internal Bit-Depth Increase (IBDI) | 4 bits (LoCo: off) | Off (LoCo: 4bits) | BitIncrement : 0 (LoCo: BitIncrement : 4) |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. acc, high-efficiency | Rand. acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| Internal Bit Depth Increase (IBDI) on/off | Toshiba | Toshiba | Toshiba | Toshiba | Toshiba | Toshiba |
|  | TI, Qualcomm | TI, Qualcomm | TI, Qualcomm | TI, Qualcomm | TI, Qualcomm | TI, Qualcomm |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C057 | m18080 | TE12.7: Evaluation of IBDI and TPE (transform precision extension) | M. Zhou (TI) |
| JCTVC-C088 | m18111 | TE12.7: Experimental results of IBDI | T. Chujoh, T. Yamakage (Toshiba) |
| JCTVC-C252 | m18293 | TE12.7: Report on evaluation of internal bit depth increase and transform precision extension | R. Joshi, M. Karczewicz (Qualcomm) |

## Encoder-only Tools

The following encoder-only tests were performed:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Rate Distortion Optimized Quantization (RDOQ) | On | Off | RDOQ : 0 |

The companies that volunteered to perform these tests were:

|  | Intra, high-efficiency | Intra, low-complexity | Rand. acc, high-efficiency | Rand. acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| Rate Distortion Opti. Quant (RDOQ) on/off | RIM | NTT Docomo | RIM | NTT Docomo | RIM | NTT Docomo |
|  | Ericsson | Ericsson | Ericsson | Ericsson | Ericsson | Ericsson |

The following documents relating to these tests were registered:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C044 | m18065 | TE12.1, 12.3, 12.8: Results for experiments on max CU size, RDOQ and AIS | F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C159 | m18185 | TE12.8: Cross verification of RDOQ | K. Andersson (Ericsson) |
| JCTVC-C238 | m18279 | TE12.8: Results on RDOQ in high efficiency settings | J. Zan, J. Meng, M. T. Islam, D. He (RIM) |

# Results

All of the tools listed in the Tool Experiment 12 test plan [3] were evaluated, with the exception of one encoder-only test on CABAC-based RDO on/off.  In most cases the tests used versions TMuC 0.7 and subsequent versions of the TMuC released by the Software AHG.  However, three of the tests used company-specific branches: PU-Based Merging (tested with TMuC 0.8-hhi-bugfix), Enhanced CIP (tested with 0.7-BBC branch) and ALF default/3-input ALF (tested with 0.7-panasonic branch).

Detailed results are reported by the TE12 participants in individual input documents.

It is anticipated that the results of this work will be used to guide improvements in the default operating points of the TMuC.  In addition, it should provide insight on how the various tools contribute to the overall performance of the TMuC and hence should help inform the process of forming the first Test Model. However, it should be noted that the various tools in TMuC interact in a complex non-linear manner, so that careful interpretation of the results is required.

- References

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312rev6, JCT-VC Meeting, Geneva, July 2010