| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 07-14 October, 2010 | Document: JCTVC-C227 WG11 Number: m18267 |
| --- | --- |

| Title: | Parallelization of HHI_TRANSFORM_CODING |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. Madhukar Budagavi Texas Instruments Inc. | Email: Email: | sze@ti.com |
|  |  |  | madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

While various parallel bin encoding approaches have been proposed, context modeling remains a parallelism challenge in the entropy coding engine.  HHI_TRANSFORM_CODING uses a highly adaptive context modeling approach for significance map, where context selection depends on coefficients in neighboring positions.  While it provides coding gains between 0.8% to 1.1%, it introduces significant dependencies which are difficult to parallelize when using zig-zag scan or adaptive scan order.  In this document, we propose a waveform diagonal scan order which enables parallel processing in conjunction with the use of the highly adaptive context selection for significance map.  Furthermore, we propose a simplification of the context selection.  These modifications were implemented in TMuC-0.7.3 and their coding efficiencies were evaluated.  The modifications come at a cost of 0.2% to 0.4% coding loss.  Thus, the parallelized HHI_TRANSFORM_CODING with simplified context selection (reduce dependency by one or two scans) and wavefront scan (down left direction) still provides gains between 0.9 to 1.4% compared with not using HHI_TRANSFORM_CODING.

# Introduction

For improved coding efficiency, HHI_TRANSFORM_CODING proposes a highly adaptive context selection approach for significance map, where the context selection can depend on the value of the coefficients in neighbouring positions (typically in the left, top-left and top directions) in the transform [2].    For instance, in Fig. 1 below, the context for position X can depend on positions A, B, C, and D (note only one of the D will be required, depending on the direction of the scan). These dependencies are derived from the source code of HHI_TRANSFORM_CODING in Fig. 2.

Fig.1 – Dependencies of context selection for significant map for improved coding efficiency.

UInt TComTrQuant::getSigCtxInc ( TCoeff* pcCoeff, const UInt uiPosX, const UInt uiPosY, const UInt uiLog2BlkSize, const UInt uiStride, const bool bDownLeft ) { UInt uiCtxInc = 0; UInt uiSizeM1 = ( 1 << uiLog2BlkSize ) - 1; . . . . . . . . . { const int* pData = &pcCoeff[ uiPosX + uiPosY * uiStride ]; int iStride = uiStride; int iStride2= iStride << 1; UInt uiCnt = ( pData[ -iStride ] ? 1 : 0 ); A uiCnt += ( pData[ -1 ] ? 1 : 0 ); uiCnt += ( pData[ -1 -iStride ] ? 1 : 0 ); if( uiPosX > 1 ) { uiCnt += ( pData[ -2 ] ? 1 : 0 ); uiCnt += ( pData[ -2 -iStride ] ? 1 : 0 ); B if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -2 +iStride ] ? 1 : 0 ); } } if( uiPosY > 1 ) { uiCnt += ( pData[ -iStride2 ] ? 1 : 0 ); uiCnt += ( pData[ -1 -iStride2 ] ? 1 : 0 ); C if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride2 ] ? 1 : 0 ); } } if( bDownLeft ) { if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride ] ? 1 : 0 ); } } else D { if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -1 +iStride ] ? 1 : 0 ); } } uiCtxInc = 10 + min<UInt>( 4, ( uiCnt + 1 ) >> 1 ); } return uiCtxInc; }

Figure 2 Source code (TComTrQuant.cpp) to process context selection for proposed context modeling of significance map.  Dependencies are highlighted.

With these dependencies, it is difficult perform context selection for decoding multiple bins at the same time; particularly if the traditional approach of zig-zag scan or the HHI_TRANSFORM_CODING adaptive scan is used.  In this contribution, we propose a different scan order to enable multiple context selection while maintaining high coding efficiency.

# Wavefront Scan

Rather than using the traditional zig-zag scan, we propose a wavefront scan approach were all diagonal scans are in the same direction (either all top-right or all down-left) (see Fig.3).  It can be thought of as waveform processing in the diagonal direction.

Fig.3 – Different scanning orders.  Note that the diagonals of the proposed wavefront (fixed) direction scan can be processed in parallel.

The benefit of this approach is that the diagonals can be processed in parallel and still satisfy the top, left, and top-left dependencies in Fig. 1 (as long as the top diagonals are a couple of positions ahead – see Fig. 4 as an example).

Fig.4 – Four parallel diagonal scans. As long as scans 1, 2, and 3 are ahead of 0, then all scans can be done in parallel.

# Simplified Context Selection for Significance Map

Note that given the context dependencies outlined in Fig.1, each diagonal scan will depend on three other diagonals.  The context dependencies can be simplified to reduce the number of dependent scans as shown in Fig. 5.

Fig.5 – Context dependency simplification to reduce the number of dependent scans.

# Experiment Results

The TMuC-0.7.3 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified in [3] are followed except that the length of the test runs are reduced to 65 frames.

## Wavefront Scan

To fix the scan direction, in TEncSbac.cpp (around line 1484)

//===== update scan direction ===== if( ( uiDownLeft == 1 && ( uiPosX == 0 ¦¦ uiPosY == uiHeight - 1 ) ) ¦¦ ( uiDownLeft == 0 && ( uiPosY == 0 ¦¦ uiPosX == uiWidth - 1 ) ) ) { // uiDownLeft = ( uiNumSigTopRight >= uiNumSigBotLeft ? 1 : 0 ); uiDownLeft = 1; // always Down Left }

Similarly, in TDecSbac.cpp (around line 1597)

//===== update scan direction ===== if( ( uiDownLeft == 1 && ( uiPosX == 0 ¦¦ uiPosY == uiHeight - 1 ) ) ¦¦ ( uiDownLeft == 0 && ( uiPosY == 0 ¦¦ uiPosX == uiWidth - 1 ) ) ) { //uiDownLeft = ( uiNumSigTopRight >= uiNumSigBotLeft ? 1 : 0 ); uiDownLeft = 1; // always Down Left }

As shown in the table below, the average loss from wavefront scan relative to adaptive scan in HHI_TRANSFORM_CODING ranges from 0.1% to 0.3%.  Detailed results in TI_Parallel_HHI_TRANSFORM_CODING..xls.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.3 | 0.3 |
| Class B | 0.2 | 0.4 | 0.5 |
| Class C | 0.1 | 0.3 | 0.3 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E | 0.1 | 0.1 | 0.1 |
| All | 0.1 | 0.2 | 0.3 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.4 | 0.2 |
| Class B | 0.2 | 0.4 | 0.5 |
| Class C | 0.1 | 0.1 | 0.5 |
| Class D | 0.1 | 0.8 | 0.5 |
| Class E |  |  |  |
| All | 0.2 | 0.4 | 0.4 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.3 | 0.6 | 0.5 |
| Class C | 0.2 | 0.7 | 0.6 |
| Class D | 0.3 | 1.3 | 0.6 |
| Class E | 0.2 | 0.0 | 0.0 |
| All | 0.3 | 0.7 | 0.5 |

Table 1  Average loss from using wavefront scan down left rather than adaptive scan. Positive numbers in the table mean wavefront scan results in coding loss.

## Simplified Context Selection

To reduce the number of dependencies for context selection from 3 to 2 scans and from 3 to 1 scan, as shown in Fig. 5, the following changes were made to TComTrQuant.cpp

UInt TComTrQuant::getSigCtxInc ( TCoeff* pcCoeff, const UInt uiPosX, const UInt uiPosY, const UInt uiLog2BlkSize, const UInt uiStride, const bool bDownLeft ) { UInt uiCtxInc = 0; UInt uiSizeM1 = ( 1 << uiLog2BlkSize ) - 1; . . . . . . . . . { const int* pData = &pcCoeff[ uiPosX + uiPosY * uiStride ]; int iStride = uiStride; int iStride2= iStride << 1; UInt uiCnt = ( pData[ -iStride ] ? 1 : 0 ); uiCnt += ( pData[ -1 ] ? 1 : 0 ); // uiCnt += ( pData[ -1 -iStride ] ? 1 : 0 );//A of scan 2 if( uiPosX > 1 ) { //uiCnt += ( pData[ -2 ] ? 1 : 0 ); //B of scan 2 //uiCnt += ( pData[ -2 -iStride ] ? 1 : 0 ); //B of scan 3 if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -2 +iStride ] ? 1 : 0 ); } } if( uiPosY > 1 ) { //uiCnt += ( pData[ -iStride2 ] ? 1 : 0 ); //C of scan 2 //uiCnt += ( pData[ -1 -iStride2 ] ? 1 : 0 ); //C of scan 3 if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride2 ] ? 1 : 0 ); } } if( bDownLeft ) { if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride ] ? 1 : 0 ); } } else { if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -1 +iStride ] ? 1 : 0 ); } } uiCtxInc = 10 + min<UInt>( 4, ( uiCnt + 1 ) >> 1 ); } return uiCtxInc; }

Figure 6 Source code (TComTrQuant.cpp) to process context selection for proposed context modeling of significance map.  Scan lines and position letters correspond to Fig. 5

Removing one scan line (from 3 to 2) results in 0 to 0.1% coding loss. Note: Wavefront scan in down-left direction is used for both. Detailed results in TI_Parallel_HHI_TRANSFORM_CODING..xls.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.1 |
| Class E | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.4 |
| Class B | 0.0 | -0.1 | 0.2 |
| Class C | 0.1 | 0.2 | 0.0 |
| Class D | 0.2 | -0.6 | 0.2 |
| Class E |  |  |  |
| All | 0.1 | -0.2 | 0.2 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | -0.1 | 0.1 |
| Class C | 0.0 | -0.5 | -0.3 |
| Class D | -0.2 | -0.7 | 0.0 |
| Class E | -0.1 | 0.8 | 0.5 |
| All | 0.0 | -0.2 | 0.0 |

Table 2  Average loss from reducing dependency to two scans compared to three scans dependency in HHI_TRANSFORM_CODING. Positive numbers in the table means coding loss.

Removing two scan lines (from 3 to 1) results in an average of 0.2% coding loss. Note: Wavefront scan in down-left direction is used for both anchor and test. Detailed results in TI_Parallel_HHI_TRANSFORM_CODING..xls.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.1 | 0.1 |
| Class B | 0.3 | 0.3 | 0.3 |
| Class C | 0.2 | 0.1 | 0.0 |
| Class D | 0.1 | 0.1 | 0.0 |
| Class E | 0.3 | 0.2 | 0.2 |
| All | 0.2 | 0.2 | 0.1 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.3 |
| Class B | 0.2 | -0.1 | 0.4 |
| Class C | 0.2 | 0.2 | -0.1 |
| Class D | 0.1 | -0.2 | 0.2 |
| Class E |  |  |  |
| All | 0.2 | 0.0 | 0.2 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.3 | -0.2 | -0.2 |
| Class C | 0.3 | -0.5 | -0.2 |
| Class D | 0.1 | -0.9 | 0.4 |
| Class E | 0.2 | 0.0 | 0.3 |
| All | 0.2 | -0.4 | 0.0 |

Table 3  Average loss from reducing dependency to one scans compared to three scans dependency in HHI_TRANSFORM_CODING. Positive numbers in the table means coding loss.

## Overall Coding Efficiency Impact

The overall impact of removing one scan line and having a wavefront scan in the down left direction over the anchor with all common condition settings is an average of 0.2% coding loss. Detailed results in TI_Parallel_HHI_TRANSFORM_CODING..xls.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.3 |
| Class B | 0.2 | 0.4 | 0.4 |
| Class C | 0.1 | 0.3 | 0.2 |
| Class D | 0.1 | 0.1 | 0.2 |
| Class E | 0.1 | 0.1 | 0.1 |
| All | 0.2 | 0.2 | 0.3 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.4 | 0.6 |
| Class B | 0.2 | 0.3 | 0.6 |
| Class C | 0.2 | 0.2 | 0.5 |
| Class D | 0.3 | 0.2 | 0.7 |
| Class E |  |  |  |
| All | 0.2 | 0.3 | 0.6 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.3 | 0.6 | 0.6 |
| Class C | 0.3 | 0.2 | 0.3 |
| Class D | 0.1 | 0.6 | 0.6 |
| Class E | 0.1 | 0.8 | 0.5 |
| All | 0.2 | 0.5 | 0.5 |

Table 4  Average loss from reducing dependency to one scans and wavefront scan down left compared to three scans dependency in HHI_TRANSFORM_CODING. Positive numbers in the table means coding loss.

These modifications to HHI_TRANSFORM_CODING enable it to still provide gains between 1.0 to 1.4% in a more parallel friendly manner.  Table below shows coding gains of the parallelized HHI_TRANSFORM_CODING. (with one scan removed). Detailed results in TI_Parallel_HHI_TRANSFORM_CODING..xls.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | 2.2 | 3.1 |
| Class B | -1.5 | 1.0 | 1.4 |
| Class C | -0.9 | 1.7 | 1.5 |
| Class D | -0.5 | 1.9 | 1.6 |
| Class E | -2.0 | 2.6 | 1.5 |
| All | -1.2 | 1.8 | 1.7 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | 1.5 | 1.7 |
| Class B | -1.5 | -0.4 | 0.4 |
| Class C | -1.1 | 0.4 | 0.5 |
| Class D | -0.5 | 0.6 | 0.9 |
| Class E |  |  |  |
| All | -1.0 | 0.3 | 0.7 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -1.8 | -1.0 | -0.6 |
| Class C | -1.3 | 0.4 | 0.3 |
| Class D | -0.8 | 1.3 | 1.4 |
| Class E | -1.8 | 2.9 | 2.9 |
| All | -1.4 | 0.6 | 0.8 |

Table 5  Average gain of parallelized HHI_TRANFORM_CODING with simplified context selection (reduced by one scan) and wavefront scan down left. Negative numbers in the table means coding gain.

With two scan removed, a gain between 0.9 to 1.2% can still be achieve with HHI_TRANSFORM_CODING as shown in the table below. Detailed results in TI_Parallel_HHI_TRANSFORM_CODING..xls.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | 2.4 | 3.2 |
| Class B | -1.2 | 1.3 | 1.7 |
| Class C | -0.7 | 1.8 | 1.5 |
| Class D | -0.4 | 2.0 | 1.6 |
| Class E | -1.8 | 2.8 | 1.7 |
| All | -1.0 | 2.0 | 1.8 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | 1.5 | 1.6 |
| Class B | -1.3 | -0.4 | 0.5 |
| Class C | -0.9 | 0.5 | 0.4 |
| Class D | -0.5 | 1.0 | 1.0 |
| Class E |  |  |  |
| All | -0.9 | 0.5 | 0.7 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -1.5 | -1.1 | -0.8 |
| Class C | -1.1 | 0.4 | 0.4 |
| Class D | -0.5 | 1.1 | 1.7 |
| Class E | -1.6 | 2.1 | 2.9 |
| All | -1.2 | 0.4 | 0.8 |

Table 6  Average gain of parallelized HHI_TRANFORM_CODING with simplified context selection (reduced by two scans) and wavefront scan down left. Negative numbers in the table means coding gain.

# Summary

In this contribution, we presented modifications to the scan order and context selection of significance map in HHI_TRANFORM_CODING that makes it easier to parallelize with negligible cost to the coding efficiency.  The parallelized HHI_TRANSFORM_CODING with simplified context selection (reduce dependency by one or two scans) and wavefront scan still provides a gain between 0.9 to 1.4% compared with not using HHI_TRANSFORM_CODING.

Based on the above findings, we propose that a tool experiment be launched on parallelization of HHI_TRANSFORM_CODING.

# References

- “TE12: Evaluation of transform coefficient coding (HHI_TRANSFORM_CODING)  with tool breakdown”, JCTVC-C059, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

- “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Measured for first 65 frames of all test sequences under common conditions.