| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C228 WG11 Number: m18268 |
| --- | --- |

| Title: | A simple cache model for measuring motion compensation bandwidth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Introduction

Motion compensation reference data accesses dominate the external memory bandwidth requirements in video decoder system. External memory access bandwidth requirements are expected to increase with HEVC since HEVC motion compensation interpolation filters are longer than those in AVC and since HEVC is expected to be used with larger resolution video such as 4Kx2K and 8Kx4K. External memory bandwidth used directly impacts power consumed and cost; in fact, power consumed due to external memory accesses in a video decoder can equal the power consumed by computational processing in video decoding. Hence techniques that reduce external memory bandwidth are required. Memory compression is one technique that is being studied in TE2 and Memory compression AhG for reducing external memory accesses. This contribution proposes a simple cache model for use in measuring external memory access bandwidth. The simple cache model is built on top of NEC’s memory bandwidth measurement module in TMuC-0.7 [1]. The main goal of this work was to have a simple cache model that captures the most relevant parameters for comparing bandwidth consumed with and without memory compression and that is fair to all block structures used in memory compression. The goal was not to develop a very sophisticated cache model. This cache model can also be used to compare memory bandwidth requirements of various interpolation filters. The cache model was developed as a part of Memory Compression AhG.

After a quick informal survey of existing L1 and L2 cache architectures on recent microprocessors and DSPs, a single level cache with following parameters was implemented in TMuC-0.7 decoder:

- Capacity (cache size): 128 KB

- Line size: 64 bytes

- Organization: 4-way set-associative

- Replacement strategy: FIFO (Least Recently Used (LRU) strategy is industry standard, but FIFO is easier to implement)

- DDR alignment: 64 bits

The software cache model was released to TE2 members after it was inspected by NEC and others. The memory bandwidth usage is printed in the same format used by NEC's memory bandwidth measurement module [1]. The code changes are isolated to be in #if MC_MEMORY_ACCESS_CACHE. MC_MEMORY_ACCESS_CALC needs to be enabled before cache can be used.

The spreadsheet provided by Panasonic [1] for collecting and comparing memory bandwidth was expanded to include comparison of cache data. A copy of the expanded spreadsheet in attached along with this contribution. The source code for cache model integrated into TMuC-0.7 is also attached along with this contribution.

# References

- K. Chono, T. Chujoh, C.S. Lim, “JCT-VC AHG report: Memory compression”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C007, 3nd Meeting: Guangzhou, CN, 7-17 October, 2010.