| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C230 |
| --- | --- |

| Title: | TE 12: Evaluation of 3-input-ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: martak@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the test results for the 3-input-ALF listed in Tool Experiment 12.

- Introduction

In the 2nd JCT-VC meeting, Tool Experiment 12 (TE11) was set up to explore the performance of all the tools in TMuC. This contribution is to test the 3-input-ALF.

The software version used for this test was TMuC 0.7. The reference was generated by using the default setting [2]. For high complexity case, original QC_ALF was enabled as a default setting. The reference was compared against the alternative setting with the 3-input-ALF on. For low complexity case, ALF was disabled as a default setting. The reference was compared against the alternative setting with the 3-input-ALF on. The executables for encoding and decoding were compiled on 32-bit Linux and the simulations were performed on the Linux cluster. The results are matching with MediaTek’s results.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.4 | 0.1 | 0.1 | -6.0 | -2.4 | -2.1 |
| Class B | -1.8 | 0.1 | 0.1 |  |  |  |
| Class C | -1.1 | 0.1 | 0.1 | -2.8 | -2.8 | -3.9 |
| Class D | -0.9 | 0.1 | 0.1 | -1.8 | -2.6 | -3.1 |
| Class E | -1.3 | 0.1 | 0.1 | -4.2 | -5.2 | -5.3 |
| All | -1.4 | 0.1 | 0.1 |  |  |  |
| Enc Time[%] | 161% |  |  | 215% |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.8 | 0.2 | 0.1 | -5.4 | -1.2 | -1.0 |
| Class B | -1.7 | -0.1 | -0.1 | -6.0 | -3.4 | -2.6 |
| Class C | -0.9 | -0.1 | 0.0 | -5.2 | -1.7 | -2.7 |
| Class D | -0.8 | 0.0 | 0.1 | -6.7 | -2.0 | -2.3 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -0.1 | 0.0 | -5.9 | -2.3 | -2.3 |
| Enc Time[%] | 109% |  |  | 146% |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 0.0 | 0.1 | -5.9 | -3.4 | -4.0 |
| Class C | -0.6 | -0.1 | -0.1 | -5.5 | -3.1 | -3.7 |
| Class D | -0.6 | 0.0 | 0.1 | -5.3 | -4.8 | -4.9 |
| Class E | -1.2 | -0.5 | -0.3 | -3.7 | -4.4 | -2.8 |
| All | -0.8 | -0.1 | 0.0 | -5.2 | -3.9 | -3.9 |
| Enc Time[%] | 107% |  |  | 154% |  |  |
| Dec Time[%] |  |  |  |  |  |  |

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).