| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C234 WG11 Number: m18274 |
| --- | --- |

| Title: | Simplified Intra Smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yunfei Zheng Muhammed Coban Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5673 martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, a simplified intra smoothing scheme is proposed to improve the current AIS scheme. In the proposed scheme, the prediction samples are filtered according to the prediction unit size and the intra prediction mode. The proposed scheme achieves 0.36% and 0.43% BD-rate reduction on average for high efficiency and low complexity intra configuration respectively. The encoding complexity is only about 50% comparing to TMuC 0.8 default setting.

- Introduction

Adaptive intra smoothing (AIS) was proposed in [1]. The method explicitly signals whether the prediction samples are filtered. In TMuC v0.7, the encoder makes the smoothing decision by testing the RD cost of intra prediction based on filtered and unfiltered prediction samples. An alternative fast AIS scheme that only tests the filtering for the best intra prediction mode, is also available[2].

In this contribution, the filtering decision is based on the prediction unit size and the intra prediction mode of the coded block.

# Proposed scheme

In the current TMuC software, intra prediction unit size can be 2x2, 4x4, 8x8, 16x16, 32x32, 64x64, and 128x128. There are a total of 33 directional intra prediction modes. Each directional prediction mode corresponds to an extrapolation direction, which is shown in  . The number of directional prediction modes depends on the prediction unit size.

Figure 2-1 Directional intra prediction modes

Table 2-1 Intra prediction modes

| PU Size | Intra prediction modes |
| --- | --- |
| 2x2 | 0~3 |
| 4x4 | 0 ~ 16 |
| 8x8 | 0 ~ 33 |
| 16x16 | 0 ~ 33 |
| 32x32 | 0 ~ 33 |
| 64x64 | 0 ~ 5 |
| 128x128 | 0 ~ 5 |

As shown in , the directions of intra prediction modes are hierarchically distributed based on prediction angle resolutions. The zones in  correspond to different angular resolution zones.  For example, when the angular resolution is equal to 90 degree (zone 0), it generates mode 0 (VER) and 1 (HOR) (all modes within zone 0); reducing the angle resolution  further (zone 1), it generates mode 3 (VER-8), 6 (VER+8), and 9 (HOR+8) additionally (all modes within zone 1); futher reducing the angle resolution in the next stage (zone 2), it generates mode 4 (VER-4), 5 (VER+4), 7 (HOR-4), 8 (HOR+4).

According to mode indexing specification defined in TMuC [3], the proposed intra smoothing scheme can be summarized by Table 2-2. In this method, modes 0, 1, and 2 are never filtered. The filtering decision of other modes depends on the PU size and the mode index.

Table 2-2  Filtered modes

| PU Size | Modes need to be filtered | Shown in Figure 2-1 |
| --- | --- | --- |
| 2x2 | no filtering for all modes | n/a |
| 4x4 | mode 3, 6, and 9 | modes between zone 0 and zone 1 |
| 8x8 | mode 3, 4, 5, 6, 7, 8, and 9 | modes between zone 0 and zone 2 |
| 16x16 | mode 3 to 17 | modes between zone 0 and zone 3 |
| 32x32 | mode 3 to 33 | modes between zone 0 and zone 4 |
| 64x64 | mode 3 to 4 | modes between zone 0 and zone 4* |
| 128x128 | mode 3 to 4 | modes between zone 0 and zone 4*Error: Reference source not found |

# Simulations & Results

The proposed simplified intra smoothing is implemented on top of TMuC v0.8. The common simulation conditions are:

- Reference:  TMuC v0.8, DEFAULT_IS = 0, AIS = 0 (no filtering)

- QP = {22, 27, 32, 37}

- Test sequences:  Class A, B, C, D, E with full sequence length

- High efficiency and low complexity Intra only defined in [4]

Simulation #1: TMuC 0.8 with DEFAULT_IS=1, AIS=1 (slow AIS)

Simulation #2: TMuC 0.8 with DEFAULT_IS=1, AIS=1 and AIS_TEST_BEST=1 (fast AIS)

Simulation #3: TMuC 0.8 with the proposed method

Table 3-3 and Table 3-4 show the comparison results in high efficiency and low complexity intra coding respectively.

Table 3-3  BD Bitrate saving comparisons to TMuC0.8 with DEFAULT_IS=0, AIS=0

| Intra High Efficiency |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Sequence | DEFAULT_IS=1, AIS=1 slow |  |  | DEFAULT_IS=1, AIS=1, fast |  |  | Proposed |  |  |
|  |  | BD-rate Y(%) | BD-rate U (%) | BD-rate V(%) | BD-rate Y(%) | BD-rate U (%) | BD-rate V(%) | BD-rate Y(%) | BD-rate U (%) | BD-rate V(%) |
| Class A, 4K | S01 Traffic | -0.57 | 0.17 | 0.14 | -0.10 | 0.54 | 0.51 | -0.43 | -0.16 | -0.17 |
|  | S02 PeopleOnStreet | -0.55 | 0.46 | 0.41 | -0.05 | 0.96 | 0.85 | -0.54 | -0.13 | -0.20 |
| Class B, 1080p | S03 Kimono | -0.61 | -0.21 | -0.30 | -0.55 | -0.21 | -0.26 | -0.39 | -0.21 | -0.26 |
|  | S04 ParkScene | -1.16 | -0.17 | -0.22 | -1.13 | -0.13 | -0.13 | -0.81 | -0.16 | -0.18 |
|  | S05 Cactus | -0.70 | 0.06 | 0.05 | 0.05 | 0.71 | 0.74 | -0.61 | -0.23 | -0.28 |
|  | S06 BasketballDrive | -0.67 | 0.10 | 0.10 | 0.96 | 1.35 | 1.39 | -0.40 | -0.23 | -0.19 |
|  | S07 BQTerrace | -0.54 | -0.03 | -0.29 | 0.38 | 1.00 | 0.98 | -0.09 | 0.24 | 0.32 |
| Class C, WVGA | S08 BasketballDrill | -0.16 | 0.48 | 0.46 | 2.28 | 2.92 | 2.89 | 0.50 | 0.66 | 0.59 |
|  | S09 BQMall | -0.23 | 0.52 | 0.50 | 1.60 | 2.18 | 2.08 | -0.38 | -0.17 | -0.19 |
|  | S10 PartyScene | -0.27 | 0.33 | 0.27 | 1.61 | 1.91 | 1.83 | -0.32 | -0.25 | -0.24 |
|  | S11 RaceHorses | -0.83 | -0.21 | -0.12 | -0.19 | 0.32 | 0.48 | -0.64 | -0.27 | -0.35 |
| Class D, WQVGA | S12 BasketballPass | -0.15 | 0.54 | 0.60 | 1.46 | 1.97 | 1.99 | -0.15 | 0.00 | 0.01 |
|  | S13 BQSquare | -0.02 | 0.58 | 0.57 | 1.61 | 1.91 | 2.01 | -0.16 | -0.04 | -0.02 |
|  | S14 BlowingBubbles | -0.31 | 0.36 | 0.36 | 1.04 | 1.40 | 1.48 | -0.30 | -0.24 | -0.28 |
|  | S15 RaceHorses | -0.40 | 0.23 | 0.34 | 0.43 | 1.01 | 1.07 | -0.46 | -0.28 | -0.34 |
| ClassE, 720p | S16 Vidyo1 | -0.11 | 0.56 | 0.48 | 1.09 | 1.81 | 1.68 | -0.16 | 0.05 | 0.11 |
|  | S17 Vidyo3 | -0.66 | 0.21 | 0.09 | 1.18 | 1.53 | 1.68 | -0.67 | 0.44 | -0.26 |
|  | S18 Vidyo4 | -0.31 | 0.42 | 0.41 | 1.13 | 1.71 | 1.73 | -0.42 | -0.13 | -0.04 |
| Class A |  | -0.56 | 0.32 | 0.28 | -0.07 | 0.75 | 0.68 | -0.49 | -0.15 | -0.19 |
| Class B |  | -0.74 | -0.05 | -0.13 | -0.06 | 0.54 | 0.54 | -0.46 | -0.12 | -0.12 |
| Class C |  | -0.37 | 0.28 | 0.28 | 1.32 | 1.83 | 1.82 | -0.21 | -0.01 | -0.05 |
| Class D |  | -0.22 | 0.43 | 0.47 | 1.14 | 1.57 | 1.64 | -0.27 | -0.14 | -0.16 |
| Class E |  | -0.36 | 0.40 | 0.33 | 1.13 | 1.68 | 1.69 | -0.42 | 0.12 | -0.06 |
| All Ave. |  | -0.46 | 0.25 | 0.21 | 0.71 | 1.27 | 1.28 | -0.36 | -0.06 | -0.11 |
| Enc. Time (proposal / anchor) |  | 1.76 |  |  | 1.08 |  |  | 0.94 |  |  |

Table 3-4  BD Bitrate saving comparisons to TMuC0.8 with DEFAULT_IS=0, AIS=0

| Intra Low Complexity |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Sequence | DEFAULT_IS=1, AIS=1 slow |  |  | DEFAULT_IS=1, AIS=1, fast |  |  | Proposed |  |  |
|  |  | BD-rate Y(%) | BD-rate U (%) | BD-rate V(%) | BD-rate Y(%) | BD-rate U (%) | BD-rate V(%) | BD-rate Y(%) | BD-rate U (%) | BD-rate V(%) |
| Class A, 4K | S01 Traffic | -1.32 | 0.77 | 0.60 | -0.69 | 1.26 | 1.02 | -0.71 | -0.07 | -0.08 |
|  | S02 PeopleOnStreet | -1.34 | 0.60 | 0.47 | -0.62 | 1.18 | 1.05 | -0.86 | -0.16 | -0.21 |
| Class B, 1080p | S03 Kimono | -0.96 | 0.39 | 0.31 | -0.74 | 0.58 | 0.50 | -0.65 | -0.14 | -0.16 |
|  | S04 ParkScene | -1.36 | 0.68 | 0.53 | -0.89 | 0.95 | 0.83 | -0.95 | -0.14 | -0.15 |
|  | S05 Cactus | -0.94 | 0.75 | 0.73 | 0.21 | 1.86 | 1.86 | -0.70 | 0.00 | 0.01 |
|  | S06 BasketballDrive | -1.11 | 0.54 | 0.61 | 0.49 | 2.60 | 2.55 | -0.49 | -0.07 | -0.03 |
|  | S07 BQTerrace | -0.71 | 0.43 | 0.14 | 0.45 | 1.72 | 1.45 | -0.03 | 0.64 | 0.70 |
| Class C, WVGA | S08 BasketballDrill | -0.12 | 1.03 | 0.89 | 2.11 | 3.44 | 3.38 | 0.98 | 0.59 | 0.52 |
|  | S09 BQMall | -0.32 | 1.17 | 1.15 | 1.37 | 2.61 | 2.56 | -0.41 | -0.02 | -0.04 |
|  | S10 PartyScene | -0.34 | 0.68 | 0.70 | 1.38 | 1.91 | 1.88 | -0.33 | -0.18 | -0.14 |
|  | S11 RaceHorses | -0.96 | 0.53 | 0.47 | 0.02 | 1.44 | 1.52 | -0.71 | -0.16 | -0.12 |
| Class D, WQVGA | S12 BasketballPass | -0.38 | 1.14 | 1.17 | 1.21 | 2.61 | 2.55 | -0.23 | 0.09 | 0.13 |
|  | S13 BQSquare | -0.17 | 0.75 | 0.70 | 1.51 | 2.04 | 2.10 | -0.15 | 0.05 | 0.02 |
|  | S14 BlowingBubbles | -0.50 | 0.72 | 0.71 | 1.07 | 1.82 | 1.84 | -0.28 | -0.08 | -0.12 |
|  | S15 RaceHorses | -0.64 | 1.03 | 0.93 | 0.48 | 1.95 | 1.89 | -0.51 | 0.05 | -0.02 |
| ClassE, 720p | S16 Vidyo1 | -0.96 | 0.82 | 0.88 | 0.27 | 2.14 | 2.12 | -0.53 | 0.17 | 0.23 |
|  | S17 Vidyo3 | -0.99 | 0.48 | 0.85 | 0.34 | 1.69 | 2.08 | -0.64 | -0.02 | -0.03 |
|  | S18 Vidyo4 | -0.85 | 0.65 | 0.80 | 0.42 | 1.74 | 1.77 | -0.62 | -0.11 | -0.17 |
| Class A |  | -1.33 | 0.69 | 0.53 | -0.65 | 1.22 | 1.03 | -0.78 | -0.11 | -0.14 |
| Class B |  | -1.01 | 0.56 | 0.46 | -0.10 | 1.54 | 1.44 | -0.56 | 0.06 | 0.07 |
| Class C |  | -0.43 | 0.85 | 0.80 | 1.22 | 2.35 | 2.34 | -0.12 | 0.06 | 0.06 |
| Class D |  | -0.42 | 0.91 | 0.88 | 1.07 | 2.11 | 2.09 | -0.29 | 0.02 | 0.00 |
| Class E |  | -0.93 | 0.65 | 0.84 | 0.34 | 1.86 | 1.99 | -0.60 | 0.01 | 0.01 |
| All Ave. |  | -0.78 | 0.73 | 0.70 | 0.47 | 1.86 | 1.83 | -0.43 | 0.02 | 0.02 |
| Enc. Time (proposal / anchor) |  | 1.79 |  |  | 1.07 |  |  | 0.99 |  |  |

# Conclusion

In this contribution, a simplified intra smoothing scheme is proposed. The proposed scheme is based on a simple but efficient partitioning rule, which avoid encoder complexity and additional signaling overhead. The proposed scheme achieves 0.36% and 0.43% BD-rate reduction on average for high efficiency and low complexity intra configuration respectively. The encoding complexity is only about 50% comparing to TMuC 0.8 default setting.

# Reference

- Fraunhofer HHI, “Description of video coding technology proposal by Fraunhofer HHI”, JCT-VC, 1st Meeting, Dresden, Deutschland, April 2010, Doc. JCTVC-A116.

- B. Bross (Fraunhofer HHI), “TE12.3: Adaptive intra smoothing (AIS) test (slow vs. fast) by Fraunhofer HHI, JCT-VC 3rd Meeting, Guangzhou, China, Oct 2010, Doc. JCTVC-C053.

- “Test Model under Consideration”, JCT-VC, 2nd Meeting, Geneva, Switzerland, July 2010, Doc. JCTVC-B205.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

For 64x64 and 128x128, there are only 5 intra prediction modes. All of them except mode 0, 1 and 2 are filtered. To make our rule more general, the rule is defined that all modes between circle 0 and circle 4 will be filtered for these two PU size though there are some modes that is not available for the PU size.