| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C235 |
| --- | --- |

| Title: | TE 12: Evaluation of Adaptive in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: martak@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the test results for the adaptive in-loop filter tool listed in Tool Experiment 12: Evaluation of TMuC Tools.

- Introduction

In the 2nd JCT-VC meeting, Tool Experiment 12 (TE12) was set up to explore the performance of the individual tools in the Test Model under Consideration (TMuC). This contribution is to test the adaptive in-loop filter tool of TE 12.

The software version used for this test was TMuC 0.7. The reference was generated by using the default setting [2]. For the high complexity case, the adaptive in-loop filter was enabled in the default setting, and the reference was compared against the alternative setting with the adaptive in-loop filter disabled (by setting ALF=0). For the low complexity case, the adaptive in-loop filter was disabled in the default setting, and the reference was compared against the alternative setting with the adaptive in-loop filter enabled  (by setting ALF=1).The executables for encoding and decoding were compiled on 32-bit Linux and the simulations were performed on the Linux cluster.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.2 | 3.7 | 3.3 | -4.1 | -2.5 | -2.2 |
| Class B | 2.1 | 3.0 | 2.6 | -2.1 | -2.8 | -2.2 |
| Class C | 2.0 | 3.3 | 4.3 | -1.8 | -2.8 | -3.9 |
| Class D | 0.9 | 3.4 | 3.9 | -0.9 | -2.5 | -3.0 |
| Class E | 2.9 | 6.2 | 6.5 | -3.1 | -5.3 | -5.4 |
| All | 2.2 | 3.8 | 4.0 | -2.2 | -3.1 | -3.3 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 82% |  |  | 122% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.9 | 3.6 | 2.6 | -3.9 | -1.2 | -1.1 |
| Class B | 5.8 | 10.6 | 7.6 | -4.7 | -3.4 | -2.5 |
| Class C | 2.6 | 3.8 | 5.7 | -4.4 | -1.6 | -2.8 |
| Class D | 1.7 | 4.9 | 6.4 | -5.9 | -2.1 | -2.4 |
| Class E |  |  |  |  |  |  |
| All | 3.7 | 6.4 | 6.1 | -4.8 | -2.3 | -2.4 |
| Enc Time[%] | 97% |  |  | 102% |  |  |
| Dec Time[%] | 86% |  |  | 110% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 6.0 | 3.4 | 3.2 | -4.4 | -3.1 | -3.6 |
| Class C | 4.1 | 2.9 | 4.1 | -4.9 | -3.0 | -3.5 |
| Class D | 2.7 | 2.9 | 3.0 | -4.5 | -4.5 | -4.7 |
| Class E | 8.5 | 3.2 | 5.6 | -2.9 | -4.1 | -2.2 |
| All | 5.2 | 3.1 | 3.8 | -4.3 | -3.6 | -3.6 |
| Enc Time[%] | 99% |  |  | 96% |  |  |
| Dec Time[%] | 86% |  |  | 115% |  |  |

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).