| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C236 WG11 Number: m18277 |
| --- | --- |

| Title: | Report of complexity analysis of geometric partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kenneth Vermeirsch Jan De Cock Rik Van de Walle Gaston Crommenlaan 8 P.O. Box 201 9050 Ghent-Ledeberg Belgium | Tel: Email: | +32 93314957 kenneth.vermeirsch@elis.ugent.be |
| Source: | Ghent University -- IBBT |  |  |

_____________________________

# Abstract

This is a report on a complexity analysis which was carried out at the Multimedia Lab of Ghent University some time ago.  In this analysis a geometric partition mode is defined in function of a partition boundary line having a certain orientation (angle) and offset from the block center, as in Figure 1.  First, the complexity and compression gains of geometric partitioning were determined as a function of the number of allowed orientations.  Secondly, the analysis investigated the histograms of the location of the partition boundary within blocks w.r.t. the block center.  Based on these observations a strategy for complexity reduction can be put forward.  By only testing a subset of partition modes, the complexity can be reduced by a factor of 40 while retaining over 50% of the gains of the full set.

Figure 1:  Definition of ‘angle’ and ‘offset’ parameters used for signaling the location of the partition boundary relative to the center of the block.

This analysis was carried out on blocks of 16x16 pixels.  The authors believe that the conclusions hold for bigger block sizes as well, at least qualitatively.  The actual trade-off points between complexity and performance may be different, though.

- Introduction

In light of the renewed interest in geometric partitioning within TE3, it is important to look not only at compression efficiency; one must be attentive to complexity as well.  The encoder-side complexity of geometric partitioning can be excessive if too much freedom is allowed in the parameters.  It makes sense to analyse all possible modes and disallow those which do not bring a lot of additional compression gains, as these will almost never be selected by the mode decision process but need to be tested nonetheless.  A second advantage of employing a lower number of modes is a reduced signaling overhead.  This advantage would have been absent if the complexity problem were to be solved by some fast decision algorithm (such as edge detection based partitioning).

This report aims primarily to increase the level of understanding of how the different geometric modes compete with each other in the mode decision process and to supply a rough indication of how much R-D degradation can be expected for a given encoder complexity decrease.  Only highlights are given in this document; for the complete study, we refer to [1].

- Number of different orientations of the partition boundary

First an experiment was carried out to confirm that the optimal directions of the partition boundary are uniformly distributed along the circle.

The actual performance vs. complexity trade-off in the number of permissible angles can best be explained using Figure 2. We have separated the curves for compression and complexity in order not to imply that a crossover point would exist. These curves show clearly that most of the compression gain is obtained using just a few well-chosen angles, while encoder complexity scales approximately linearly with the number of allowed angles. In practice, it is up to the implementer or to the requirements of the application scenario to determine how much encoder complexity is tolerable.

We believe that for higher block sizes the tradeoff may favor a higher number of angles more than in the case of 16x16 blocks, since the ratio of signaling overhead bits to the number of residual samples in the block is more favorable for larger block sizes.  However, the saturation effect we notice in Figure 2 will also be present in the case of larger block sizes.

Figure 2: Compression performance and complexity compared against the number of permissible partition boundary orientations values. While complexity increase is roughly linear, most of the compression gains are obtained using just a few well-chosen orientations (horizontal, vertical, and diagonal).

- Histogram analysis of the partition boundary location

To reduce the number of modes by constraining the offset parameter r, three strategies may be taken:

- Increase the quantization step size for r;

- Restrict the minimum and maximum representation values for r, thereby quantizing all values 
r < rmin or r > rmax to this minimum or maximum representation value, respectively;

- A combination of Strategies 1 and 2.

The process of choosing the best strategy must be supported by empirical data. Much insight can be gained from measurements of the relative occurrence of values of the offset parameter r, which we present for the Foreman sequence in Figure 3. Similar results are obtained for other test sequences. A number of observations can now be made.4

Figure 3: Selection frequencies for offset parameter r for partition boundary orientations of +45 (top left) and -45 (bottom left) degrees and for horizontal (top right) and vertical (bottom right) boundaries.

First, the data show that for the diagonal modes, the higher values of |r| are almost never selected. This can be explained by the fact that the area of the smallest subpartition decreases quadratically with increasing |r|; such a small subpartition has a limited impact on the rate and distortion of the residual signal within the entire block, but it generates an extra motion vector. Hence, diagonal partition modes where |r| is large will not be selected very often, and Strategy 2 or 3 should be deployed for the diagonal modes.

Figure 4 shows the results of applying Strategy 3 to the diagonal modes in the Foreman test sequence. The sequence was coded once with all diagonal modes and then again with just those modes for which r ∈ {0, ±2, ±3, ±6}. The curves show the rate-distortion results for each setting while the numbers in parenthesis indicate the required number of adaptive-shape partition modes for each curve. Clearly the same trend that we observed in Fig. 2 when we studied the number of angle representations is also present here: in this example, using only 14 modes yields 79% of the rate-distortion gain attained when using all 62 diagonal modes.

Figure 4: Applying Strategy 3 to the diagonal modes reduces the number of modes over fourfold while retaining 79% of the coding gain

Secondly, for the horizontal and vertical modes, the situation is different. The area of the smallest subpartition decreases only linearly with increasing |r|, and all modes are selected to a certain extent. (A notable outlier is that the modes |r| = 1 have a relatively low occurrence, because the adjacent r = 0 modes are signaled with a shorter code — these are the classical 16×8 and 8×16 modes.) Thus, Strategy 1 or 3 would seem more appropriate for the horizontal and vertical modes. Fig. 5 illustrates the application of Strategy 1 to the Foreman test sequence, with quantization step sizes ∆r of 1 through 4. Again, we observe the same trend as before: in this case, only 6 partition modes yield about half of the rate-distortion gain attained when using all 30 modes.

Figure 5: Application of Strategy 1 to the horizontal and vertical partition modes, with quantization step sizes ∆r of 1 through 4. A five-fold decrease in number of modes still attains half of the full coding gain.

- References

[1] K Vermeirsch, J De Cock, S Notebaert, P Lambert, J Barbarien, A Munteanu, and R Van de Walle, “Efficient Adaptive-shape Partitioning of Video,” in Multimedia Tools and Applications, Springer, 2010.  Published online http://dx.doi.org/10.1007/s11042-010-0593-0.

# Patent rights declaration(s)

Ghent University and/or IBBT may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).