| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C238 WG11 Number: m18158 |
| --- | --- |

| Title: | TE12.8: Results on RDOQ in high efficiency settings |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinwen Zan Jin Meng Mohammad Towhidul Islam Dake He 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 jzan@rim.com dhe@rim.com |
| Source: | Research in Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results of the cross-checking activities performed by RIM on RDOQ (described in JCTVC-B312) for all three high-efficiency settings.

- Tested tools:

According to TE12 description, the following tools of JCTVC-B312 have been tested:

- Software Specification: TMuC 0.7 Version with RDOQ option on and off.

- Platform Specification:

  - MAC OS X (64 bit) (all sequences in intra and low-delay settings, and all Class A sequences in random-access setting.)

  - Windows XP (32 bit) (All sequences except Class A sequences in random access setting.)

# Cross verification results

Test results have been compared to those generated by Ericsson. For “intra” and “lowdelay” modes, RIM test results matches Ericsson’s; for the “random access” mode, there is minor (less than 0.1%) differences on four sequences: 1) the PeopleOnStreet sequence on the V component; 2) the Kimono sequence on the U component; 3) the ParkScene sequence on the U and V component; and 4)the RaceHorsesC sequence on the V component. Overall, the difference is very minor and all the data on the summary tab of the results spreadsheet matched each other.

# Tool analysis

The test results have shown that RDOQ is an effective technique to enhance the coding performance (Rate and PSNR) of video coding algorithm. Specifically, for intra only setting, the gain by using RDOQ on Y-component is on average 4.7% (in BD-rate); for random access setting, the gain on Y-component is on average 4.4%; and for low-delay setting, the gain on Y-component is on average 2.5%.  It can also be observed that RDOQ offers the largest coding performance gain on Class B sequences.

# Patent rights declaration(s)

Research In Motion Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).