| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C240 WG11 Number: m18281 |
| --- | --- |

| Title: | TE3 - Cross-check results of Local Intensity Compensation tool from Mitsubishi Electric |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report on Subtest 3 Local Intensity Compensation tool of Tool Experiment 3: Inter Prediction in HEVC] |  |  |
| Author(s) or Contact(s): | Damian Karwowski ul. Polanka 3 61-131 Poznań, POLAND | Tel: Email: | +48 61 665 38 44 dkarwow@multimedia.edu.pl |
| Source: | [Poznań University of Technology (PUT), Chair of Multimedia Telecommunications and Microelectronics, Poznań, Poland] |  |  |

_____________________________

# Abstract

The contribution presents experimental results on coding efficiency and computational complexity for Local Intensity Compensation (LIC) tool proposed by Mitsubishi Electric [1]. Experiments were done in the context of Subtest 3: Multi-hypothesis inter prediction defined in Tool Experiment 3: Inter Prediction in HEVC [4]. The aim of experiments was to cross check the results obtained by authors of the method (Mitsubishi Electric).

- Introduction – short review of LIC tool

Local Intensity Compensation technique was presented in [1] in detail. The main idea behind the method is to predict the current image block with some number of reference blocks, where the reference blocks can be arbitrary weighted. The method also assumes adding the offset to such obtained weighted prediction. The idea of the method was presented in Fig. 1.

Figure 1. The idea of Local Intensity Compensation technique [source: 4]

In more detail, the LIC operation can be formulated as:

,

where Bp is the current block prediction, Bcn is n-th reference block, o is offset and wn is the weight associated with block Bcn. N relates to the number of reference blocks. The method was defined to operate at the prediction unit (PU) level, where for each block unit in a partition a different set of parameters can be derived and transmitted to the decoder. Parameters are calculated by minimization of a cost function.

- Methodology of experiments

The experiments have been aimed at testing the coding efficiency of Local Intensity Compensation (LIC) tool [1] in the framework of TMuC software [3]. The computational complexity of video encoder and video decoder with LIC tool were also tested. Experiments were done under test conditions presented in the document of Tool Experiment 3 [4] (test sequences as defined in the CfP document [2] were used, original and the modified TMuC software with LIC tool was evaluated, video encoders were set up with configuration files provided by the TMuC software group [3]).

PUT received the software of modified TMuC with LIC tool from Mitsubishi Electric. The source code was analyzed. Experiments were done with 0.7.1 version of TMuC software. Both the original and the modified TMuC codecs were built from source code with Microsoft Visual Studio 2005 Professional Edition under the Windows 7 64-bit operation system. The compression performance and computational complexity tests were carried out on machines with parameters:

- Computer 1: Core 2 Duo E6600 (2.4GHz), 4GB of RAM, Windows 7 64-bit;

- Computer 2: QuadCore Intel Core i7 920, 2.833 GHz, 12GB of RAM, Windows Vista 7 ultimate.

In each experiment the bitrate, Y PSNR, U PSNR, V PSNR for encoded sequences were reported. Additionally, encoding and decoding times were reported. Obtained results were presented in next sections.

- Experimental results on coding efficiency

In this section, results on compression performance of LIC tool were presented. Due to the time constraints, experiments were done only for ‘lowdelay_loco’ and ‘randomaccess_loco’ encoder configurations. Detailed results were presented in Table1 and Table 2.

Table 1. Experimental results for Random Access Low Complexity scenario.

Table 2. Experimental results for Low Delay Low Complexity scenario.

The results were compared with results obtained by Mitsubishi Electric. The conclusion is that presented results match the results of Mitsubishi Electric.

- Experimental results on computational complexity

This section presents results on complexity of video codec with and without LIC tool. Only results for class D encoded in ‘lowdelay_loco’ and ‘randomaccess_loco’ scenarios were reported. For other cases, due to the time constraints experiments were carried out by running the number of encoders that exceeded total number of processor cores. These results would not be reliable, so were not presented in the document. Computational complexity results for class D for ‘lowdelay_loco’ and ‘randomaccess_loco’ encoder configurations were presented in Table 3.

Table 3. Complexity of TMuC codec with LIC tool.

- Conclusions

In general, application of LIC tool in TMuC leads to increase the BD-rate Y metrics (‘randomaccess_loco’: 1.1; 0.4; 0.4 on average for class B, C, D respectively, ‘lowdelay_loco’: 0.2; 0.3; 0.4 on average for class C, D, E respectively). The obtained results match the results of Mitsubishi Electric. Application of LIC technique increases complexity of TMuC encoder by 6.5% and complexity of TMuC decoder by 1.25%.

# Patent rights declaration(s)

Poznań University of Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

| [1] | N. Sprljan, S. Paschalakis, P. Wu, “Local intensity compensation for inter prediction in HEVC,” Doc. JCTVC-B096, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010 |
| --- | --- |
| [2] | ISO/IEC JTC1/SC29/WG11, “Joint Call for Proposals on Video Compression Technology,” MPEG Document N11113, Jan 2010 |
| [3] | F. Bossen, “Common test conditions and software reference configurations,” Doc. JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010 |
| [4] | A. Krutz, et. al. “Tool Experiment 3: Inter Prediction in HEVC,” Doc. JCTVC-B303, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010 |