| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C241 WG11 Number: m18282 |
| --- | --- |

| Title: | TE 12: Report on Interpolation Filter : SIFO(DIF) / MOMS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rahul Panchal, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-658-4410 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the test results for Interpolation filter tool listed in Tool Experiment 12: Evaluation of TMuC Tools.

- Introduction

In the 2nd JCT-VC meeting, Tool Experiment 12 (TE12) was set up to explore the performance of the individual tools in the Test Model under Consideration (TMuC). This contribution is to test the MOMS 6 tap interpolation filter tool of TE 12.

The software version used for this test was TMuC 0.7. The reference was generated by using the default setting [2]. In the default setting, for High Efficiency tests, SIFO 12 Tap is used and for Low Complexity tests, Diagonal Interpolation Filters were used. The reference was compared against the alternative setting with the interpolation filter set to MOMS 6 tap (by setting InterpFilterType=2). The executables for encoding and decoding were compiled on 32-bit Linux and the simulations were performed on the Linux cluster. The summary of experimental results is presented below. It should be noted that the simulations were performed on a cluster, where the computational power of each node was different. Therefore the encoding/decoding time results are not very reliable.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | -0.6 | -0.9 | 0.6 | -5.7 | -6.9 |
| Class B | 0.2 | -3.6 | -3.0 | -1.1 | -13.4 | -15.8 |
| Class C | -1.2 | -5.7 | -5.4 | -6.9 | -14.2 | -13.7 |
| Class D | -2.2 | -7.7 | -9.5 | -13.8 | -19.4 | -21.6 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -4.8 | -5.1 | -5.8 | -14.2 | -15.6 |
| Enc Time[%] | 61% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 119% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.1 | 7.8 | 12.4 | -1.8 | 1.0 | 11.8 |
| Class C | -2.1 | -1.3 | -2.0 | -8.2 | -11.1 | -9.2 |
| Class D | -4.6 | -2.7 | -3.9 | -15.5 | -12.6 | -14.3 |
| Class E | 12.4 | 49.5 | 71.0 | 0.9 | 59.8 | 30.4 |
| All | 1.0 | 10.7 | 15.7 | -6.3 | 5.6 | 3.5 |
| Enc Time[%] | 66% |  |  | 108% |  |  |
| Dec Time[%] | 114% |  |  | 126% |  |  |

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Qualcomm  may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).