| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C244 WG11 Number: m18285 |
| --- | --- |

| Title: | TE12.3: Report on planar mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | David Flynn | Email: | davidf@rd.bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

TE12 proposes to evaluate the performance of individual tools currently proposed in TMuC.  This report presents the configuration and results from testing the intra planar prediction mode.  When this mode is enabled, there is, for most configurations, no significant change in BD-Rate, with low-complexity configurations, there is a loss of between 0.9% (Intra) and 0.2% (Random Access) BD-Rate.  With all configurations, there is a reduction in encoder runtime associated with enabling the intra planar prediction mode.

# Encoding environment: Anchor generation

The anchors used as the reference in comparisons were generated using TMuC 0.7 on a 64bit linux cluster comprised of:

| Class | System |
| --- | --- |
| A | AMD Opteron (k10) 2.4-2.6GHz, 16 GiB RAM |
| B,C,D,E | Intel X3450 (nehalem) 2.66GHz, 4 GiB RAM |

Binaries were produced using a custom makefile to allow an out-of-tree build with gcc-4.4.5. Two sets of binaries were produced to allow for machine specific build optimizations.  Specifically:

| System | CXXFLAGS |
| --- | --- |
| AMD | -O3 -march=amdfam10 |
| Intel | -O3 -march=core2 |

The generated anchors match those provided by the SW AHG for the TE12 experiment.  However, it should be noted that these bitstreams were used in the validation of the SW AHG anchors.

# Encoding environment: Test conditions

Test results were generated using TMuC 0.7.3 on a 64bit linux cluster comprised of Intel X3450 with 8GiB RAM.  NB, different systems were used for the class A sequences to those used for anchor generation.

Binaries were produced in an identical manner to the anchor binaries.

# Test setup

The TMuC software was patched with the following to disable the planar mode:

diff --git a/source/Lib/TLibCommon/TypeDef.h b/source/Lib/TLibCommon/TypeDef.h
index 494f3b5..3006175 100644
--- a/source/Lib/TLibCommon/TypeDef.h
+++ b/source/Lib/TLibCommon/TypeDef.h
@@ -90,7 +90,7 @@
 #define ANG_INTRA                         1           // Enable angular ...
 #endif
 
-#define PLANAR_INTRA                      1           // Enable planar Intra coding
+#define PLANAR_INTRA                      0           // Enable planar Intra coding
 #define TENTM_DEBLOCKING_FILTER           1           // Enable TENTM deblocking
 #if HHI_INTERP_FILTER
 #define TEN_DIRECTIONAL_INTERP            1           ///< AF: interpolation filter

# Interpretation of runtime results

The machines used in the experiment all have four cores.  Due to the single threaded nature of the current TMuC software, up to four instances of the encoder were run per system.  The speed of the software is observed to vary according to the number of instances of the encoder that are running.  The total effect is to greatly speed up the execution of the experiment at the penalty of introducing noise to the runtime measurements.

As previously stated, different systems were used in generating the class-A anchors to the planar test result, as such, the runtimes are not directly comparable.

# Interpretation of BD-rate

Since this test is measuring the effect of turning off the planar mode with respect to the reference, a negative BD-rate indicates that there is a gain in turning the mode off.

# Results summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | -0.9 | -0.8 | -0.7 |
| Class B | 0.0 | 0.1 | 0.1 | -0.9 | -0.7 | -0.6 |
| Class C | 0.0 | 0.0 | 0.0 | -0.9 | -0.8 | -0.7 |
| Class D | -0.1 | 0.0 | 0.0 | -0.8 | -0.7 | -0.6 |
| Class E | 0.0 | 0.0 | 0.1 | -0.8 | -0.2 | -0.6 |
| All | 0.0 | 0.0 | 0.0 | -0.9 | -0.6 | -0.6 |
| Enc Time[%] | 102% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class A | 0.0 | 0.0 | 0.0 | -0.3 | -0.3 | -0.2 |  |
| Class B | 0.0 | 0.0 | -0.1 | -0.2 | -0.2 | -0.2 |  |
| Class C | 0.0 | -0.1 | 0.1 | -0.2 | -0.2 | -0.1 |  |
| Class D | 0.0 | 0.0 | -0.1 | -0.2 | -0.3 | -0.2 |  |
| Class E |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | -0.2 | -0.2 | -0.2 |  |
| Enc Time[%] | 93% |  |  | 93% |  |  |  |
| Dec Time[%] | 89% |  |  | 96% |  |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |  |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.1 |  |
| Class D | 0.0 | 0.2 | 0.2 | 0.0 | 0.1 | -0.1 |  |
| Class E | 0.0 | 0.3 | -0.2 | 0.0 | -0.7 | 0.2 |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 |  |
| Enc Time[%] | 102% |  |  | 104% |  |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  |  |

# Verification

These results are identical to those produced by Nokia in C188 within tolerance of ±1kbit/sec, ±0.005dB PSNR.

# Conclusion

In many cases, disabling planar results in no significant change in BD-rate; where there is a measurable change, it is almost always to the detriment of enabling planar.  The most significant change is observed in intra low complexity measurements, where there is a loss when the mode is enabled.  It should be noted however, that enabling the mode does lead to a reduction in runtime.

The root cause of the loss in compression performance has not been analyzed.  It is suspected that it is brought about by the combination of:

  - Planar mode effects an implicit skip=true flag,  it is suspected that the reductions in runtime that planar mode brings is through the use of the skip flag.

  - It is suspected that the error for planar blocks is not distributed uniformly across the block, rather, the error is greatest at the lower and right edges.  Such an error can potentially affect subsequent prediction units since no weighting is applied to the distortion calculation.