| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C245 WG11 Number: m18286 |
| --- | --- |

| Title: | TE8: Cross verification of TI-PCP proposal for significance map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinwen Zan Jin Meng Mohammad Towhidul Islam Dake He 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 jzan@rim.com dhe@rim.com |
| Source: | Research in Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results of the cross-checking activities performed by RIM on TI parallel context model proposal on significance map (described in JCTVC-B088 Section 3.4).

- Tested tools:

According to TE8 description, the following tool of JCTVC-B088 has been tested:

- significance map PCP (Section 3.4)

The TI modified version of TMuC-0.7 is used for the simulations. In the modified version of TMuC-0.7, two macros are defined for testing the tools defined in JCTVC-B088: TI_B088_RUN1_SIGN_BIN0 and TI_B088_RUN2_SIGMAP. In this test, the second macro is enabled.

By enabling the macro TI_B088_RUN2_SIGMAP, the encoder encodes one “Last” flag after encoding N “Significant” flags for the coefficients (NSIG1L), thus reducing the number of bins needs to be processed. The speed up factor, which is the ratio of the number of bins need to processed and the actual number of bins processed by employing the tested tool is reported after decoder decodes a video bitstream.

# Cross verification results

RIM test results matched that of TI’s under the “intra” and “lowdelay” modes. There are some minor mismatch results under the “random access” mode on the Class B sequences, to be more specific: 1) on the Kimono sequence, 0.2% on BD-rate U and 0.3% on BD-rate V have been observed; 2) on the BasketballDrive sequence, 0.1% on BD-rate V has been observed; and 3) on the RaceHorsesC sequence, 0.1% on BD-rate V has been observed. Further examination showed that these differences are quite minor, and could be a result of testing environment: RIM ran those sequences on Windows XP (32-bit) while TI ran them on Linux. Overall, the test data matched each other well.

|  | Intra - TI-PCP-Run1 |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 1.6 | 1.6 |
| Class B | 0.3 | 1.4 | 1.5 |
| Class C | 0.2 | 1.7 | 1.6 |
| Class D | 0.2 | 1.4 | 1.5 |
| Class E | 0.7 | 1.1 | 1.5 |
| All | 0.3 | 1.5 | 1.5 |
| Avg speedup factor | 1.1 |  |  |
| Max speedup factor | 1.2 |  |  |
|  |  | Random access - TI-PCP-Run1 |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 1.0 | 0.8 |
| Class B | 0.1 | 1.2 | 1.0 |
| Class C | 0.0 | 1.0 | 1.0 |
| Class D | 0.1 | 0.7 | 0.5 |
| Class E |  |  |  |
| All | 0.1 | 1.0 | 0.8 |
| Avg speedup factor | 1.1 |  |  |
| Max speedup factor | 1.3 |  |  |
|  | Low delay - TI-PCP-Run1 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.1 | 1.4 | 1.7 |
| Class C | -0.1 | 0.7 | 1.0 |
| Class D | -0.1 | 1.5 | 1.4 |
| Class E | 0.1 | 3.4 | 2.1 |
| All | -0.1 | 1.6 | 1.5 |
| Avg speedup factor | 1.0 |  |  |
| Max speedup factor | 1.2 |  |  |

# Tool analysis

In the tested tool NSIG1L, the larger the number N is, the more the speed up becomes. However, this speed up comes at the cost of the RD performance degradation. In the current TI setting, which employs 2 significant and 1 last (2SIG1L), a BD –rate performance degradation of 1.5% has been observed for the U and V components in “intra” mode; 1.0% on the U component in “random access” mode; and 1.6% and 1.5% for the U and V components in the “lowdelay” mode.

The speed up factor, on average, is 1.1 times for the “intra” and “random access” modes, and around the same as the anchor for the “lowdelay” mode.

It is expected that there could be further speed up if 3SIG1Lor 4SIG1L is used, however, the degradation of the RD performance needs to be tested.

# Patent rights declaration(s)

Research In Motion Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).