| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C249 WG11 Number: m18290 |
| --- | --- |

| Title: | TE8: Reports on V2V coding and context modeling by RIM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinwen Zan Gergely Korodi Jin Meng Mohammad Towhidul Islam Dake He 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 jzan@rim.com gkorodi@rim.com dhe@rim.com |
| Source: | Research in Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results on V2V codes and improved context processing as described in JCTVC-B308.

- Tested tools:

According to JCTVC-B308, the following tools are tested:

- V2V codes proposed by RIM

- Improved context processing in HEVC

Specifically, the following three tests are run.

- Bernoulli tests of  RIM V2V codes against PIPE V2V codes

- Improved context processing in TMuC0.7 against high efficiency anchors as defined in JCTVC-B312.

- RIM V2V codes in TMuC0.7 against high efficiency anchors as defined in JCTVC-B312.

A modified version of TMuC-0.7 is used for the testing. The PCP as proposed in JCTVC-B036 is implemented, and could be built by enabling a macro RIM_PARALLEL_CONTEXT. In this implementation, the DCT coefficient significance map contexts are grouped together by the same context used. Each group is processed in parallel, thus more than one bin could be processed in each cycle. In the current implementation, the maximum number of parallel bins is set to be 4.

It is noted that by enabling this macro, the tool QC_MDDT is automatically switched off. Thus the anchor data used in this test is TMuC-0.7 with QC_MDDT off (this data set is also generated/verified by TE12-MDDT, i.e., by SamSung, HKUST, and Qualcomm).

The throughput improvement as measured in the number of bins processed in parallel is reported at the end of decoding. To collect these statistics, the decoder has to carry out extra operations. In order to get a more accurate estimation on decoding speed, the macro RIM_TEST_THROUGHPUT has to be disabled and the decoder rebuilt.

Almost all of the tests are run under Windows-XP (32-bit), the only exception is the “Random Access” setting of the two Class A sequences, Traffic and PeopleOnStreet, which are tested under Linux.

- V2V coding of the TMuC TE12 anchors

The straightforward way of comparing the performance of the PIPE coder and the V2V coder is to encode the TMuC TE12 anchors using each one of them, and compare their performance. However, given the constrained computing resource available and the tight time limit, a transcoder has been designed for this purpose. The transcoder takes in a TMuC TE12 anchor bitstream (encoded with PIPE), decodes it using PIPE, outputs a reconstructed video sequence, and re-encodes it using the V2V coder. The transcoder could also take in a transcoded bitstream (V2V encoded), decode and output a reconstructed video sequence. The reconstruction video sequences from the PIPE and V2V bitstreams must be identical, meaning the RD performance comparison of the PIPE and V2V coders could be made based on the sizes of the compressed bitstream.

# Cross verification results

- V2V codes

This test is cross verified by TI. TI’s test results support the results of RIM. The compression performance was found to be exactly the same for both companies, and the reported throughput numbers are a close practical match.

The following table shows the performance loss of various algorithms compared to the entropy in percentages (column “Loss”), processing 1 billion bins per coding tree. The encoding and decoding throughput values show the million bin symbols processed per second.

| Test Results | Loss | Enc Throughput | Dec Throughput |
| --- | --- | --- | --- |
| BAC | 0.159 | 57.7 | 144.4 |
| PIPE | 0.147 | 68.7 | 110.4 |
| RIM-V2V | 0.122 | 222.0 | 251.1 |

- Improved context processing

This test is cross verified by TI. The TI cross check results show that for the “intra” mode, both results match exactly; for the “lowdelay” mode, there is one mismatch of 0.1% on BD-rate V of  Kimono; for the “random access” mode, there are two mismatches: 1) on Kimono, -0.1% on BD-rate Y, 0.1 on BD-rate U, and -0.3 on BD-rate V have been observed, and 2) on Cactus, -0.1% on BD-rate V has been observed. These minor differences could be explained by test environments used: RIM used Windows-XP (32-bit) for the testing, while TI used Linux (64-bit). Overall, the test results matched each other.

|  | Intra - RIM-PCP |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | 0.0 |
| Class B | 0.1 | 0.1 | 0.1 |
| Class C | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E | 0.0 | 0.0 | 0.0 |
| All | 0.1 | 0.1 | 0.1 |
| Avg SigCoeff Throughput | 1.9 |  |  |
| Avg LastCoeff Throughput | 2.1 |  |  |
|  |  | Random access - RIM-PCP |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |
| All | 0.1 | 0.0 | 0.1 |
| Avg SigCoeff Throughput | 1.7 |  |  |
| Avg LastCoeff Throughput | 1.9 |  |  |
|  | Low delay - RIM-PCP |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 |
| Avg SigCoeff Throughput | 1.4 |  |  |
| Avg LastCoeff Throughput | 1.5 |  |  |

- V2V codes in the TMuC 0.7 reference software

This test is cross verified by MediaTek and Qualcomm. For MediaTek, the cross check results show that for the “intra” and “lowdelay” modes, test results matched; for the “random access” mode, the TMuC anchor bitstreams used are slightly different. This could be caused by test setting: MediaTek runs test under Linux, while RIM runs tests under Windows-XP. It is noted that the difference is very minor, the largest difference is on Kimono, qp=27, and the difference is 0.12%. For Qualcomm, the same observation as above has been confirmed. In fact, because both cross checkers use Linux as their operating system, the test results from the tests matched exactly.

The comparison is summarized into the table below. The percentage is calculate as (anchor_size – V2V_file_size) * 100.0 / anchor_size.

Average File Size Difference Between PIPE and V2V

| Classes | INTRA | RANDOM ACCESS | LOWDELAY |
| --- | --- | --- | --- |
| Class A | 0.065% | 0.047% | N/A |
| Class B | 0.082% | -0.005% | -0.000% |
| Class C | 0.086% | -0.136% | -0.107% |
| Class D | 0.014% | -0.590% | -0.508% |
| Class E | 0.082% | N/A | -0.596% |
| ALL | 0.066% | -0.193% | -0.266% |

# Tool analysis

- V2V codes

The test results clearly demonstrate that RIM V2V codes provide significantly higher encoding and decoding throughput than either PIPE or BAC, all at competitive if not better compression efficiency. These results are further commented in a related contribution.

- Improved context processing

The performance of this test is measured from two aspects: 1) the RD performance change; and 2) the bin throughput improvement on coefficient significant map.

As can be seen from the test reports attached, the tool under test does not degrade the RD performance: the recorded BD rate measurement is less than 0.1% on average across all three test settings. For throughput, an average of 1.9 and 2.1 bins have been recorded for the “intra” mode, for the significant and last coefficient respectively, meaning that the throughput is doubled as compared to TMuC-0.7; the numbers for the “random access” and “lowdelay” modes are 1.7 and 1.9, and 1.4 and 1.5, respectively.

- V2V coding of the TMuC TE12 anchors

The sizes of the TMuC-0.7 TE12 anchors and those of the transcoded bitstreams are collected and compared. The file sizes from these two coders are very close, with V2V slightly better in the “intra” mode (0.07%), and PIPE slightly better in the “random access” mode (0.19%) and the “lowdelay” mode (0.27%).

# Patent rights declaration(s)

Research In Motion Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).