| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C250 WG11 Number: m18291 |
| --- | --- |

| Title: | Low complexity adaptive coefficient scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Rajan Joshi Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Adaptive coefficient scanning process is used to capture the coefficient statistics based on the intra prediction mode. In this proposal we introduce a partial sort at the coefficient level for determining the scan pattern. The proposed scheme reduces the complexity of the existing adaptive coefficient scan algorithm with average BD-rate penalty of 0.04%.

- Introduction

In the current TMuC software, quantized intra transform coefficients are scanned according to an adaptive coefficient scan pattern that is dependent on the prediction mode Error: Reference source not found. Separate scan tables are maintained for different block sizes and modes.  Scan tables are derived from scan statistics, which are initialized at the beginning of each slice and updated as the CU’s are coded. After encoding a CU, scan orders are updated according to the collected scan statistics by performing a full sort. Following the sorting operation the collected scan statistics are normalized.  Employing a full sort algorithm (insertion sort) that has a worst case performance of O(n2) introduces significant complexity to the decoder and the encoder.

In this contribution a low complexity adaptive coefficient scanning algorithm is introduced. The proposed algorithm replaces the full sort with a partial sort at the coefficient level to reduce the computation complexity significantly.

# Proposed method

In the proposed method, initialization of the scan statistics at the beginning of each slice is same as the existing method in [2]. During coefficient coding, the scan statistics are updated by incrementing the counter at a scan position whenever a non-zero coefficient is encountered. The incremented scan position’s counter is compared with the counter for the previous position in the scan order. If the current position’s counter is greater than the previous position’s counter, the scan order of the neighboring positions is swapped. A maximum count for the scan positions of a scan pattern is tracked. After encoding/decoding of a CU, scan statistics are normalized (scaled). The normalization decision is based on the maximum counts of the scan patterns.

# Simulations & Results

The proposed algorithm is implemented in TMuC v0.8. The simulation conditions are:

- Reference:

  - TMuC v0.8

- Proposed:

  - TMuC v0.8, fast adaptive coefficient scan

- QP = {22, 27, 32, 37}

- Test sequences:  Class A, B, C, D, E with full sequence length

- High efficiency Intra only

- Low complexity Intra only

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.00 | 0.03 | -0.01 | 0.00 | 0.01 | -0.01 |
| Class B | -0.01 | 0.03 | 0.07 | 0.05 | 0.08 | 0.08 |
| Class C | 0.01 | 0.05 | 0.04 | 0.07 | 0.06 | 0.08 |
| Class D | 0.05 | 0.08 | 0.05 | 0.15 | 0.11 | 0.14 |
| Class E | -0.01 | 0.00 | 0.06 | 0.05 | 0.01 | 0.03 |
| All | 0.01 | 0.04 | 0.05 | 0.07 | 0.07 | 0.08 |
| Enc Time[%] | 1.05 |  |  | 1.01 |  |  |
| Dec Time[%] | 0.51 |  |  | 0.37 |  |  |

# Conclusion

In this contribution, a low complexity adaptive coefficient scanning algorithm is proposed. The proposed algorithm reduces the computational complexity of the existing adaptive coefficient scan algorithm significantly (49-63% speedup) with ~0.04% drop in BD rate performance.

# Reference

- Y. Ye and M. Karczewicz, “Improved Intra Coding,” VCEG Contribution VCEG-AG11, Y. Ye and M. Karczewicz, Shenzhen, China, 20 October, 2007.

- “Test Model under Consideration”, JCT-VC, 2nd Meeting, Geneva, Switzerland, July 2010, Doc. JCTVC-B205.

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).