| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C253 WG11 Number: m18294 |
| --- | --- |

| Title: | Encoder speedup for bidirectional averaging with rounding control |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rahul Panchal, Marta Karczewicz, Peisong Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-658-4410 rpanchal@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Encoder speedup for bidirectional averaging with rounding control is presented here.

# Proposal

In the 2nd JCT-VC meeting, bidirectional averaging with rounding control (BiRound) was proposed and adopted in TMuC. In this contribution we are proposing further encoder optimizations leading to 50% complexity reduction of the BiRound with no changes in the encoder bitstream.

In TMuC 0.8, a new function with ‘BiRound’ parameter is always used for bidirectional motion estimation. To achieve further speedup, the new function is used only when ‘BiRound’ parameter is 1. Otherwise, the original function is used.

The results are obtained using TMuC 0.8 in low delay configuration with IBDI=0 and MRG=0. The reference was generated by undefining the macro ”ROUNDING_CONTROL_BIPRED”.

|  | No BiRound | BiRound |  |  | BiRound+ BitExact_Speedup |  |  | BiRound+ All Speedups |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class B | Kimono | -0.22 | -0.41 | -0.67 | -0.22 | -0.41 | -0.67 | -0.13 | -0.40 | -0.73 |
|  | Parkscene | -0.22 | -0.67 | -1.41 | -0.22 | -0.67 | -1.41 | -0.18 | -0.59 | -0.93 |
|  | Cactus | -1.41 | -1.85 | -2.18 | -1.41 | -1.85 | -2.18 | -1.38 | -1.63 | -2.04 |
|  | Basketball | -0.56 | -1.91 | -1.15 | -0.56 | -1.91 | -1.15 | -0.46 | -1.71 | -1.04 |
|  | BQSquare | -1.40 | -4.39 | -7.34 | -1.40 | -4.39 | -7.34 | -0.94 | -3.92 | -6.68 |
|  | Avg | -0.76 | -1.85 | -2.55 | -0.76 | -1.85 | -2.55 | -0.62 | -1.65 | -2.28 |
| Class C | Basketball | -0.43 | -0.80 | -0.50 | -0.43 | -0.80 | -0.50 | -0.13 | -0.76 | -0.38 |
|  | BQSquare | -0.73 | -0.47 | -1.41 | -0.73 | -0.47 | -1.41 | -0.57 | -0.31 | -1.22 |
|  | Partyscene | -0.56 | -0.95 | -0.79 | -0.56 | -0.95 | -0.79 | -0.43 | -0.85 | -0.72 |
|  | Racehorse | -0.14 | -0.37 | -0.75 | -0.14 | -0.37 | -0.75 | -0.12 | -0.52 | -0.62 |
|  | Avg | -0.47 | -0.65 | -0.86 | -0.47 | -0.65 | -0.86 | -0.31 | -0.61 | -0.74 |
| Class D | Basketball | -0.20 | -1.38 | -0.03 | -0.20 | -1.38 | -0.03 | -0.11 | -1.01 | -0.07 |
|  | BQSquare | -1.21 | -1.73 | -1.95 | -1.21 | -1.73 | -1.95 | -0.98 | -0.96 | -1.54 |
|  | Partyscene | -0.61 | -0.66 | -0.81 | -0.61 | -0.66 | -0.81 | -0.50 | -0.66 | -1.13 |
|  | Racehorse | -0.09 | -0.38 | -0.38 | -0.09 | -0.38 | -0.38 | -0.10 | -0.81 | -0.29 |
|  | Avg | -0.53 | -1.04 | -0.79 | -0.53 | -1.04 | -0.79 | -0.42 | -0.86 | -0.76 |
| Class E | Vidyo1 | -3.34 | -5.26 | -6.69 | -3.34 | -5.26 | -6.69 | -2.87 | -4.56 | -5.77 |
|  | Vidyo3 | -3.15 | -2.16 | -4.75 | -3.15 | -2.16 | -4.75 | -2.71 | -2.13 | -4.82 |
|  | Vidyo4 | -3.52 | -6.00 | -5.84 | -3.52 | -6.00 | -5.84 | -3.36 | -5.41 | -5.01 |
|  | Avg | -3.34 | -4.47 | -5.76 | -3.34 | -4.47 | -5.76 | -2.98 | -4.04 | -5.20 |
|  | All | -1.11 | -1.84 | -2.29 | -1.11 | -1.84 | -2.29 | -0.94 | -1.64 | -2.06 |
|  | Enc Time[%] | 108% |  |  | 104% |  |  | 98% |  |  |

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

- Y. Ye, P. Chen, and M. Karczewicz, “High Precision Interpolation and Prediction,” ITU-T SG16 contribution, VCEG-AI33, Berlin, Germany, July 2008.

- T. Chujoh, G. Yasuda, and T. Yamakage, “Bidirectional prediction for stored B-slice,” ITU-T SG16 contribution, VCEG-AI20, Berlin, Germany, July 2008.

# Patent rights declaration(s)

Qualcomm  may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).