| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C254 WG11 Number: m18295 |
| --- | --- |

| Title: | Test results of SIFO for sequences with illumination change |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rahul Panchal, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5673 martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the test results of Switched Interpolation Filters with Offset (SIFO) for sequences with illumination change.

- Role of Offset in SIFO

SIFO is not just an interpolation filter but it also has capability to compensate for illumination changes. Below we present results of SIFO for some sequences which has illumination changes to highlight the fact that for such sequences, the offset component of SIFO provides useful gains. The test was done using TMuC 0.8 in the low delay configuration.

| Reference=DCTIF 12 Tap | SIFO 12 Tap, w/o Offset | SIFO 12 Tap, w/ Offset | Offset Gain |
| --- | --- | --- | --- |
| WQVGA60_Crew | -0.18 | -1.14 | -0.96 |
| WQVGA_nuts3_416x240_30 | -0.16 | -1.55 | -1.39 |
| WVGA_nuts3_832x480_30 | -0.28 | -2.10 | -1.82 |
| WVGA_nuts5_832x480_30 | -0.07 | -0.58 | -0.51 |
| 720p60_600_Crew | -0.21 | -2.99 | -2.78 |
| 720p60_600_Shuttle_Start300 | -0.33 | -0.89 | -0.56 |
| Average | -0.21 | -1.54 | -1.34 |

- SIFO Encoder Speedup

SIFO Encoder Speedup has been integrated into TMuC 0.8.1. The table below shows BD-rate comparison when SIFO speedup is used. The SIFO encoder speedup shows BD-rate improvements while simultaneously achieving encoder speedup around  25%.

|  | SIFO Encoder Speedup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.03 | 0.04 | 0.02 | 0.02 | -0.01 | 0.00 |
| Class B | -0.13 | -0.08 | -0.31 | -0.09 | -0.08 | -0.24 |
| Class C | -0.12 | -0.22 | -0.18 | -0.18 | -0.19 | -0.16 |
| Class D | -0.37 | -0.31 | -0.30 | -0.36 | -0.48 | -0.55 |
| Class E |  |  |  |  |  |  |
| All | -0.18 | -0.16 | -0.23 | -0.17 | -0.21 | -0.27 |
| Enc Time[%] | 85% |  |  | 77% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.01 | -0.05 | 0.05 | 0.09 | 0.09 | 0.02 |
| Class C | -0.16 | -0.40 | -0.37 | -0.09 | -0.06 | -0.12 |
| Class D | -0.20 | -0.36 | -0.15 | -0.20 | 0.04 | -0.37 |
| Class E | -0.24 | -0.86 | -0.51 | 0.00 | -0.24 | -0.16 |
| All | -0.13 | -0.37 | -0.21 | -0.04 | -0.02 | -0.15 |
| Enc Time[%] | 92% |  |  | 87% |  |  |

- SIFO Decoder Complexity

Whenever SIFO 12 Tap is used, it actually uses 8 or 12 tap filters on the border subpels (a,b,c,d,h,l) which requires 1D interpolation and SIFO always uses 8 tap filters on inner subpels (e,f,g,I,j,k,m,n,o) which requires 2D interpolation. Thus, the decoder complexity of SIFO 12 Tap is less than using 12 tap filters for all subpel positions.

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

Qualcomm  may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).