| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C255 WG11 Number: m18296 |
| --- | --- |

| Title: | DCT+Hadamard low complexity large transform for Inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Ajit Gupte Texas Instruments Inc., India | Tel: Email: Tel: Email: | madhukar@ti.com a-gupte1@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a class of transforms for large block sizes which is a combination of DCT+Hadamard transforms for reducing computational complexity of Inter transforms. The best performing 32x32 transform in this group provides most of the coding gains of 32x32 DCT for high-efficiency configurations but with 50% reduction in number of multiplications assuming direct matrix multiplication DCT. If Chen’s DCT implementation is assumed, then the number of multiplications reduce by 24%.

# Introduction

TMuC uses several sizes of transform up to 64x64. The complexity of transforms typically increases with O(Nlog(N)) for fast algorithms and O(N^3) for implementation with direct matrix multiplication. E.g. a 16x16 transform implemented using Chen’s algorithm requires 1408 multiplication where as 32x32 transform requires 7424 multiplications which is almost a 5X increase in complexity. Direct matrix multiplication requires 16^3 multiplications for 16x16 and 32^3 multiplications for 32x32 transform which is 8X increase in complexity. Hence it is desirable to reduce complexity of large size transforms.

The following transforms were implemented in TMuC-0.7.3:

- Hadamard32

- DCT16+Hadamard2

- Hadamard2+DCT16

- DCT32

A summary of results is provided in table below for 65 frames of common conditions sequences. The comparison in table below is with respect to the case of using all DCT transforms up to 32x32. No32 stands for no DCT32, HAD stands for Hadamard32, T216 stands for Hadamard2+DCT16 and T162 stands for DCT16+Hadamard2. RA stands for Random access and LD stands for low-delay. It can be seen from columns RA-T162 and LD-T162 that the average bitrate increase when compared to using DCT32 is 0.19% and 0.37% respectively. The maximum bitrate increase is 1.59% and 1.64% respectively and it happens for Kimino sequence. Simulations results on all frames and for low complexity configurations will be provided after the simulations complete.

|  |  |  | RA-No32 | RA-HAD | RA-T216 | RA-T162 |  | LD-No32 | LD-HAD | LD-T216 | LD-T162 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S01 | Traffic |  | 1.41 | 0.14 | 0.16 | 0.09 |  |  |  |  |  |
| S02 | PeopleOnStreet |  | 0.29 | 0.06 | 0.00 | 0.03 |  |  |  |  |  |
| S03 | Kimono |  | 5.81 | 3.54 | 2.80 | 1.59 |  | 4.56 | 4.91 | 3.53 | 1.64 |
| S04 | ParkScene |  | 0.77 | 0.06 | 0.03 | 0.08 |  | 0.35 | 0.40 | 0.24 | 0.16 |
| S05 | Cactus |  | 2.34 | 1.35 | 0.97 | 0.45 |  | 2.48 | 1.92 | 1.20 | 0.54 |
| S06 | BasketballDrive |  | 4.36 | 1.30 | 0.83 | 0.25 |  | 5.20 | 1.74 | 1.18 | 0.70 |
| S07 | BQTerrace |  | 1.48 | 0.24 | 0.20 | 0.00 |  | 1.03 | 0.30 | 0.24 | 0.14 |
| S08 | BasketballDrill |  | 2.37 | 0.69 | 0.58 | -0.10 |  | 3.06 | 1.47 | 1.04 | 0.30 |
| S09 | BQMall |  | 1.32 | 0.30 | 0.23 | 0.03 |  | 0.92 | 0.32 | 0.44 | 0.29 |
| S10 | PartyScene |  | 0.33 | 0.16 | 0.16 | 0.02 |  | 0.40 | 0.30 | 0.26 | 0.06 |
| S11 | RaceHorses |  | 0.29 | -0.05 | -0.03 | -0.05 |  | 0.31 | 0.43 | 0.21 | 0.16 |
| S12 | BasketballPass |  | 1.14 | 0.04 | -0.05 | 0.01 |  | 1.01 | 0.24 | 0.08 | 0.06 |
| S13 | BQSquare |  | 0.78 | 0.07 | 0.16 | 0.20 |  | 0.32 | 0.23 | 0.12 | 0.05 |
| S14 | BlowingBubbles |  | 0.57 | 0.41 | 0.19 | 0.29 |  | 0.48 | 0.67 | 0.36 | 0.69 |
| S15 | RaceHorses |  | 0.06 | -0.03 | -0.12 | -0.01 |  | -0.04 | 0.12 | -0.04 | 0.04 |
| S16 | Vidyo1 |  |  |  |  |  |  | 5.93 | 0.70 | 0.57 | 0.14 |
| S17 | Vidyo3 |  |  |  |  |  |  | 3.78 | 0.45 | 0.41 | 0.43 |
| S18 | Vidyo4 |  |  |  |  |  |  | 5.25 | 1.09 | 0.64 | 0.52 |
|  | Avg |  | 1.55 | 0.55 | 0.41 | 0.19 |  | 2.19 | 0.95 | 0.66 | 0.37 |
|  | Min |  | 0.06 | -0.05 | -0.12 | -0.10 |  | -0.04 | 0.12 | -0.04 | 0.04 |
|  | Max |  | 5.81 | 3.54 | 2.80 | 1.59 |  | 5.93 | 4.91 | 3.53 | 1.64 |
|  | Stats over sequences for which 32x32 transform matters |  |  |  |  |  |  |  |  |  |  |
|  | Avg |  | 2.53 | 0.95 | 0.72 | 0.29 |  | 3.59 | 1.42 | 0.99 | 0.50 |
|  | Min |  | 1.14 | 0.04 | -0.05 | -0.10 |  | 1.01 | 0.24 | 0.08 | 0.06 |
|  | Max |  | 5.81 | 3.54 | 2.80 | 1.59 |  | 5.93 | 4.91 | 3.53 | 1.64 |

# Conclusions

The class of DCT+Hadamard transforms provide an attractive trade-off in terms of complexity reduction and rate-distorition performance. Other complexity reduction techniques such as fast DCT algorithms, zonal coding etc. can be built on top it to further reduce complexity. Hence, we would like to recommend starting a tool/core experiment to further study DCT+Hadamard transforms.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).