| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C257 WG11 Number: m18298 |
| --- | --- |

| Title: | On motion vector competition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yeping Su and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

We propose a method to reduce the complexity of motion vector competition.  Our focus is reducing the size of the motion vector buffer needed for storing temporal collocated vectors.  The size of the motion vector buffer is significant, considering the granularity of motion representation and thjat there are two vectors per motion block for B_SLICE.  To reduce the size of that buffer and the associated memory access bandwidth, we propose to use a simple block-based median filter to store motion vectors on a larger granularity.

# Introduction

In the TMuC software [1], motion vector competition is used to reduce the amount of motion information in the compressed bitstream. This tool improves the coding of motion vector data by enabling a variety of motion vector predictors, and it allows an encoder to select an appropriate motion vector predictor within an R-D framework.  One very useful property of this tool is the use of a temporally collocated motion vector.  This is included in the motion competition process when mv_competition_temporal_flag is set to 1.

In this document, we propose a method to reduce the size of the motion vector buffer for storing temporal collocated vectors.  The size of the motion vector buffer is significant, considering the granularity of motion representation and that there are two vectors per motion block for B_SLICE.  To reduce the size of the buffer and the associated memory access bandwidth, we propose to use a simple block-based median filter to store motion vectors on a larger granularity than the decoding granularity.  The motion vector buffer compression process replaces the decoded motion vector buffer with a lower resolution buffer, with each compressed vector calculated as the component-wise median filter of a block of MVBufferCompRatio-by-MVBufferCompRatio decoded vectors.  We propose to signal the ratio parameter MVBufferCompRatio in the sequence parameter set, though we anticipate the value to ultimately be defined by a profile/level.  The motion vector buffer compression process is performed after the adaptive loop filter process, and before the decoded picture is put into DPB.

# Syntax and Semantics

### Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| ..... |  |  |
| motion_vector_buffer_comp_flag | 0 | u(1) |
| if ( motion_vector_buffer_comp_flag ) |  |  |
| motion_vector_buffer_comp_ratio_log2 | 0 | u(8) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

Semantics

motion_vector_buffer_comp_flag equals to 1 specifies the motion vector buffer compression process is applied.

motion_vector_buffer_comp_ratio specifies the compression ratio in the motion vector buffer compression process. When motion_vector_buffer_comp_ratio is not present, it shall be inferred to be equal to 0. The motion vector buffer compression ratio is specified as follows:

MVBufferCompRatio = 1<< motion_vector_buffer_comp_ratio_log2

# Results

Results are included in the XLS accompanying the proposal for the low delay configuration.

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0 | 0.1 | 0.0 |
| Class C | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.3 |
| Class D | 0.2 | 0.0 | 0.1 | 0.1 | 0.2 | 0.0 |
| Class E | 0.1 | 0.2 | 0.0 | 0.1 | 0.3 | 0.2 |
| All | #VALUE! | #VALUE! | #VALUE! | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 1: Performance of buffer compression with a 4x reduction in bandwidth

# Conclusion

We propose a method to compress the collocated motion vector predictors to reduce the memory requirements (and bandwidth) of storing the collocated motion information in the motion vector competition scheme.  We propose that the technology be further evaluated in a core experiment.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCT-VC, “Test Model under Consideration”, JCTVC-B205, 1. JCT-VC Meeting, Geneva, July 2010.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-Curves,” ITU-T Q.6/SG16 Doc., VCEG-M33, Austin, Apr. 2001.