| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C261 WG11 Number: m18302 |
| --- | --- |

| Title: | TE4: Summary of TE4 on variable length coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | Xianglin Wang 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-5135 xianglin@qualcomm.com |
| Source: | TE coordinator (Qualcomm Inc.) |  |  |

_____________________________

- Introduction

This document summarizes the activities in the Tool Experiment TE4 on Variable Length Coding (VLC). A group of five companies and universities has registered for participation in TE4. One TE report, JCTVC-C262, is submitted on VLC coding for coded block flag.

# Document List

X. Wang, M. Karczewicz, W.-J.Chien, “TE4: Report on VLC on Coded Block Flag”, JCTVC-C262

# Summary of proposal

Document JCTVC-C262 reports TE results based on a coding schemes presented in a previous proposal JCTVC-B098 [1].  In JCTVC-B098, coding schemes for coded block flag both at CU level and at block level are proposed. Due to the fact that the proposed coded block flag coding at CU level is very close to the one introduced in phase-2 VLC integration, JCTVC-C262 will further report results of coded block flag coding only at block level.

According to the proposal, if a coded block flag for a certain video component at coding unit level is not zero and transform block size is smaller than the coding unit size, the coded block flags from the four quarter-sized blocks are grouped and coded together. Due to coded block flag at coding unit level, the coded block flags for the four quarter-sized blocks can not all be zero. As a result there are 15 different combinations in total for these flags. A new VLC table is designed and used accordingly. It is reported that on average around 0.2% coding gain can be obtained using the three low complexity coding configurations.

# Conclusions

- Results are to be reviewed and discussed in this meeting.

# References

[1] M. Karczewicz, P. Chen, W.J.Chien, X.Wang, “Variable length coding for coded block flag and large transform”, JCTVC-B098, Geneva, Switzerland, July 2010.