| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C262 WG11 Number: m18303 |
| --- | --- |

| Title: | TE4: Report on VLC for coded block flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang, Marta Karczewicz, Wei-Jung Chien 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-5135 xianglin@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of a VLC coding scheme for coded block flag. According to the scheme, four coded block flags of each video component at block level are grouped and coded together. Simulation results show that on average around 0.2% percent coding gain can be obtained with all the three loco configurations.

- Introduction

According to the current TMuC variable length coding (VLC), coded block flags are coded both at coding unit level and at block level. At coding unit level, a coded block flag of a video component indicates if the component of the coding unit contains any non-zero coefficient. If a coded block flag for a certain component at coding unit level is not zero and transform block size is smaller than the coding unit size, coded block flags at block level are further coded, using 1-bit each, to indicate if each of the transform blocks contains any non-zero coefficient.

- Proposed solution

At the Geneva meeting in July, a VLC coding scheme for coded block flag was proposed [1]. Initially, coding schemes for coded block flag both at CU level and at block level are proposed. Due to the fact that the proposed coded block flag coding at CU level is very close to the one introduced in phase-2 VLC integration, this document will further report results of coded block flag coding only at block level.

According to the proposed scheme, the coded block flags for the four quarter-sized blocks are grouped and coded together. Due to coded block flag at coding unit level, the coded block flags for the four quarter-sized blocks can not all be zero. As a result there are 15 different combinations in total for these flags. A new VLC table is introduced with 15 codewords in total. One codeword has 3 bits and the remaining 14 codewords all have 4 bit each. If the block to be coded is inter block, only the new VLC table is used in coding its coded block flags. If it’s an intra block, two of the existing VLC tables, together with the new VLC table, are used. VLC table switching and adaptation is the same as that used for coding other syntax elements.

It is worth noting that the proposed scheme is designed for the case when residual quadtree is not used.

# Simulation results

The coding scheme described in Section 2 is simulated in TMuC 0.7.4. The same LOCO configurations used for TE12 are used in the simulation, with the following two modifications:

- Enable phase-2 VLC (define LCEC_PHASE2 as 1).

- Disable residual quadtree (define RQT as 0).

Simulation software is provided with the proposal. In the software, a macro QC_BLK_CBP is used to enable the proposed coding scheme.

Results using different configurations are shown in Table 1, 2 and 3 respectively.

Table 1: Coding results with the all intra configuration

| All intra | New CBF coding |
| --- | --- |
| low complexity |  |
| WQVGA30_RaceHorses | -0.3 |
| WQVGA50_BasketballPass | -0.2 |
| WQVGA50_BlowingBubbles | -0.4 |
| WQVGA60_BQSquare | -0.3 |
| WQVGA AVE | -0.3 |
| WVGA30_RaceHorses | -0.2 |
| WVGA50_BasketballDrill | -0.1 |
| WVGA50_PartyScene | -0.4 |
| WVGA60_BQMall | -0.2 |
| WVGA AVE | -0.2 |
| 720p60_Vidyo1 | -0.1 |
| 720p60_Vidyo3 | -0.1 |
| 720p60_Vidyo4 | -0.2 |
| 720p AVE | -0.2 |
| 1080p24_ParkScene | -0.2 |
| 1080p24_Kimono | -0.2 |
| 1080p50_BasketballDrive | -0.2 |
| 1080p50_Cactus | -0.2 |
| 1080p60_BQTerrace | -0.3 |
| 1080p AVE | -0.2 |
| PeopleOnStreet | -0.1 |
| Traffic | -0.2 |
| 4kx2k AVE | -0.2 |
| Average | -0.2 |

Table 2: Coding results with the low delay configuration

| Low delay | New CBF coding |
| --- | --- |
| low complexity |  |
| WQVGA30_RaceHorses | -0.1 |
| WQVGA50_BasketballPass | -0.2 |
| WQVGA50_BlowingBubbles | -0.2 |
| WQVGA60_BQSquare | -0.1 |
| WQVGA AVE | -0.2 |
| WVGA30_RaceHorses | -0.2 |
| WVGA50_BasketballDrill | -0.2 |
| WVGA50_PartyScene | -0.3 |
| WVGA60_BQMall | -0.1 |
| WVGA AVE | -0.2 |
| 720p60_Vidyo1 | 0.0 |
| 720p60_Vidyo3 | -0.2 |
| 720p60_Vidyo4 | -0.1 |
| 720p AVE | -0.1 |
| 1080p24_ParkScene | -0.2 |
| 1080p24_Kimono | -0.2 |
| 1080p50_BasketballDrive | -0.2 |
| 1080p50_Cactus | -0.2 |
| 1080p60_BQTerrace | -0.2 |
| 1080p AVE | -0.2 |
| Average | -0.2 |

Table 3: Coding results with the random access configuration

| Random access | New CBF coding |
| --- | --- |
| low complexity |  |
| WQVGA30_RaceHorses | -0.1 |
| WQVGA50_BasketballPass | -0.2 |
| WQVGA50_BlowingBubbles | -0.2 |
| WQVGA60_BQSquare | -0.2 |
| WQVGA AVE | -0.2 |
| WVGA30_RaceHorses | -0.2 |
| WVGA50_BasketballDrill | 0.0 |
| WVGA50_PartyScene | -0.2 |
| WVGA60_BQMall | -0.1 |
| WVGA AVE | -0.2 |
| 1080p24_ParkScene | -0.2 |
| 1080p24_Kimono | -0.1 |
| 1080p50_BasketballDrive | -0.1 |
| 1080p50_Cactus | -0.2 |
| 1080p60_BQTerrace | -0.2 |
| 1080p AVE | -0.2 |
| PeopleOnStreet | -0.1 |
| Traffic | -0.2 |
| 4kx2k AVE | -0.1 |
| Average | -0.2 |

# Conclusions

- This contribution report coding results of a coding scheme for block level coded block flag. With simple grouping of four coded block flags from for the four quarter-sized transform blocks, an average coding gain of about 0.2% can be achieved.

We recommend such a coding scheme on coded block flag coding to be adopted into TMuC.

# References

[1] M. Karczewicz, P. Chen, W.J.Chien, X.Wang, “Variable length coding for coded block flag and large transform”, JCTVC-B098, Geneva, Switzerland, July 2010.

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).