| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C263 WG11 Number: m18304 |
| --- | --- |

| Title: | Improvements on VLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Wei-Jung Chien, Xianglin Wang 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-658-5673 martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents improvements on variable length coding (VLC). In this contribution, a modification is proposed to the current transform coefficient coding for intra blocks. With the introduction of new VLC mapping tables and the changes to table selection in coding {levelID, run} pair, a coding gain of 2.3% is achieved in simulation.

New coding schemes are also proposed for coding the prediction mode of both intra and inter block respectively. With more suitable VLC table selection and appropriate symbol combination, coding performance on prediction mode is improved.

- Introduction

According to the current TMuC, transform coefficient coding is done in run mode followed by level mode. In run mode coding, the number of zero coefficients is signaled starting from the previous nonzero coefficient. The syntax element isLevelOne_run combines information about the run and the levelID. Run is defined as the number of zero coefficients between the current nonzero coefficient at position k and the next nonzero coefficient in the reverse scanning order. The levelID indicates whether the absolute value of that coefficient level is 1 (levelID =0) or great than 1(levelID=1). The mapping between {levelID, run} pair and code number cn is also dependent on value k. There are 29 VLC tables specified for different values of k, with up to 128 entries in the table. Currently the same mapping tables are used both for intra and inter coded blocks.

In coding of intra prediction modes, a most probably mode is first derived. A 1-bit flag is sent to indicate if the chosen intra prediction mode is the same as the most probable mode. If it is different, depending on the number of prediction modes available, a fixed length VLC codeword is sent to signal the chosen intra prediction mode.

Planar mode is a special intra prediction mode. Currently it is indicated with a 1-bit flag for each coding unit (CU).

Inter prediction modes currently include at least split flag, skip flag, direct flag, Inter_2Nx2N, Inter_Nx2N, Inter_2NxN, Inter_NxN, Inter_2NxnU, Inter_2NxnD, Inter_nLx2N, Inter_nRx2N, Intra_2Nx2N, and Intra_NxN. The first three flags are coded in the following order. If split flag is 1, the mode coding is ended; otherwise, an adaptive loop filter flag and a skip flag are sent. If the skip flag is 1, the mode coding is ended; otherwise, a direct flag is sent. If the direct flag is 1, the mode coding is ended; otherwise, a VLC code is sent to indicate which mode is chosen among all other modes. Such a coding scheme is applied to CU prediction mode regardless of CU depth.

- Proposed solution

## Transform coefficient coding

According to the proposal, the current tables in TMuC that map the {levelID, run} pair to code number are only used for inter blocks. New tables are introduced for blocks coded in intra mode. In addition, for an intra block the selection of the table is dependent both on the position k of the current nonzero coefficient and a parameter n. Parameter n is defined as zero if the absolute value of any of the coefficient levels coded so far (in the inverse scan order) is larger than 1. Otherwise, it is defined as the number of non-zero coded coefficients, clipped to 4.

To minimize memory usage we modified the tables obtained from training to have more structured format, which allows a simple mapping to a code number cn given values of leveleID, run, k and n as follows:

if (levelID==0){

if (run<c0)

cn=run;

else

cn=2*run-c0+1;

}

else{

if (run>(k-c0 +1))

cn=k+run+2;

else

cn=c0 +2*run;

}

Where c0 represents the code number when run=0 and levelID=1. Only values of c0, which are dependent both on parameter k and n, have to be stored.

## Intra prediction mode coding

Coding of intra prediction modes is modified for 4x4 blocks (17 modes), 16x16 and 32x32 blocks (34 modes). For example for 4x4 blocks two tables are used in mapping intra prediction mode to code number, with each mapping associated with a corresponding VLC table. Most probable intra mode for a current block is determined from intra prediction modes of its left and above blocks. Based on the most probable intra prediction mode one of the two (mapping table, VLC table) pairs is selected. If the chosen prediction mode is equal to the most probable mode, code number 0 is used. Otherwise code numbers m and n are found, respectively, for the most probable intra mode and the chosen intra mode using selected mapping table. If n<m code number n is modified: n=n+1. In case of 16x16 and 32x32 blocks 4 tables are used.

For planar mode, instead of being signaled with a 1-bit flag for each CU, it is signaled as a regular intra prediction mode.

## Inter prediction mode coding

According to the proposal, split flag, skip flag, and direct flag are grouped and coded together with a symbol Inter_Partition. Unary codes are used. When Inter_Partition in coded, it indicates that additional information about prediction modes will follow. Mapping between symbol and codeword index is adaptive. The adaptation method is the same as that for other VLC coded syntax elements. Such adaptation is applied separately at each CU depth. When Inter_Partition is coded, additional code is sent to signal one of other prediction modes for the current CU. For example, its mode may be Inter_2Nx2N or Inter_Nx2N, etc. A fixed VLC table for each CU depth is used for this purpose.

Currently in TMuC, a termination bit is sent at LCU (largest CU) level to indicate whether the current CU is the last one in the slice. This bit is redundant since the termination can be determined using stuffing bits at the end of slice as in H.264.

# Simulation results

The coding scheme described in Section 2 is simulated in TMuC 0.8 with phase-2 VLC changes enabled (LCEC_PHASE2 defined as 1). TE12 loco configurations are used.

Table 1 shows the results of the proposed coding scheme using the all intra loco configuration. Improvement brought by each proposed modification is given.

The following Table 2 and Table 3 show coding results respectively for low delay loco configuration and random access loco configuration.

Table 1: Simulation results of the proposed scheme for intra block

| Low complexity all intra | Scan | Scan, Coeff Mod | Scan, Coeff Mod, Intra Mode | Scan, Coeff Mod, Intra Mode, Planar |
| --- | --- | --- | --- | --- |
| WQVGA30_RaceHorses | 0.03 | -2.43 | -2.81 | -3.73 |
| WQVGA50_BasketballPass | -1.37 | -3.61 | -4.19 | -5.20 |
| WQVGA50_BlowingBubbles | -0.09 | -2.50 | -2.97 | -3.78 |
| WQVGA60_BQSquare | -0.15 | -2.89 | -3.32 | -3.98 |
| WQVGA AVE | -0.39 | -2.86 | -3.32 | -4.17 |
| WVGA30_RaceHorses | -0.07 | -2.39 | -2.72 | -3.49 |
| WVGA50_BasketballDrill | -0.47 | -2.09 | -2.16 | -3.16 |
| WVGA50_PartyScene | -0.34 | -2.75 | -3.39 | -4.18 |
| WVGA60_BQMall | -1.29 | -3.63 | -4.22 | -5.20 |
| WVGA AVE | -0.54 | -2.72 | -3.12 | -4.01 |
| 720p60_Vidyo1 | -1.21 | -3.48 | -3.98 | -4.92 |
| 720p60_Vidyo3 | -1.36 | -3.47 | -3.94 | -4.72 |
| 720p60_Vidyo4 | -0.66 | -3.06 | -3.69 | -4.53 |
| 720p AVE | -1.08 | -3.34 | -3.87 | -4.72 |
| 1080p24_ParkScene | -0.38 | -3.25 | -3.83 | -4.61 |
| 1080p24_Kimono | 0.60 | -1.74 | -2.06 | -2.66 |
| 1080p50_BasketballDrive | -3.39 | -5.32 | -5.64 | -6.49 |
| 1080p50_Cactus | -0.07 | -2.25 | -2.70 | -3.57 |
| 1080p60_BQTerrace | -1.98 | -4.27 | -4.55 | -5.15 |
| 1080p AVE | -1.04 | -3.37 | -3.75 | -4.50 |
| PeopleOnStreet | 0.14 | -2.43 | -3.03 | -3.94 |
| Traffic | -0.11 | -2.75 | -3.24 | -4.06 |
| 4kx2k AVE | 0.02 | -2.59 | -3.14 | -4.00 |
| Average | -0.68 | -3.02 | -3.47 | -4.30 |

Table 2: Results of the proposed coding scheme for inter block prediction mode with low delay configuration

| Low complexity low delay | Pred. mode | Pred. mode, termination bit | Intra mod., pred. mode, termination bit |
| --- | --- | --- | --- |
| WQVGA30_RaceHorses | -0.93 | -1.07 | -1.12 |
| WQVGA50_BasketballPass | -0.95 | -1.12 | -1.19 |
| WQVGA50_BlowingBubbles | -0.25 | -0.44 | -0.58 |
| WQVGA60_BQSquare | -0.89 | -1.07 | -1.05 |
| WQVGA AVE | -0.75 | -0.92 | -0.98 |
| WVGA30_RaceHorses | -0.96 | -1.13 | -1.26 |
| WVGA50_BasketballDrill | -0.50 | -0.82 | -0.27 |
| WVGA50_PartyScene | -0.52 | -0.68 | -1.01 |
| WVGA60_BQMall | -0.37 | -0.77 | -0.59 |
| WVGA AVE | -0.59 | -0.85 | -0.78 |
| 720p60_Vidyo1 | -2.92 | -5.32 | -3.57 |
| 720p60_Vidyo3 | -2.00 | -3.97 | -2.36 |
| 720p60_Vidyo4 | -2.89 | -5.31 | -3.53 |
| 720p AVE | -2.60 | -4.87 | -3.15 |
| 1080p24_ParkScene | -2.49 | -3.03 | -2.66 |
| 1080p24_Kimono | -3.68 | -4.35 | -3.82 |
| 1080p50_BasketballDrive | 1.01 | 0.54 | 0.42 |
| 1080p50_Cactus | -3.35 | -3.90 | -3.91 |
| 1080p60_BQTerrace | -0.47 | -1.13 | -0.61 |
| 1080p AVE | -1.80 | -2.38 | -2.12 |
| Average | -1.39 | -2.10 | -1.69 |

# Conclusions

- Improvements on VLC coding are presented in this contribution. Changes are proposed to both coefficient coding for intra block, as well as prediction mode coding for intra and inter block.

- With a change to VLC mapping table and table selection scheme, an over 2% average coding gain is observed with the all intra loco configuration. In addition, with proposed intra prediction mode coding, an overall of over 4% average coding gain is achieved. For inter prediction mode coding, with appropriate symbol combination and VLC table selection, coding performance is also clearly improved.

- We recommend the proposed coding schemes mentioned above to be considered for adoption into TMuC.

Table 3: Results of the proposed coding scheme for inter block prediction mode with random access configuration

| Low complexity random access | Pred. mode | Pred. mode, termination bit | Intra mod., pred. mode, termination bit |
| --- | --- | --- | --- |
| WQVGA30_RaceHorses | -1.45 | -1.63 | -2.10 |
| WQVGA50_BasketballPass | -1.33 | -1.56 | -2.09 |
| WQVGA50_BlowingBubbles | -0.85 | -1.10 | -2.62 |
| WQVGA60_BQSquare | -0.71 | -1.02 | -2.38 |
| WQVGA AVE | -1.09 | -1.33 | -2.29 |
| WVGA30_RaceHorses | -1.18 | -1.40 | -2.13 |
| WVGA50_BasketballDrill | -1.10 | -1.50 | -2.24 |
| WVGA50_PartyScene | -0.64 | -0.87 | -2.79 |
| WVGA60_BQMall | -1.19 | -1.68 | -2.39 |
| WVGA AVE | -1.03 | -1.36 | -2.39 |
| 1080p24_ParkScene | -0.97 | -1.55 | -3.15 |
| 1080p24_Kimono | -1.46 | -2.31 | -2.92 |
| 1080p50_BasketballDrive | -1.76 | -2.40 | -3.57 |
| 1080p50_Cactus | -1.49 | -2.21 | -2.95 |
| 1080p60_BQTerrace | -1.27 | -2.15 | -3.65 |
| 1080p AVE | -1.39 | -2.12 | -3.25 |
| PeopleOnStreet | -1.16 | -1.42 | -2.05 |
| Traffic | -0.97 | -1.83 | -3.20 |
| 4kx2k AVE | -1.07 | -1.62 | -2.62 |
| Average | -1.17 | -1.64 | -2.68 |

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).