| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C264 WG11 Number: m18305 |
| --- | --- |

| Title: | TE 12: Evaluation of Block Merging (MRG) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Peisong Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the test results for the block merging tool listed in Tool Experiment 12: Evaluation of TMuC Tools.

- Introduction

In the 2nd JCT-VC meeting, Tool Experiment 12 (TE12) was set up to explore the performance of the individual tools in the Test Model under Consideration (TMuC). This contribution is to test the block merging tool of TE 12.

The software version used for this test was TMuC 0.7.1. The reference was generated by using the default setting [2]. In the default setting, the block merging was enabled. The reference was compared against the alternative setting with the block merging disabled (#define HHI_MRG 0) and early skip disabled (#define EARLY_SKIP_THRES 0). The executables for encoding and decoding were compiled on 64-bit Linux and the simulations were performed on the Linux cluster. It should be noted that the simulations were performed on a cluster, where the computational power of each node was different. Therefore the encoding/decoding time results are not very reliable.

# Simulation result

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -1.2 | -1.4 | -2.4 | -3.1 | -2.9 |
| Class B | -0.7 | -0.9 | -1.0 | -3.5 | -3.4 | -3.4 |
| Class C | -0.8 | -1.2 | -1.2 | -1.8 | -2.5 | -2.7 |
| Class D | -0.7 | -0.9 | -1.0 | -2.5 | -2.6 | -3.0 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -1.0 | -1.1 | -2.6 | -2.9 | -3.0 |
| Enc Time[%] | 107% |  |  | 109% |  |  |
| Dec Time[%] | 116% |  |  | 107% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.3 | -0.6 | -2.3 | -1.9 | -1.9 |
| Class C | -0.5 | -1.0 | -0.8 | -1.1 | -1.5 | -1.7 |
| Class D | 0.2 | -0.4 | -0.4 | -0.5 | -0.6 | -1.0 |
| Class E | 0.7 | 1.2 | 0.6 | -9.8 | -8.9 | -10.2 |
| All | 0.1 | -0.2 | -0.4 | -2.9 | -2.8 | -3.2 |
| Enc Time[%] | 104% |  |  | 101% |  |  |
| Dec Time[%] | 119% |  |  | 109% |  |  |

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).