| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C266 WG11 Number: m18307 |
| --- | --- |

| Title: | TE1: Cross-check result of DMVD proposal from JCTVC-C138 (JVC/Kenwood) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Peisong Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the crosscheck results for Decoder-Side Motion Vector Derivation (DMVD) proposal from JVC/Kenwood (JCTVC-C138) listed in Tool Experiment 1: Decoder-Side Motion Vector Derivation.

- Introduction

In the 2nd JCT-VC meeting, Tool Experiment 1 (TE1) was set up to explore the performance of the individual DMVD proposal. This contribution is to crosscheck the DMVD proposal from JVC/Kenwood.

The software version used for this test was TMuC 0.7. The reference was generated by using the default setting [1]. The reference was compared against the alternative setting with the DMVD tool enabled (#define JVC_DMVD). The executables for encoding and decoding were compiled on 64-bit Linux and the simulations were performed on the Linux cluster. It should be noted that the simulations were performed on a cluster, where the computational power of each node was different. Therefore the encoding/decoding time results are not very reliable.

# Simulation result

|  |  | Random access |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.6 | -3.1 | -2.9 |
| Class B | -1.6 | -1.4 | -1.6 |
| Class C | -1.9 | -2.0 | -2.2 |
| Class D | -1.6 | -1.6 | -1.7 |
| Class E |  |  |  |
| All | -1.8 | -1.8 | -1.9 |
| Enc Time[%] | 135% |  |  |
| Dec Time[%] | 144% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E | 0.1 | -0.1 | 0.1 |
| All | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 105% |  |  |

- JCT-VC, “Decoder-Side Motion Vector Derivation”, JCTVC-B301, JCT-VC Meeting, Geneva, July 2010.