| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C267 WG11 Number: m18308 |
| --- | --- |

| Title: | Test Results of Transform Skip Flag under Phase 2 VLC Integration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Peisong Chen, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the test results for Transform Skip flag (HHI_RQT_ROOT) under Phase 2 VLC integration.

- Introduction

In the 2nd JCT-VC meeting, Tool Experiment 12 (TE12) was set up to explore the performance of the individual tools in the Test Model under Consideration (TMuC). This contribution presents supplemental testing results of the transform skip flag tool under phase two variable length coding integration.

The software version used for this test was TMuC 0.8. The reference was generated by using the default setting [2] for low complexity configuration with two modifications.  First modification is that Phase 2 VLC is enabled (LCEC_PHASE2=1). Second modification is that RQT is disabled (RQT=0). The reference was compared against the two alternative settings with residual qualtree enabled (RQT=1) and both residual qualtree and transform skip flag  enabled (RQT=1, HHI_RQT_ROOT =1). The executables for encoding and decoding were compiled on 64-bit Linux and the simulations were performed on the Linux cluster.

# Simulation result

| Low complexity low delay | RQT=1 | RQT=1, RQT_ROOT=1 |
| --- | --- | --- |
| WQVGA30_RaceHorses | 2.16 | 0.21 |
| WQVGA50_BasketballPass | 2.78 | 1.00 |
| WQVGA50_BlowingBubbles | 1.24 | -0.85 |
| WQVGA60_BQSquare | 0.71 | -1.09 |
| WQVGA AVE | 1.72 | -0.18 |
| WVGA30_RaceHorses | 1.32 | 0.02 |
| WVGA50_BasketballDrill | 2.36 | -0.06 |
| WVGA50_PartyScene | 0.45 | -0.67 |
| WVGA60_BQMall | 3.81 | 0.05 |
| WVGA AVE | 1.98 | -0.16 |
| 720p60_Vidyo1 | 11.99 | 0.47 |
| 720p60_Vidyo3 | 10.81 | 0.00 |
| 720p60_Vidyo4 | 10.56 | 0.20 |
| 720p AVE | 11.12 | 0.22 |
| 1080p24_ParkScene | 3.59 | -0.53 |
| 1080p24_Kimono | 4.48 | 1.54 |
| 1080p50_BasketballDrive | 3.63 | 1.45 |
| 1080p50_Cactus | 3.86 | 0.01 |
| 1080p60_BQTerrace | 3.06 | -0.93 |
| 1080p AVE | 3.72 | 0.31 |
| Average | 4.18 | 0.05 |

| Low complexity random access | RQT=1 | RQT=1, RQT_ROOT=1 |
| --- | --- | --- |
| WQVGA30_RaceHorses | 3.96 | 0.47 |
| WQVGA50_BasketballPass | 3.97 | 0.43 |
| WQVGA50_BlowingBubbles | 3.20 | 0.05 |
| WQVGA60_BQSquare | 2.94 | -0.35 |
| WQVGA AVE | 3.52 | 0.15 |
| WVGA30_RaceHorses | 2.78 | 0.17 |
| WVGA50_BasketballDrill | 2.97 | -0.13 |
| WVGA50_PartyScene | 2.25 | -0.01 |
| WVGA60_BQMall | 4.86 | 0.33 |
| WVGA AVE | 3.22 | 0.09 |
| 1080p24_ParkScene | 4.62 | 0.31 |
| 1080p24_Kimono | 5.99 | 1.37 |
| 1080p50_BasketballDrive | 5.00 | 0.87 |
| 1080p50_Cactus | 5.57 | 0.69 |
| 1080p60_BQTerrace | 5.00 | -0.39 |
| 1080p AVE | 5.23 | 0.57 |
| PeopleOnStreet | 6.28 | 1.88 |
| Traffic | 4.84 | 0.42 |
| 4kx2kp AVE | 5.56 | 1.15 |
| Average | 4.01 | 0.38 |

# Conclusions

According to the simulation results, further investigations and testings should be conducted to indentify the interaction of the transform skip flag and the phase 2 VLC.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).